3GPP TSG-SA5 Meeting #161	S5-252636

Fukuoka, Japan, 19 - 23 May 2025

Source:	Ericsson France S.A.S

Title:	Rel-19 pCR TS 28.561 Clarify NDT definition and collaborative NDT use case

Document for:	Approval

Agenda item:	6.19.5

Spec:	TS 28.561

Version:	0.2.0

Work Item:	NDT

Comments

Improve the description of collaborative NDTs. The existing text about benefits does not describe any advantage of the collaboration.

Proposed Changes

First change

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Network Digital Twin (NDT): virtual replica of a mobile network or part of one, that captures its attributes, behaviour and interactions

NOTE 1:	Mobile network includes both RAN and Core.

NOTE 2: 	An NDT can collaborate with other NDTs during activities, and dynamically adapt its internal behaviour accordingly.

next change

### 5.5.2	Use Cases

#### 5.5.2.1	Collaboration between NDTs - NDTADV_01

A single NDT might not be able to fulfil a task by itself and may be dependent on or need to use the service or outputs of another NDT during the simulation/emulation activity. This means that NDTs should be able to interact with each other, exchange information and dynamically adapt their behavior according to the exchanged information. Thus, the MnS consumer may express its preferences regarding relationships between NDTs to jointly fulfil the scenario to be modelled. Based on the MnS consumer's input, the MnS producer may evaluate whether such collaboration is necessary and determines how it can be realized. .

The benefits of collaboration between NDTs are:

-	NDTs collaboration may lead to a broader system awareness and promotes flexibility.

-	Complex scenarios can be fulfilled jointly by collaborating NDT instances.

next change

##### 5.5.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- NDTAUT-GEN-01 | The 3GPP management system should support a capability enabling an authorized MnS consumer to provide its expectation on the relation between NDTs. | NDTADV_01 |

End of changes