3GPP TSG-SA5 Meeting #161	S5-252668

Fukuoka, Japan, 19 - 23 May 2025

Source:		Rakuten Mobile

Title:	pCR TR 28.869 Add evaluation and conclusion for management data streaming based on message bus

Document for:	Approval

Agenda item:	6.19.6

Spec:	3GPP TR 28.869

Version:	1.4.0

Work Item:	FS_Cloud_OAM

Comments

Proposed Changes

* * * First Change * * * *

#### 5.2.2.4	Evaluation

Solution #1 Data streaming based on message bus provides the capability of multipoint-to-multipoint steaming, which can improve scalability, efficiency and resiliency for data reporting compared against point-to-point streaming.

The proposed solution implies potential impacts on the 3GPP management system to support the streaming of management data based on message bus. This includes:

- Impact to existing streaming data reporting service API defined in TS 28.532: to support message bus-based data reporting from the MnS producer, a new streaming data reporting service needs to be introduced.

- Impact to other management aspects, e.g. for network slicing, analytic and AI/ML etc: the 3GPP management system may use the same streaming data reporting service defined in TS28.532.

- Impact to conventional NFs which are not cloud native who may use the same streaming data reporting service defined in TS28.532.

Solution #2 Data streaming based on existing WebSocket based streaming mechanism implies no change to the existing standard for NFs running in the cloud. There is minimum impact compared to Solution #1.

Both Solution #1 and Solution #2 can coexist. Solution #2 can be the default streaming method for backward compatibility while Solution #1 would be an additional streaming method optionally required only for the cloud-native deployment scenario where the benefits of multipoint-to-multipoint communication outweigh the impacts.

As per 3GPP SBMA principle, it is up to the MnF to implement the required MnS service producer roles. It is not mandated for any NF (conventional or cloud-native) or MnF to implement the new streaming data reporting service based on message bus in Solution #1. Therefore, the impacts 1 and 2 above can be avoided if the existing streaming data reporting service defined in TS 28.532 remains unchanged and a new service is introduced in TS 28.532 for Solution #1.

Solution #1 combined with Solution #2 together meet the requirements for most of the cloud-native deployment scenarios.

* * * Next Change * * * *

# 6 			Conclusions and recommendations

Editor's Note: This clause captures the conclusions and the recommendations of the study.

## 6.x	Data streaming

It is recommended to start further normative work to enable 3GPP management system to support the new message bus-based streaming data reporting service for cloud-native NF deployment. Implementation of the new streaming data reporting service is not mandated in any MnF as per 3GPP SBMA principle.

It is also recommended to further study the application of data streaming based on message bus in broader scope in Rel20 SBMA topic where the multipoint-to-multipoint communication pattern can potentially benefit.

* * * End of Changes * * * *