3GPP TSG-SA5 Meeting #161	S5-252951

Fukuoka, Japan, 19 - 23 May 2025

Source:	Nokia, Samsung

Title:	Pseudo-CR on Rel-19 pCR TS 28.579 Add usecase solution for authorization of the external MnS consumers at the CCF

Document for:	Approval

Agenda item:	6.19.21.1

Spec:	3GPP TS 28.579

Version:	0.2.0

Work Item:	MExpo

Comments

This pCR proposes to add a use case solution for the “Authorization of the external MnS consumer to access the service APIs” use case to TS 28.579.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: " Management and orchestration; Architecture framework".

[3]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[4]	3GPP TS 29.222[4]: "Common API Framework for 3GPP Northbound APIs; stage 3".

[5]	3GPP TS 28.622: “Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

[6]	TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[7]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[8]	IETF RFC 7231: “Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content”.

[X1]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[X2]	3GPP TS 28.319: “Management and orchestration; Access control for management services”

[X3]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

## 6.X	Authorization of the external MnS consumer to access service APIs

### 6.X.1	Introduction

The authorization of the external MnS consumer at the CCF to discover and later access service APIs at the AEF of MSED is a three-step process that involves the following:

- First, enrolment of the external MnS consumer at MSED to enable the onboarding of the external MnS consumer at the CCF (see clause 6.1 of TS 33.122[X1]).

- Next, the onboarding of the API invoker to the CCF to become a recognized user using the information from the enrolment phase.

- Lastly, the provision of the authorization information to the external MnS consumer by the CCF to enable the service API invocation(s) at the AEF of MSED (see clause 6.5.2.3 of TS 33.122[X1]).

Clause 6.X.2 describes how the management system can enable the CCF to successfully authorize external MnS consumers to access service APIs at the AEF of MSED.

* * * Next Change * * * *

### 6.X.2 Authorization of the external MnS consumer at the CCF to access service APIs

Upon enrolment of the external MnS consumer at MSED, the external MnS consumer can proceed to onboard itself at the CCF. Further, to consume service APIs at the AEF of MSED, the external MnS consumer can obtain authorization information from the CCF. To this end, the following steps occur as shown in Figure 6.X.2.2-1:

Figure 6.X.2.2-1: Authorization of external consumers at the CCF to consume management services

Precondition:

-   There has been an offline enrolment of the external MnS consumer at the management system (see clause 6.X.2.1) and they have the CAPIF enrolment information.

1.	To begin the onboarding procedure, the API invoker establishes a secure connection with the CCF based on TLS server-side authentication. The server certificate is CCF’s Root CA, which was provided to the API invoker after enrolment (see clause 6.1 of TS 33.122[X1]).

2.	The external MnS consumer sends an “onboard API invoker request” to the CCF over the CAPIF-1/CAPIF-1e interface (see clause 5.5.2.2 of TS 29.222[4]). This data type includes the onboarding credential, which was sent to the external MnS consumer after enrolment (see clause 6.X.2.1).

3.	To enable the onboarding of the external MnS consumer, the CCF interacts with the authentication and authorization MnS producer.

4.	If the onboarding is successful, the CCF generates the API invoker profile. This profile includes the “API invoker id”, which represents a unique identifier for the external MnS consumer within the CCF.

5.	The CCF sends the onboard API invoker response to the API invoker with the response body represented by the “APIInvokerEnrolmentDetails” data type. The response includes the assigned “API invoker Id”.

Once the external MnS consumer is successfully onboarded to the CCF; to invoke one or more service APIs at the AEF of MSED, the following steps occur:

6.	When the API invoker wants to invoke specific service APIs at the AEF of MSED, it establishes a TLS session with the CCF based on certificate-based mutual authentication.

7.	The API invoker sends an “access token request” to the CCF providing the API invoker id and optionally a list of service API(s) it wants to invoke. The request body carries the information described by the AccessTokenReq data type (clause 8.5.4.2.6 of TS 29.222[4]).

8.	The CCF interacts with the authentication and authorization MnS producer and generates the access token for the external MnS consumer. The Scope parameter of Token claims shall indicate the granular access authorization for the external MnS consumer.

9.	The CCF sends an access token response to the API invoker. The CCF sends a "service API authorization response", which carries the information described by the AccessTokenRsp data type (see clause 8.5.4.2.7 of TS 29.222[4]).

Subsequently, the API invoker can successfully invoke the service API at the AEF of MSED as described in clause 6.5.2.3 of TS 33.122[X1].

Editor's Note: The interactions between the CCF and the authentication and authorization MnS producer, enabling the authorization of the external MnS consumer (i.e., steps 3 and 8 in Figure 6.X.2.2-1), is FFS.

* * * End of Changes * * * *