3GPP TSG-SA5 Meeting #161 	S5-252971

Fukuoka City, Fukuoka, Japan, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | 0556 | rev | 1 | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.622 Corrections on MDT PLMN List |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on TS32.422, MDT PLMN List is required for Signalling based Logged/Immediate MDT. For instance, in subclause 4.1.2.17.4 of TS 32.422: “Handling of various scenarios for Signalling based Logged/Immediate MDT are addressed below: 1) Management System initiating MDT activation shall validate that PLMNs specified in the MDT PLMN List are supported by all the cells specified in the area scope If the gNB receives a request where none of the PLMNs in the MDT PLMN List match any PLMN in its list, it shall ignore the request. ” However, the PLMN List is missing in TraceJob for immediate MDT, which is not consistent with TS32.422 specfiications and many RAN3/RAN2 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding plmnList for both immediate MDT and logged MDT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification, which may lead to incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.58, 4.3.60 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS28.623 CR 0535 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

### 4.3.58	MdtConfig <<dataType>>

#### 4.3.58.1	Definition

This <<dataType>> defines the configuration parameters of IOC TraceJob which are specific for MDT or any combination of MDT.

The attribute anonymizationOfMdtData specifies the level of anonymization of MDT data.

The attribute areaScope defines the area scope of MDT, which is specified in clause 5.10.2 of TS 32.422 [30].

The optional attribute sensorInformation allows to specify the sensor information to include.

Based on the value configured for attribute jobType in IOC TraceJob, the attributes immediateMdtConfig or loggedMdtConfig are available: In case of IMMEDIATE_MDT_ONLY or any combination of Immediate MDT the attribute immediateMdtConfig is applicable. In case of LOGGED_MDT_ONLY or LOGGED_MBSFN_MDT the attribute loggedMdtConfig is applicable.

The optional attribute plmnList allows to specify the PLMNs where measurements are collected, status indication and log reporting is allowed.

#### 4.3.58.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| anonymizationOfMdtData | M | T | T | F | T |
| areaScope | M | T | T | F | T |
| sensorInformation | CO | T | T | F | T |
| immediateMdtConfig | CM | T | T | F | T |
| loggedMdtConfig | CM | T | T | F | T |
| mNOnly | CO | T | T | F | T |
| plmnList | CO | T | T | F | T |

#### 4.3.58.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sensorInformation | This attribute may be present only if NR is supported. |
| immediateMdtConfig | This attribute shall be present only if Immediate MDT is supported. |
| loggedMdtConfig | This attribute shall be present only if Logged MDT is supported. |
| mNOnly | This attribute may be present if signalling based MDT for NR is supported and MN only for MDT is supported. |
| plmnList | This attribute may be present only if multiple PLMNs are supported. |

#### 4.3.58.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions.

*** START OF NEXT CHANGE ***

### 4.3.60	LoggedMdtConfig <<dataType>>

#### 4.3.60.1	Definition

This <<dataType>> defines the configuration parameters of IOC TraceJob which are specific for Logged MDT or Logged MBSFN MDT.

Based on the value configured for attribute jobType in IOC TraceJob, different attributes are available. In case of LOGGED_MDT_ONLY, the attributes reportType, eventListForEventTriggeredMeasurement, eventThresholdL1, hysteresisL1, timeToTriggerL1, areaConfigurationForNeighCell and npnIdentityList are applicable. In case of LOGGED_MBSFN_MDT, the attribute mbsfnAreaList is applicable.The optional attribute areaConfigurationForNeighCell allows to specify the area for which UE is requested to perform measurements logging for neighbour cells which have list of frequencies

For logged MDT in UMTS and LTE, the reporting is periodical. Parameter loggingInterval determines the interval between the reports and parameter loggingDuration determines how long the configuration is valid meaning after this duration has passed no further reports are sent. In NR, the reporting can be periodical or event based, determined by parameter reportType. For periodical reporting the same parameters as in LTE and UMTS apply. For event based reporting, parameter eventListForEventTriggeredMeasurement configures the event type, namely ‘out of coverage’ or ‘L1 event’. In case ‘L1 event’ is selected as event type, the logging is performed according to parameter loggingInterval at regular intervals only when the conditions indicated by eventThresholdL1, hysteresisL1, timeToTriggerL1 (defining the thresholds, hysteresis and time to trigger) are met and if UE is ‘camped normally’ state (TS 38.331 [38], TS 38.304 [42]). In case ‘out of coverage’ is selected as event type, the logging is performed according to parameter loggingInterval at regular intervals only when the UE is in ‘any cell selection’ state. Furthermore, logging is performed immediately upon transition from the ‘any cell selection’ state to the ‘camped normally’  state ( TS 38.331 [38], TS 38.304 [42]).

#### 4.3.60.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| traceCollectionEntityId | M | T | T | F | T |
| loggingDuration | M | T | T | F | T |
| loggingInterval | M | T | T | F | T |
| reportType | CM | T | T | F | T |
| eventListForEventTriggeredMeasurement | CM | T | T | F | T |
| eventThresholdL1 | CM | T | T | F | T |
| hysteresisL1 | CM | T | T | F | T |
| timeToTriggerL1 | CM | T | T | F | T |
| areaConfigurationForNeighCell | CO | T | T | F | T |
| mbsfnAreaList | CM | T | T | F | T |
| nPNIdentityList | CM | T | T | F | T |

#### 4.3.60.3	Attribute constraints

| Name | Definition |
| --- | --- |
| reportType (support qualifier) | This attribute shall be present only if NR is supported. |
| eventListForEventTriggeredMeasurement (support qualifier) | This attribute shall be present only if NR is supported. |
| eventThresholdL1 (support qualifier) | This attribute shall be present only if NR is supported. |
| hysteresisL1 (support qualifier) | This attribute shall be present only if NR is supported. |
| timeToTriggerL1 (support qualifier) | This attribute shall be present only if NR is supported. |
| areaConfigurationForNeighCell (support qualifier) | This attribute may be present only if NR is supported. |
| mbsfnAreaList (support qualifier) | This attribute shall be present only if E-UTRAN is supported. |
| nPNIdentityList (support qualifier) | This attribute shall be present only when these conditions are met: NR is supported; several NPNs are supported in the RAN (see TS 38.331 [38]). |

#### 4.3.60.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions.

*** END OF CHANGE ***