3GPP TSG-SA5 Meeting #161	S5-252356

Fukuoka, Japan, 19 - 23 May 2025

Source:	Nokia

Title:	Pseudo-CR on Definitions of CCL conflicts

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.2.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add definitions of CCL conflicts and clarify their differences

Proposed Changes

* * * First Change * * * *

# 4	Concepts and overview

## 4.X	Closed Control Loops conflicts

Multiple CCLs could co-exist and concurrently act within the same environment. The CCLs can affect one another, in the worst cases leading to conflicts. The conflicts may occur among goals, scopes or actions of the CCLs. The possible conflict scenarios include:

-	CCL Goals and Goal-target conflicts: These are conflict among the targets within the goals of the individual CCLs sharing a given scope. An example scenario is where a target on a network configuration parameter is part of the goals of more than one CCL in a given scope.

-	CCL Scope conflicts: These are conflicts among the scopes of the CCLs, specifically the scenarios where a given scope is considered differently by distinct CCL instances. An example is where the measurement scope of one CCL is the control scope of another CCL. Where applicable, it is desirable that the scopes are allocated such that that one CCL instance does not read a scope that is concurrently being controlled or adjusted by another CCL.

-	CCL actions conflicts: These are conflicts among the actions of the CCLs, specifically the scenarios where two CCL instances attempt to differently control the same parameters of the same managed objects. Where applicable, it is desirable that the actions are decided and allowed such that that two CCL instances will not control or adjust the same set of parameters on the same set of managed objects.

There are 2 subtypes of CCL actions conflicts – concurrent and non-concurrent actions conflicts.

-	CCL concurrent actions conflicts: These are conflicts where the actions are executed within a time period less than the impact time of the action, i.e., the action of the second CCL instance is executed before the impact of the first CCL instance is registered. In the simplest scenario, the two CCL instances try to execute the contradictory actions at exactly the same time. Concurrent actions conflicts are also called “action-execution-time conflicts”

-	CCL non-concurrent actions conflicts: These are conflicts where the actions are executed within a time period longer than the impact time of the action, i.e., the action of the second CCL instance is executed after the impact of the first CCL instance is registered. The second CCL instance in effect tries to undo the impact of the CCL instance.

-	CCL metric-value conflicts: These are conflicts for the desired value of one or more performance metrics by two CCL instances that do not have goal or action conflicts. The two CCL instances which have different goals or targets within their goals and two distinct control and measurement scopes but the actions of one CCL instance have impact on the measurement scope of the other CCL instance, i.e. one CCL will affect the network performance metrics that the other CCL is responsible for. For example, a conflict could occur among the metrics if a CCL that optimizes energy consumption affects handover performance metrics which are supposed to be optimized by another CCL. Metric-value conflicts are also called “Indirect actions conflict conflicts”

There are 2 subtypes of CCL metric-value conflicts – concurrent and non-concurrent metric-value conflicts.

-	CCL concurrent metric-value conflicts: These are conflicts where the CCL instances are triggered to act concurrently or to execute actions within the same time. Concurrent metric-value conflicts are also called “Trigger-time conflicts”

-	CCL non-concurrent metric-value conflicts: These are conflicts where the CCL instances are triggered to act in different time periods, e.g. where one CCL instance is active while the other is only monitoring its measurement scope.

Examples characterizing the differences among the conflicts are summarized by Table 4.X-1.

Table 4.X-1: Types of potential conflicts among CCL instances for goals g1, g2 and g3

| Conflict Type | Description | CCL-A | CCL-B | Comments |
| --- | --- | --- | --- | --- |
| Target Conflict | For CCLs C1 and C2, when same at least 1 target of a goal is present in both CCL asking for different outcomes on that target on same controlled entity (ME1). | Control Scope: ME1 Goal targets: - Load > 90 % (to maximize resource utilization) - Latency < 10 ms | Control Scope: ME1 Goal target: - Load < 90 % (to avoid congestion) | Conflict among the targets within the goals - due to different required target outcomes |
| Scope conflict | For CCLs C1 and C2, C1 and C2 have different goals and actions but their scopes are overlapping - e.g. C1's control scope (i.e. the controlled entities in the network) is part of C2's measurement scope (i.e. the measured entities in the network). | Measurement scope: - cells g1 Control Scope: - g1 Goal targets: - EC/bit is < 1WA Actions: - Target Entity: gNB-g2 - Target Change: switch off g2 | Measurement scope: cells g1, g2, g3, g4 Control Scope: - g2 Goals: - Load < 80 % Actions: - Target Entity: gNB-g2 - Target Change: change CIO | By switching off g2, C1 affects the scope which C2 reads for its load distribution measurements |
| Action Conflict | Concurrent direct actions conflicts: For CCLs C1 and C2, when both C1 and C2 are trying to configure the same characteristics of same target entity (gNB-g1) in contradiction. | Goals: - Throughput > 10 Gbps Actions: - Target Entity: gNB-g1 - Target Change: scale-out - Target Time: 04:00 | Goals: - EC is < 10KVA Actions: - Target Entity: gNB-g1 - Target Change: scale-in - Target Time: 04:00 | Conflict due to the time of executing the configuration actions on the same scope at the execution step |
|  | Non-concurrent direct actions conflicts: For CCLs C1 and C2, when both C1 and C2 is trying to configure the same characteristics of same target entity (gNB-g1) in contradiction. | Example 1 |  | Conflict due to configuration actions at execution step because both CCL want contradicting values for a particular characteristic of gNB-g1. Effect: the value may ping-pong continuously. |
|  |  | Goals target: - Throughput > 10 Gbps Actions: - Target Entity: gNB-g1 - Target Change: scale-out virtual resource | Goals target: - EC is < 10 KVA Actions: - Target Entity: gNB-g1 - Target Change: scale-in virtual resource |  |
|  |  | Example 2 |  |  |
|  |  | Goal target: - HO failure is < 2 % Actions: - Target Entity: gNB-g1 - Target Change: set CIO to a small positive value{to guarantee HOs with low chances of HO failure} | Goal target: - Load < 80 % Actions: - Target Entity: gNB-g1 - Target Change: set CIO to a small negative value [to advance HOs and move load to other cells] |  |
| Metric-value conflict | For CCLs C1 and C2, when C1 [optimize handover] and C2 [minimize interference] have different goals but the actions of C1 affect the goals of C2. | Goal target: - HO failure is < 2 % Actions: - Target Entity: gNB-g1 - Target Change: reduce CIO {to reduce chances of HO failure} | Goal target: - SINR > 10 dB Actions: - Target Entity: gNB-g1 - Target Change: lower antenna tilt | By reducing antenna tilt to minimize interference C2 affect the HO goal target of C1 |

The CCL may detect or observe events that identify the possibility of any one of the above conflicts. The conflict can be avoided using some information or the policies (e.g. priority) provided by the consumer. If the conflict actually occurs, the CCL MnS producer should support services to inform MnS consumers the confirmed detected conflicts. This may also include informing MnS consumer about the potential conflict.

* * * Next Change * * * *

## 5.7	CCL Conflict management Capability - CONF

### 5.7.1	Description

A CCL may experience direct conflicts on its goals, targets, scopes, trigger time and execution time. The management system needs to support capabilities to avoid, detect and resolve the conflicts.

### 5.7.2	Use Cases

#### 5.7.2.1	CCL concurrent metric-value conflicts handling - CONF_01

Typically, a CCL whose start is triggered based on conditions, needs to be triggered to run at a specific time and terminate when certain conditions are met, to run when a certain performance threshold is crossed. If triggered independently, there may be conflicts among the CCLs. The triggers for different CCLs to be executed need to be coordinated to avoid conflicts among the CCLs.

The management system should support the capability for avoidance of concurrent metric-value conflicts, also called trigger-time conflicts. Since each CCL focuses on a smaller scope of the network problem space, several CCLs may need to be executed.  For actions in a given network scope, the CCLs can be explicitly scheduled by the management system. Where the scopes overlap, the CCLs need to align the action plans, for example, which action plan to execute and when. There is a need to assess each plan and choose the most appropriate combination of action plan(s) based on the selection policy and then notify the selected action plan(s) to the related CCLs. The MnS consumer may also be notified when it is safe to ignore the conflict. The MnS consumer may configure the criteria for evaluating the severity of conflicts.

#### 5.7.2.2	CCL non-concurrent actions conflicts handling –CONF_02

When two (or more) CCLs attempt to adjust the same network parameter but with different and contradicting values, the desired actions of the 2 CCL will be in conflict. For example, a CCL assuring throughput of a slice may be scaling-out the virtual resources of the slice. Whereas a CCL minimizing the energy consumption may be scaling-in the virtual resource of the same slice. It can be when the CCLs execute actions at the same time. However, it also happens when the CCLs execute at different times, and the scenario for actions to be separated in time is the more likely than actions occurring simultaneously. casein these conflict scenarios, the network parameter continuously ping-pongs between the two values. Such a conflict may be called an action conflict.

NOTE:	A potential conflict can for example be detected if a CCL observed that PMs on a certain object keep flipping between two values. The constant flipping can be an indication that 2 CCL instances are attempting to change the same scope.

The CCL may detect or observe events that identify the conflicts. The conflict can be avoided using some information or the policies (e.g. priority) provided by the consumer. If the conflict actually occurs, the CCL MnS producer should support services to inform MnS consumers the confirmed detected conflicts. It is needed to maximize the avoidance of conflict, including “requesting” information from MnS consumer and to inform MnS consumer about the potential conflict. CCL MnS Producer may also provide recommendations, for updating/deleting the conflicting CCLs, that would result in the resolution of detected conflict. The recommendation for update may include suggestions for modified targets.

Editor’s Note:	The exact information that can be exchanged is FFS

#### 5.7.2.3	CCL non-concurrent metric-value conflicts handling – CONF_03

Two (or more) CCLs configuring different control parameter may all influence the same metric. In other cases, the two CCLs influence two metrics Y1 and Y2 that are couple, i.e., which have a logical relationship between them. E.g. handover (HO) failure and SINR are coupled since a bad SINR can lead to more HO failures. If the two CL desire different values for the metric, or different values for two target metrics Y1 and Y2 but the targets are coupled, the CCLs are in conflict for the metric resulting into a target-metric-value conflict or indirect target conflicts.

Two target metrics Y1 and Y2 may be coupled such that actions to optimize any of them lead to correlated oscillations/degradations in Y1 or Y2, e.g. Y1 ensuring "HO failure is < 2 %" and Y2 wanting "SINR > 10dB". The correlated oscillations indicate a potential conflict, but the CCLs may not see the oscillations in the metric that is not of their interest. The management system should support the capability for detecting potential indirect targets conflicts. An MnS consumer may analyse the correlations to detect the potential conflict between CCL1 and CCL2. The MnS consumer should be able to inform CCL1 and CCL2 about the detected potential conflict represented by the correlated oscillations.

This severity of degradation in the performance metrics of the related CCLs could be the confirmation that a detected potential conflict is an actual harmful conflict. The management system should support the capability for detecting or confirming actual indirect targets conflicts. The threshold to determine the severity may be defined by the MnS consumer (e.g. the operator) so that if the degree of degradation is higher than the threshold then it is a confirmed conflict that requires resolution.

The management system should support the capability for avoiding potential non-concurrent metric-value conflicts, also called indirect target conflicts. CCLs need to avoid large and frequent changes to network parameters which may affect network stability since they increase the probability of occurrence of conflicts. CCLs should take small smooth changes in the cases where the impact is not so clear and only make the large changes when the CCL is sure that the impact is positive. It is desirable for the CCL to notify to the MnS consumer the planned change, its claimed/predicted performance improvement and reliability/confidence in that action/decision. The MnS consumer may evaluate the claimed performance improvement and reliability/confidence to determine if the action should be allowed or not. The MnS consumer should be enabled notify the decision and possibly the failed criteria to the CCL - to either be executed or to be used to compute better decisions. Based on the inputs, the CCL may update its decision-making and repeat the decision evaluation process. If the CCL has consistently made good large-action-decisions, the MnS consumer should be enabled to inform the CCL that the CCL has consistently made good decisions and achieved its ultimate trust and that no more coordination of its decisions is needed.

The management system should support the capability for resolving detected indirect targets conflict. The MnS consumer should be enabled to trigger one or more CCLs to respond to the detected potential conflict. And if the triggered CCLs is unable to resolve that conflict, the CCL should inform the MnS consumer about the failure to resolve the problem. The MnS consumer can set the thresholds for performance degradation that triggers conflict detection and resolution.

Editor’s Note 1: The criteria for accurately setting the thresholds for performance degradation is FFS.

* * * End of Changes * * * *