3GPP TSG-SA5 Meeting #161	S5-252398

Fukuoka, Japan, 19 - 23 May 2025

Source:	Samsung

Title:	UC and solution for consumer getting authorization to access MnS.docx

Document for:	Agreeent

Agenda item:	6.19.21.1

Spec:	3GPP TS 28.561

Version:	0.2.0

Work Item:	MExpo

Comments

This contribution proposes UC the solution for consumer getting authorization to access MnS.docx.

Proposed Changes

First Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: " Management and orchestration; Architecture framework".

[3]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[4]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; stage 3".

[5]	3GPP TS 28.622: “Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

[6]	TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[7]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[8]	IETF RFC 7231: “Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content”.

[x]	3GPP TS 32.122: "Security aspects of Common API Framework (CAPIF) for 3GPP".

[y]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs".

[z]	3GPP TS 28.532: " Generic management services".

Next Modified Section

## 5.2	Use cases

### 5.2.1	Registration of the MSED into the CCF

#### 5.2.1.1	Definition

For MSED to enable the exposure of the management services using CAPIF, MSED has to be registered as a recognized API provider domain at the CCF. The MSED registration procedure ensures that the constituent API provider domain functions (i.e., API Exposing Function (AEF), the API Publishing Function (APF) and the API Management Function (AMF) are recognized users of the CCF and can support the use cases and functionalities described in clause 5.2.

Traceability: REQ-MEXPO-REG-01, REQ-MEXPO-REG-02, REQ-MEXPO-REG-03, REQ-MEXPO-REG-04

### 5.2.2	Publishing of management services into the CCF

#### 5.2.2.1	Definition

This use case describes how to publish management services to the CAPIF core function (CCF) to be exposed to the external MnS consumers. Before publishing management services to the CCF, the operator shall determine, subject to the network operator policy, regulatory requirements or contractual obligations the following:

- The management services to be published

- The specific set of resources and corresponding operations of the management services to be published

- Whether the specific set of resources and corresponding operations of the management service will be published as one or more service APIs to the CCF.

- Optionally, the performance and fault data associated to the management service (referred to as MnS component C) to be published.

Traceability: REQ-MEXPO-PUB-01, REQ-MEXPO-PUB-02, REQ-MEXPO-PUB-03, REQ-MEXPO-PUB-04, REQ-MEXPO-PUB-05.

### 5.2.3	Logging the management service API invocations to the CCF

#### 5.2.3.1	Definition

When exposing service APIs to external MnS consumers, the operator can benefit from monitoring information related to the service API invocations. The monitoring information can include:

- The invoked service API details (e.g., the name, resource(s), and operations)

- The identity of the external MnS consumer who performed the service API invocation (i.e., the API invoker ID)

- The result of the service API invocation (e.g., success or failure)

- The time and duration of the service API invocation

Traceability: REQ-MEXPO-LOG-01, REQ-MEXPO-LOG-02.

### 5.2.3	MnS Consumer getting appropriate authorizations to access MnS.

#### 5.2.3.1	Definition

This use case describes how an external MnS Consumer gets authorized to acces management services. The external MnS consumer need to get the enrolment details from MSED. With those enrollement details it need to executes CAPIF API Invoker onboarding process to get recogonized by the CAPIF framework. Thereafter, MnS Consumer need  to request OAuth token describing the granular access authorizations assigned to the consumer. The granular access control for management seevices are based on MnS Component A, B and C [2].

Traceability: REQ-MEXPO-AUTH-01, REQ-MEXPO-AUTH-02.

## 5.3	Requirements

REQ-MEXPO-FUN-01: 3GPP management system should support the capability to expose management services to external MnS consumers using CAPIF.

REQ-MEXPO-REG-01: The exposure of management services using CAPIF shall have the capability to provide the required mappings between the management service-related information and the CAPIF-defined API provider enrolment related information to enable the registration of MSED to the CCF.

REQ-MEXPO-REG-02: The exposure of management services using CAPIF shall provide the capability to register MSED to the CCF.

REQ-MEXPO-REG-03: The exposure of management services using CAPIF shall provide the capability to deregister MSED from the CCF.

REQ-MEXPO-REG-04: The exposure of management services using CAPIF shall provide the capability to update the registration details of MSED at the CCF.

REQ-MEXPO-PUB-01: The exposure of management services using CAPIF shall have the capability to map the management service-related information into the service API related information to enable the publishing of management services to the CCF.

REQ-MEXPO-PUB-02: The exposure of management services using CAPIF shall have the capability to publish the service APIs to the CCF.

REQ-MEXPO-PUB-03: The exposure of management services using CAPIF shall have the capability to retrieve the published service APIs at the CCF.

REQ-MEXPO-PUB-04: The exposure of management services using CAPIF shall have the capability to update the published service APIs at the CCF.

REQ-MEXPO-PUB-05: The exposure of management services using CAPIF shall have the capability to unpublish the service APIs from the CCF.

REQ-MEXPO-LOG-01: The exposure of management services using CAPIF shall support the capability to create logs based on the service API invocations by the external MnS consumers.

REQ-MEXPO-LOG-02: The exposure of management services using CAPIF shall support the capability to log the service API invocations to the CCF.

REQ-MEXPO-AUTH-02: The exposure of management services using CAPIF shall support the capability to authorize external MnS consumer to access the management service(s).

REQ-MEXPO-AUTH-02: The exposure of management services using CAPIF shall support the capability for granular access control (based on MnS Component A, B and C [2]) for external MnS Consumer accessing the management service(s).

Next Modified Section

## 6.3	Consumer getting authorization to access MnS

### 6.3.1	Introduction

In order to access the required MnS, the external MnS Consumer have to request for appropriate authorizations. The mechanism described in clause 6.1 and 6.5.2.3 of [x] is utilized.

### 6.3.2	Procedure Flow

The folwling flow depicts the process of MnS Consumer getting the required authorization to access required MnS.

Figure 1: External consumer getting authorizing for access MnS

- The MnS Consumer obtains onboarding enrolment information. See clause 6.1 of [x].

Note: The MnS Consumer is considerd as API Invoker and API Provider Domain is considered as MSED.

Note: The A&A MnS Producer generates an Identiy MOI for the MnS Consumer and provides a related client_id as part of onboarding credential (OAuth 2.0 access token).

- The MnS Consumer and CCF establish a secure session based on TLS. See clause 6.1 of [x]

- The Consumer sends an onboard API invoker request to the CCF. See clause 6.1 of [x] and clause 5.5.2.2 of [y].

- CCF, behaving as Provisioning MnS Consumer, send getMOIAttributes request [z] to read the available Identity instances where the client_id matches with the client_id received as part of OAuth 2.0 access token.

- A&A MnS Producer sends the getMOIAttributes response [z].

- The CCF generates the "API invoker id" that represents a unique identifier for the API invoker in the CCF. The CCF associates the generated "API invoker id" with the queried Identity instance. See clause 6.1 of [x]

- The CCF sends an onboard API invoker response to the MnS Consumer. See clause 6.1 of [x] and clause 5.5.2.2 of [y].

- When the Consumer wants to invoke specific service APIs, it sends an Access Token Request message to the CAPIF core function. See clause 6.5.2.3 of [x].

- CCF, behaving as Provisioning MnS Consumer, send getMOIAttributes request [z] to read the Identity instance associated with the received "API invoker id".

- A&A MnS Producer sends the getMOIAttributes response [z].

- CCF, generate the OAuth token. See clause 6.5.2.3 of [x]. The Scope parameter of Token claims shall indicate the granular access authorization for the Consumer.

- CCF send the access token request containing the OAuth token generated. See clause 6.5.2.3 of [x]

- Consumer will access the required MnS using the received OAuth token.

End Of Modified Section