3GPP TSG-SA5 Meeting #161	S5-252411

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.277 | CR | 0060 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the editor note in clause 6.5.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Editor’s notes are not permitted in frozen specifications. The editor’s note in clause 6.5.2.1, regarding a potential revisit of an information element structure, is inconsistent with the frozen status and 3GPP editorial rules. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the editor’s note in clause 6.5.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An editor’s note in a frozen specification may lead to confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

6.5.2.1	Definition of ProSe Information

The fields of the ProSe specific charging information used for 5G ProSe is provided within the Service Specification Information.

The detailed structure of the ProSe Information can be found in table 6.5.2.1.1.

Table 6.5.2.1.1: Structure of ProSe information

| Information Element | Category | Description |
| --- | --- | --- |
| Announcing PLMN ID | OC | Described in table 6.3.1.2.1. |
| Announcing UE HPLMN Identifier | OM | Described in table 6.3.1.2.1. |
| Announcing UE VPLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Monitoring UE HPLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Monitoring UE VPLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Discoverer UE HPLMN Identifier | OM | Described in table 6.3.1.2.1. |
| Discoverer UE VPLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Discoveree UE HPLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Discoveree UE VPLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Monitored PLMN Identifier | OC | Described in table 6.3.1.2.1. |
| ProSe Application ID | Oc | Described in table 6.3.1.2.1. |
| Application ID | Oc | Described in table 6.3.1.2.1. |
| Application Specific Data | Oc | Described in table 6.3.1.2.1. |
| ProSe functionality | Oc | This IE holds the ProSe functionality UE is requesting, e.g. direct discovery (Announcing, Monitoring, or Match Report), direct communication. |
| ProSe Event Type | Oc | Described in table 6.3.1.2.1. |
| Direct Discovery Model | Oc | Described in table 6.3.1.2.1. |
| Validity Period | Oc | Described in table 6.3.1.2.1. |
| Role of UE | Oc | Described in table 6.3.1.2.1. |
| ProSe Request Timestamp | Oc | Described in table 6.3.1.2.1. |
| PC3 Protocol Cause | Oc | Described in table 6.3.1.2.1. |
| Monitoring UE Identifier | Oc | Described in table 6.3.1.2.1. |
| Requestor PLMN Identifier | OC | Described in table 6.3.1.2.1. |
| Requested Application Layer User ID | Oc | Described in table 6.3.1.2.1. |
| Requested PLMN Identifier | Oc | Described in table 6.3.1.2.1. |
| Time Window | Oc | Described in table 6.3.1.2.1. |
| Range Class | Oc | Described in table 6.3.1.2.1. |
| Proximity Alert Indication | Oc | Described in table 6.3.1.2.1. |
| Proximity Alert Timestamp | Oc | Described in table 6.3.1.2.1. |
| Proximity Cancellation Timestamp | Oc | Described in table 6.3.1.2.1. |
| Hop Count | Oc | This value reflects the number of Relays for the 5G ProSe Remote UE to reach the network in ProSe multi-hop UE-to-Network relay communication as defined in TS 23.304 [241] clause 5.8.3.1. |
| Relay IP address | Oc | Described in table 6.3.1.2.1. in case of layer-3 multi-hop UE-to-Network relay communication, it represents the IP address used by 5G ProSe UE-to-Network Root Relay as defined in TS 23.304 [241], clause 5.8.3.1 |
| ProSe UE-to-Network Relay UE ID | Oc | Described in table 6.3.1.2.1. |
| ProSe Destination Layer-2 ID | Oc | The identifier of a link-layer that identifies a device or a group of devices that are recipients of ProSe communication frames. |
| Intermediate Relay Information Container | Oc | This field holds a list of Intermediate Relay Information, (the Root Relay is not included), used for 5G ProSe Multi-hop UE-to-Network Relay Communication charging. |
| Intermediate Relay IP address | Oc | The IP address used by the 5G ProSe intermediate UE-to-Network Relay as defined in TS 23.304 [241] clause 6.4.3.8.2 and clause 6.4.3.9.2. |
| ProSe UE-to-Network Intermediate Relay UE ID | Oc | Link layer identifier of ProSe UE-to-Network intermediate Relay UE that is used for direct communication as defined in TS 23.304 [238] clause 4.6.4.3. |
| PFI Container information | OC | This field holds a list of PFI data container information defined in clause 6.5.2.3. |
| Transmission Data Container | OC | Described in table 6.3.1.2.1. |
| Local Sequence Number | Oc | Described in table 6.3.1.2.1. |
| Change Time | Oc | Described in table 6.3.1.2.1. |
| Coverage status | Oc | Whether UE is served by NG-RAN or not, i.e. in coverage, out of coverage. |
| User Location Information | Oc | The location of the UE, e.g. NCGI. |
| Data Volume Transmitted | Oc | Described in table 6.3.1.2.1. |
| Change Condition | Oc | Described in table 6.3.1.2.1. |
| VPLMN Identifier | Oc | Described in table 6.3.1.2.1. |
| Usage information report sequence number | Oc | Described in table 6.3.1.2.1. |
| Radio Resources indicator | Oc | Described in table 6.3.1.2.1. |
| Radio Frequency | Oc | Described in table 6.3.1.2.1. |
| PC5 Radio Technology | OM | Described in table 6.3.1.2.1. |
| Reception Data Container | Oc | Described in table 6.3.1.2.1. |
| Local Sequence Number | Oc | Described in table 6.3.1.2.1. |
| Change Time | Oc | Described in table 6.3.1.2.1. |
| Coverage status | Oc | Described in table 6.3.1.2.1. |
| User Location Information | Oc | The location of the UE, e.g. NCGI. |
| Data Volume Received | Oc | Described in table 6.3.1.2.1. |
| Change Condition | Oc | Described in table 6.3.1.2.1. |
| VPLMN Identifier | Oc | Described in table 6.3.1.2.1. |
| Usage information report sequence number | Oc | Described in table 6.3.1.2.1. |
| Radio Resources indicator | Oc | Described in table 6.3.1.2.1. |
| Radio Frequency | Oc | Described in table 6.3.1.2.1. |
| PC5 Radio Technology | OM | Described in table 6.3.1.2.1. |

End of Change