3GPP TSG-SA5 Meeting #161	S5-252429

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0220 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.554 Updates to NF energy consumption based on VNF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Rakuten Mobile, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 28.880 has studied on energy efficiency and energy savings aspects of 5G networks and services. As part of the use case #2 documented in clause 5.2 of TR 28.880, solutions for alternative options to obtain energy consumption of VNF/VNFC have been documented and recommended (see clause 6.2 of TR 28.880), whereby it is recommended to document relevant new estimated KPIs in the TS 28.554. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: update existing references to refer to new measurements available in ETSI NFV. Change #2: add options for the energy consumption of VNFC based on OS-containers and for directly collected energy consumptions of VNFC and resources from other systems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for energy consumption in other common forms of deployment of 3GPP NF like based on OS containers will not be possible, as well as less accurate forms of estimating the energy consumption would remain in 3GPP management system related specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.7.3.1.2, 6.7.3.1.3, 6.7.3.1.A (new), 6.7.3.1.B (new), 6.7.3.1.C (new), 6.7.3.1.D (new), 6.7.3.1.E (new), 6.7.3.1.F (new), 6.7.3.1.G (new), 6.7.3.1.H (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | Y |  | O&M Specifications |  |  |  | TS 28.310 CR 0080 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void.

[3]	ITU-T Recommendation E.800: "Definitions of terms related to quality of service".

[4]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[5]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[6]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[7]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[8]	ETSI ES 203 228 V1.2.1 (2017-04): "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[9]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[10]	ETSI 202 336-12 V1.2.1 (2019-02): "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[11]	ETSI GS NFV-IFA 027 V5.2.1 (2024-09): "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Performance Measurements Specification".

[12]	3GPP TS 38.314: "NR; layer 2 measurements".

[13]	3GPP TS 22.261: "Service requirements for the 5G system".

[14]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[15]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[16]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[17]	3GPP TS 28.318: "Management and Orchestration; Network and Service Operations for Energy Utilities (NSOEU)".

[18]	3GPP TS 28.313: "Management and orchestration; Self-Organizing Networks (SON) for 5G networks".

[19]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Overall Description; Stage 2"

Start of Second change

#### 6.7.3.1	NF Energy Consumption (EC)

##### 6.7.3.1.1	Definition

a) ECNF

b) This KPI describes the Energy Consumption (EC) of a 5G Network Function (NF). This KPI is obtained by summing up the energy consumption of PNF(s) and/or VNF(s) which compose the NF. The unit of this KPI is J.

c)

- How a 5GC NF is composed of VNFs and PNFs is implementation specific. In particular, whether a VNF instance (respectively PNF) is shared or not between more than one NF is implementation specific. Hence, the case where a VNF instance (resp. PNF) is shared between multiple NFs is out of scope of the present document;

- ECPNF represents the Energy Consumption (EC) of a PNF;

- ECVNF represents the Energy Consumption (EC) of a VNF. It is obtained by summing up the Energy Consumption (EC) of all its constituent VNFCs;

- In the present document:

# ECPNF is measured according to ETSI ES 202 336-12 [10],

# it is considered that ECVNF cannot be measured hence is estimated. Therefore the resulting ECNF KPI is defined as:

##### 6.7.3.1.2	Estimated Virtualized Network Function (VNF) energy consumption

a) ECVNF,estimated

b) A KPI that gives an estimation of the energy consumption of a VNF. This KPI is obtained by summing up the estimated energy consumption of its constituent Virtualized Network Function Components (VNFC). The unit of this KPI is J.

c)

d) ManagedFunction

e) In this version of the document, the energy consumption of the VNFC is estimated by different means, based on the form of deployment of the VNFC and the availability of energy consumption measurements provided by other management and orchestration systems.

When the VNFC is realized with a virtual machine, ECVNFC,estimated is obtained by the following three options:

-	estimated based on its resource usage metrics as per clause 6.7.3.1.3, or

-	by collecting the total energy consumption of the virtual compute resource on which the VNFC runs, as per clause 6.7.3.1.F, or

-	by collecting the total energy consumption of the VNFC, as per clause 6.7.3.1.H.

When the VNFC is realized with OS containers, ECVNFC,estimated is obtained by the following three options:

-	estimated based on its resource usage metrics as per clause 6.7.3.1.A, or

-	by collecting the total energy consumption of the OS containerized workload that realizes the VNFC, as per clause 6.7.3.1.G, or

-	by collecting the total energy consumption of the VNFC, as per clause 6.7.3.1.H.

##### 6.7.3.1.3	Estimated Virtualized Network Function Component (VNFC) energy consumption running on virtual compute resource based on resource usage metrics

a) ECVNFC,estimated

b) A KPI that gives an estimation of the energy consumption of a VNFC that runs on a virtual compute resource. In this version of the document, this KPI is obtained by taking the estimated energy consumption of the virtual compute resource instance on which the VNFC runs. The unit of this KPI is J.

c)

d) ManagedFunction

e) In this version of the document, the energy consumption of the virtual compute resource instance is estimated based on either:

- its mean vCPU usage, as per clause 6.7.3.1.4. The method for calculating ECVNFC,estimated is described in TS 28.310 [9] clause 6.3.2.2.1, or

- its mean vMemory usage, as per clause 6.7.3.1.5, or

- its mean vDisk usage, as per clause 6.7.3.1.6, or

- its I/O traffic volume, as per clause 6.7.3.1.7.

##### 6.7.3.1.4	Estimated virtual compute resource instance energy consumption based on mean vCPU usage

a) ECvirtualCompute,estimated,VCpuUsageMean

b) A KPI that gives an estimation of the energy consumption of a virtual compute resource instance. The energy consumption of a virtual compute resource instance X is estimated as a proportion of the energy consumption of the NFVI node on which the virtual compute resource instance X runs. This proportion is obtained by dividing the vCPU mean usage of the virtual compute resource instance X, by the sum of the vCPU mean usage of all virtual compute resource instances running on the same NFVI Node as X. The unit of this KPI is J.

c)

d) ManagedFunction

e)

- VCpuUsageMean is the mean vCPU usage of the virtual compute resource instance during the observation period, provided by ETSI NFV MANO (see clause 7.1.2 of ETSI GS NFV-IFA 027 [11]),

-    is sum of the vCPU mean usage of all virtual compute resource instances running on the same NFVI Node during the same observation period, all separately provided by NFV MANO (see clause 7.1.2 of ETSI GS NFV-IFA 027 [11]),

- ECNFVINode,measured is the energy consumption of the NFVI node on which the virtual compute resource runs, measured during the same observation period, as per ETSI ES 202 336-12 [10].

##### 6.7.3.1.5	Estimated virtual compute resource instance energy consumption based on mean vMemory usage

a) ECvirtualCompute,estimated,VMemoryUsageMean

b) A KPI that gives an estimation of the energy consumption of a virtual compute resource instance. The energy consumption of a virtual compute resource instance X is estimated as a proportion of the energy consumption of the NFVI node on which the virtual compute resource instance X runs. This proportion is obtained by dividing the vMemory mean usage of the virtual compute resource instance X, by the sum of the vMemory mean usage of all virtual compute resource instances running on the same NFVI Node as X. The unit of this KPI is J.

c)

d) ManagedFunction

e)

- VMemoryUsageMean is the mean memory usage of the virtual compute resource instance during the observation period, provided by NFV MANO,

-  is the sum of the mean memory usage of all virtual compute resource instances running on the same NFVI node during the same observation period, all separately provided by NFV MANO (see clause 7.1.4 of [11],

- ECNFVINode,measured is the energy consumption of the NFVI node on which the virtual compute resource runs, measured during the same observation period, as per ETSI ES 202 336-12 [10]. The measurement defined in TS 28.552 [6] clause 5.1.1.19.3 can be used to measure ECNFVINode,measured.

##### 6.7.3.1.6	Estimated virtual compute resource instance energy consumption based on mean vDisk usage

a) ECvirtualCompute,estimated,VDiskUsageMean

b) A KPI that gives an estimation of the energy consumption of a virtual compute resource instance. The energy consumption of a virtual compute resource instance X is estimated as a proportion of the energy consumption of the NFVI node on which the virtual compute resource instance X runs. This proportion is obtained by dividing the vDisk mean usage of the virtual compute resource instance X, by the sum of the vDisk mean usage of all virtual compute resource instances running on the same NFVI Node as X. The unit of this KPI is J.

c)

d) ManagedFunction

e)

- VDiskUsageMean is the mean disk usage of the virtual compute resource instance during the observation period, provided by NFV MANO,

-  is the sum of the mean disk usage of all virtual compute resource instances running on the same NFVI Node during the same observation period, all separately provided by NFV MANO (see clause 7.1.6 of [11],

- ECNFVINode,measured is the energy consumption of the NFVI node on which the virtual compute resource runs, measured during the same observation period, as per ETSI ES 202 336-12 [10]. The measurement defined in TS 28.552 [6] clause 5.1.1.19.3 can be used to measure ECNFVINode,measured.

##### 6.7.3.1.7	Estimated virtual compute resource instance energy consumption based on I/O traffic volume

a) ECvirtualCompute,estimated,VDiskUsageMean

b) A KPI that gives an estimation of the energy consumption of a virtual compute resource instance. The energy consumption of a virtual compute resource instance X is estimated as a proportion of the energy consumption of the NFVI node on which the virtual compute resource instance X runs. This proportion is obtained by dividing the I/O traffic volume of the virtual compute resource instance X, by the sum of the I/O traffic volume of all virtual compute resource instances running on the same NFVI Node as X. The unit of this KPI is J.

c)

d) ManagedFunction

e)

- IOTrafficVolume is the sum of the incoming and outgoing traffic volumes of the virtual compute resource instance during the observation period, provided by NFV MANO.

- Incoming traffic volume is obtained by measuring the number of incoming bytes on virtual compute (VNetByteIncoming - cf. clause 7.1.8 of [11]) during the observation period.

- Outgoing traffic volume is obtained by measuring the number of outgoing bytes on virtual compute (VNetByteOutgoing - cf. clause 7.1.9 of [11]) during the observation period,

-  is the sum of the incoming and outgoing traffic volumes of all virtual compute resource instances running on the same NFVI node during the same observation period, all separately provided by NFV MANO (see clause 7.1.8 and 7.1.9 of [11]),

- ECNFVINode,measured is the energy consumption of the NFVI node on which the virtual compute resource runs, measured during the same observation period, as per ETSI ES 202 336-12 [10]. The measurement defined in TS 28.552 [6] clause 5.1.1.19.3 can be used to measure ECNFVINode,measured.

Start of Third change

##### 6.7.3.1.A	Estimated Virtualized Network Function Component (VNFC) energy consumption running on OS containers based on resource usage metrics

a) ECVNFC,estimated

b) A KPI that gives an estimation of the energy consumption of a VNFC that runs as an OS containerized workload. In this version of the present document, this KPI is obtained by taking the estimated energy consumption of the OS container workload instance on which the VNFC runs. The unit of this KPI is J.

c) ECVNFC, estimated=ECOS container workload, estimated

d) ManagedFunction

e) In this version of the present document, the energy consumption of the OS container workload instance is estimated is estimated as a proportion of the energy consumption of the compute node (i.e., a container infrastructure services (CIS) instance) on which the OS container workload instance runs, based on resource usage metrics either:

- its mean CPU usage, as per clause 6.7.3.1.B. The method for calculating ECVNFC,estimated is described in TS 28.310 [9] clause 6.3.2.3.1, or

- its mean memory usage, as per clause 6.7.3.1.C, or

- its mean storage resource usage, as per clause 6.7.3.1.D, or

- its I/O traffic volume, as per clause 6.7.3.1.E.

NOTE:	The following formulation assumes the case that OS container workloads are deployed on CIS instances that are bare-metal compute nodes, which can be mapped 1:1 to an NFVI Node.

##### 6.7.3.1.B	Estimated OS container workload instance energy consumption based on mean CPU usage

a) ECOSContainerWorkload,estimated,CpuUsageMean

b) A KPI that gives an estimation of the energy consumption of an OS container workload instance. The energy consumption of an OS container workload instance X is estimated as a proportion of the energy consumption of the NFVI node on which the OS container workload instance X runs. This proportion is obtained by dividing the CPU mean usage of the OS container workload instance X, by the sum of the CPU mean usage of all OS container workload instances running on the same NFVI Node as X. The unit of this KPI is J.

c) ECOSContainerWorkload, estimated, CpuUsageMean=CpuUsageMeanMcioOSContainerWorkloadCpuUsageMeanMcio*ECNFVINode, measured

d) ManagedFunction

e)

- CpuUsageMeanMcio is the mean CPU usage of the OS container workload instance during the observation period, provided by NFV-MANO (see clause 7.4.2 of ETSI GS NFV-IFA 027 [11]),

- OSContainerWorkloadCpuUsageMeanMcio is sum of the CPU mean usage of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV-MANO (see clause 7.4.2 of ETSI GS NFV-IFA 027 [11]),

- ECNFVINode, measured is the energy consumption of the NFVI node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [10].

##### 6.7.3.1.C	Estimated OS container workload instance energy consumption based on mean memory usage

a) ECOSContainerWorkload,estimated,MemoryUsageMean

b) A KPI that gives an estimation of the energy consumption of an OS container workload instance. The energy consumption of an OS container workload instance X is estimated as a proportion of the energy consumption of the NFVI node on which the OS container workload instance X runs. This proportion is obtained by dividing the memory mean usage of the OS container workload instance X, by the sum of the memory mean usage of all OS container workload instances running on the same NFVI Node as X. The unit of this KPI is J.

c) ECOSContainerWorkload, estimated, MemoryUsageMean=MemoryUsageMeanMcioOSContainerWorkloadMemoryUsageMeanMcio*ECNFVINode, measured

d) ManagedFunction

e)

- MemoryUsageMeanMcio is the mean memory usage of the OS container workload instance during the observation period, provided by NFV-MANO (see clause 7.4.4 of ETSI GS NFV-IFA 027 [11]),

-  OSContainerWorkloadMemoryUsageMeanMcio is the sum of the mean memory usage of all OS container workload instances running on the same NFVI node during the same observation period, all separately provided by NFV-MANO (see clause 7.4.4 of ETSI GS NFV-IFA 027 [11]),

- ECNFVINode,measured is the energy consumption of the NFVI node on which the OS container workload instance runs, measured during the same observation period, as per ETSI ES 202 336-12 [10]. The measurement defined in TS 28.552 [6] clause 5.1.1.19.3 can be used to measure ECNFVINode,measured.

##### 6.7.3.1.D	Estimated OS container workload instance energy consumption based on mean storage resource usage

a) ECOSContainerWorkload,estimated,StorageUsageMean

b) A KPI that gives an estimation of the energy consumption of an OS container workload instance. The energy consumption of an OS container workload instance X is estimated as a proportion of the energy consumption of the NFVI node on which the OS container workload instance X runs. This proportion is obtained by dividing the storage resource mean usage of the OS container workload instance X, by the sum of the storage resource mean usage of all OS container workloads instances running on the same NFVI Node as X. The unit of this KPI is J.

c) ECOSContainerWorkload, estimated, StorageUsageMean =StorageUsageMeanMcioOSContainerWorkloadStorageUsageMeanMcio*ECNFVINode, measured

d) ManagedFunction

e)

- StorageUsageMeanMcio is the mean storage resource usage of the OS container workload instance during the observation period, provided by NFV-MANO (see clause 7.4.10 of ETSI GS NFV-IFA 027 [11]),

- OSContainerWorkloadStorageUsageMeanMcio is the sum of the mean storage resource usage of all OS container workload instances running on the same NFVI Node during the same observation period, all separately provided by NFV MANO (see clause 7.4.10 of ETSI GS NFV-IFA 027 [11]),

- ECNFVINode,measured is the energy consumption of the NFVI node on which the virtual compute resource runs, measured during the same observation period, as per ETSI ES 202 336-12 [10]. The measurement defined in TS 28.552 [6] clause 5.1.1.19.3 can be used to measure ECNFVINode,measured.

##### 6.7.3.1.E	Estimated OS container workload instance energy consumption based on I/O traffic volume

a) ECOSContainerWorkload,estimated,IOTrafficVolume

b) A KPI that gives an estimation of the energy consumption of an OS container workload instance. The energy consumption of an OS container workload instance X is estimated as a proportion of the energy consumption of the NFVI node on which the OS container workload instance X runs. This proportion is obtained by dividing the I/O traffic volume of the OS container workload instance X, by the sum of the I/O traffic volume of all OS container workload instances running on the same NFVI Node as X. The unit of this KPI is J.

c) ECOSContainerWorkload, estimated, IOTrafficVolume=IOTrafficVolumeOSContainerWorkloadIOTrafficVolume*ECNFVINode, measured

d) ManagedFunction

e)

- IOTrafficVolume is the sum of the incoming and outgoing traffic volumes of the OS container workload instance during the observation period, provided by NFV-MANO.

- Incoming traffic volume is obtained by measuring the number of incoming bytes on the OS container workload instance (NetByteIncomingMcio - cf. clause 7.4.6 of ETSI GS NFV-IFA 027 [11]) during the observation period.

- Outgoing traffic volume is obtained by measuring the number of outgoing bytes on virtual compute (NetByteOutgoingMcio - cf. clause 7.4.7 of ETSI GS NFV-IFA 027 [11]) during the observation period,

IOTrafficVolume=NetByteIncomingMcio+NetByteOutgoingMcio

- OSContainerWorkloadIOTrafficVolume is the sum of the incoming and outgoing traffic volumes of all OS container workload instances running on the same NFVI node during the same observation period, all separately provided by NFV-MANO (see clauses 7.1.8 and 7.1.9 of ETSI GS NFV-IFA 027 [11]),

- ECNFVINode,measured is the energy consumption of the NFVI node on which the virtual compute resource runs, measured during the same observation period, as per ETSI ES 202 336-12 [10]. The measurement defined in TS 28.552 [6] clause 5.1.1.19.3 can be used to measure ECNFVINode,measured.

##### 6.7.3.1.F	Estimated Virtualized Network Function Component (VNFC) energy consumption running on virtual compute resource based on collected energy consumption

a) ECVNFC,estimated

b) A KPI that gives an estimation of the energy consumption of a VNFC that runs on a virtual compute resource. This KPI is obtained by collecting the total energy consumption of the virtual compute resource on which the VNFC runs over the collection period provided by NFV-MANO (see clause 7.1.18 of ETSI GS NFV-IFA 027 [11]), which equals to the energy consumption of the VNFC. The unit of this KPI is J.

c) ECVNFC,estimated=ECVirtualCompute,collected

d) ManagedFunction

e) None.

##### 6.7.3.1.G	Estimated Virtualized Network Function Component (VNFC) energy consumption running on OS containers based on collected energy consumption

a) ECVNFC,estimated

b) A KPI that gives an estimation of the energy consumption of a VNFC that runs on an OS container workload. This KPI is obtained by collecting the total energy consumption of the OS container workload instance on which the VNFC runs over the collection period provided by NFV-MANO (see clause 7.4.12 of ETSI GS NFV-IFA 027 [11]), which equals to the energy consumption of the VNFC. The unit of this KPI is J.

c) ECVNFC,estimated=ECOSContainerWorkload,collected

d) ManagedFunction

e) None.

##### 6.7.3.1.H	Estimated Virtualized Network Function Component (VNFC) energy consumption based on collected energy consumption

a) ECVNFC,estimated

b) A KPI that gives an estimation of the energy consumption of a VNFC that runs either on a virtual compute resource or an OS container workload. This KPI is obtained by collecting the total energy consumption of the VNFC instance over the collection period provided by NFV-MANO (see clause 7.2.16 of ETSI GS NFV-IFA 027 [11]), which equals to the energy consumption of the VNFC. The unit of this KPI is J.

c) ECVNFC,estimated=ECVNFC,collected

d) ManagedFunction

e) The method for collecting ECVNFC,collected is described in clause 6.3.3 of TS 28.310 [9].

End of changes