3GPP TSG-SA5 Meeting #161	S5-252998

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to DraftCR Rel-19 TS 28.105 Update on the use case of managing ML models in use in a live network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The solution for impact management in S5-252108 targets to satisfy the reqirements of managing ML models in use in a live network. However, the proposed solution is to enhance the AIMLInferenceReport by adding the potential impcat information generated by the ML Model. But the requirements propose to identify ML Model(s) that caused performance degradation and recommend remedial actions to address performance measurement and/or KPI degradation caused by ML model(s). In other words, REQ-AI/ML_INF-LIVES-01 and REQ-AI/ML_INF-LIVES-03 is not satisfied and REQ-AI/ML_INF-LIVES-02 is partially satisfied. This contribution proposes to update the use case and reqirement to align with the proposed solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update on the use case of managing ML models in use in a live network |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The req is not aligned with the solution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No impact on Stage 3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the First Change

### 7.3a.4	Information model definitions for ML inference

##### 7.3a.4.2.6	AIMLInferenceReport

###### 7.3a.4.2.6.1	Definition

This IOC represents a report from a AI/ML Inference.

An AIMLInferenceFunction may generate one or more AIMLInferenceReport(s).

Each AIMLInferenceReport provides information about inference outputs from one or more MLModel.

The AIMLInferenceReport also provides historical inference outputs for a series of time stamps.

The AIMLInferenceReport instance can be created by the MnS producer when creating an AIMLInferenceFunction instance.

The potentialImpactInfo includes impacted managed objects and network performance metrics, which can be utilized to manage the network performance, i.e. give up performing the recommended action in the inference output to prevent network performance degradation or take remedial actions when network performance degradation appears

###### 7.3a.4.2.6.2	Attributes

The AIMLInferenceReport includes inherited attributes from Top IOC (defined in TS 28.622 [12]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceOutputs | M | T | F | F | T |
| potentialImpactInfo | O | T | F | F | T |
| Attributes related to role |  |  |  |  |  |
| mLModelRef | M | T | F | F | T |

###### 7.3a.4.2.6.3	Attribute constraints

None.

###### 7.3a.4.2.6.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

End of the First Change

### 7.4.X3	PotentialImpactInfo <<dataType>>

#### 7.4.X3.1	Definition

This datatype define the potential network impacts due to the inference output results.

#### 7.4.X3.2	Attributes

The PotentialImpactInfo includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| impactedScope | M | T | F | F | T |
| impactedPM | M | T | F | F | T |

#### 7.4.X3.3	Attribute constraints

None.

#### 7.4.X3.4	Notifications

The notifications specified for the IOC using this <<datatype>> for its attribute(s), shall be applicable.

End of the First Change