3GPP TSG-SA5 Meeting #161 	S5-252261

Fukuoka City, Fukuoka, Japan, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0522 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.422 Trace Failure Notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Data_SREP |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed and concluded in TR28.873, the Trace Failure Notification is specified in TS32.422 for trace only and it is limited to UMTS and EPS. It shall be generic for trace, MDT, and QoE as well |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the Trace Failure Notification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Trace Failure Notification for MDT and QoE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

## 4.x	Trace failure notification

In case of the failure at Trace Recording Session activation, Trace reporting, MDT reporting, RCEF reporting, RLF reporting, RRC reporting, or 5GC UE level measurements, a producer shall report this failure to a consumer.

In case of file-based reporting, a file preparation failure notification shall be sent to the TCE.

In case of a streaming-based reporting, a Trace administrative messages shall be sent to the Trace Reporting MnS consumer. Trace administrative messages are sent as Common Trace Payload of Streaming Trace Record as specified in subclause 5.2.1 of TS 32.423[3]. Trace administrative messages are defined in subclause 5.2.4 of 3GPP TS 32.423[3].

*** END OF CHANGE ***