3GPP TSG-SA5 Meeting #161	S5-252491

Fukuoka, Japan, 19 - 23 May 2025

Source:	Huawei

Title:	pCR TS 28.567 Correct information models for CCL management in clause 6.2.1

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS 28.567

Version:	V0.2.0

Work Item:	CCLM

Comments

The figure name in clause 6.2.1 is “Relations for common information models for AI/ML management”. It should be corrected to “Relations for common information models for CCL management”. Besides, the relation between ClosedControlLoop and CCLPurpose is wrong, it should be bidirectional association relationship rather than name-containment based on the corresponding UML diagram discussed in the endorsed DP S5-252029. Also the attribute “cCLPurposeRefList” is included in the CCLControlLoop IOC. See below：

#### 6.3.1.2	Attributes

The CCLControlLoop IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.1.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |
| cCLPurposeRefList | M | T | T | T | T |

Proposed Changes

* * * First Change * * * *

### 6.2.1	Relationships

Figure 6.2.1-1: Relations for common information models for CCL management

Editor’s Note: The handling of Goal, targets or objectives for the general closed control loops is FFS

* * * Next Change * * * *

## A.1.1	CCL NRM fragment (Figure 6.2.1-1)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class ClosedControlLoop <<InformationObjectClass>>

class CCLPurpose << ProxyClass >>

class CCLScope << InformationObjectClass >>

class CCLReport <<InformationObjectClass>>

ManagedEntity "1" *-- "*" ClosedControlLoop: <<names>>

ClosedControlLoop "1" <--> "*" CCLPurpose

ClosedControlLoop "1" *-- "*" CCLScope: <<names>>

ClosedControlLoop "1" *-- "*" CCLReport: <<names>>

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

note top of CCLPurpose

Can be any of these CCL purposes:

NetworkProblemRecovery

FaultManagement

...

end note

@enduml

Source code for Figure 6.2.1-1 CCL NRM fragment

* * * End of Changes * * * *