3GPP TSG-SA5 Meeting #161	S5-252518

Fukuoka, Japan, 19 - 23 May 2025

Source:	ZTE Corporation

Title:	Pseudo-CR on TS 28.561 NDT LCM Overview

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.2.0

Work Item:	NDT

Comments

This pCR is to Add a description of NDT LCM Overview.

Proposed Changes

Start of First Changes

4.1.x	NDT Life-cycle management

An NDTFunction may have multiple simulation/emulation NDTJob instances each considering a different network scenario and use case. The NDTJob instance is an object that can be managed by the 3GPP management system. The NDT Life-cycle management includes LCM of the NDTFunction instance and the NDTJob instances. Accordingly, the following capabilities are included:

-	NDT Creation: The MnS producer who provides the NDT simulation/emulation capabilities creates an NDTFunction instance that is capable to model a specific scenario.

-	NDTJob Creation: NDT MnS producer receives the request to create an NDTJob instance. The NDT MnS producer instantiates and configure it to execute the simulation/emulation task. The NDTJob instance can be further configured by the NDT MnS consumer at any time.

-	NDTJob Suspension: NDT MnS producer receives the request to pause or suspend the simulation/emulation of an NDTJob instance.

- 	NDTJob Deletion: NDT MnS producer receives the request to stop the execution of the simulation/emulation of an NDTJob instance, i.e.,  delete the NDTJob instance.

-	NDT Deletion: The MnS producer may delete an NDTFunction instance that is not active.

NOTE: All of the life cycle operations may not be valid for all NDTJob instances. For example, a particular NDTJob instance may not support suspending the execution.