3GPP TSG-SA5 Meeting #161	S5-252543

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.201 | CR | 0019 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.201 Correction of NSPA Container Information reference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The format of the table reference is wrong, and the names of the tables are mixed or incorrect, i.e. the NSPA Container Information refers to the NSPA Charging Information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting naming and references for the tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect table references can lead to mis understanding and interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.2 and 6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.2.1.2	Definition of Performance and Analytics Charging Information

Specific charging information used for network slice performance and analytics charging is provided within the NSPA (Network Slice Performance and Analytics) Charging Information.

The detailed structure of the NSPA Charging information can be found in table 6.2.1.2-1.

Table 6.2.1.2-1: Structure of NSPA Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Single NSSAI | M | This field holds network slice information the performance and analytics information belongs to. |

* * * Next Change * * * *

#### 6.2.1.3	Definition of NSPA Container Information

Specific charging information used for network slice performance and analytics charging is provided within the NSPA Container Information.

The detailed structure of the NSPA Container Information can be found in table 6.2.1.3-1.

Table 6.2.1.3-1: Structure of NSPA Container Information

| Information Element | Category | Description |
| --- | --- | --- |
| Uplink Latency | OC | This field holds uplink latency as described in TS 28.541 [252] clause 6.4 uLLatency attribute (see NOTE 1). |
| Downlink Latency | OC | This field holds downlink latency as described in TS 28.541 [252] clause 6.4 dLLatency attribute. |
| Uplink Throughput | OC | This field holds uplink throughput of one single network slice as described in TS 28.541 [252] clause 6.4 uLThptPerSlice attribute (see NOTE 2). |
| Downlink Throughput | OC | This field holds downlink throughput of one single network slice as described in TS 28.541 [252] clause 6.4 dLThptPerSlice attribute |
| Maximum packet loss rate UL | OC | This field holds maximum packet loss rate uplink as described in TS 28.541 [252] clause 5.4 maxPacketLossRateUl attribute (see NOTE 3). |
| Maximum packet loss rate DL | OC | This field holds maximum packet loss rate downlink as described in TS 28.541 [252] clause 5.4 maxPacketLossRateDl attribute. |
| Service Experience statistics data | OC | This field holds service experience statistics data as described in TS 23.288 [150] |
| Number of PDU sessions | OC | This field holds the number of PDU sessions as described in TS 28.554 [271]. |
| Number of registered Subscribers | OC | This field holds the number of registered subscribers as described in TS 28.554 [271]. |
| Load level | OC | This field holds the load level as described in TS 23.288 [150]. |
| NOTE 1: For the backwards compatibility the Uplink Latency may be bound to Latency. NOTE 2: For the backwards compatibility the Uplink Throughput may be bound to Throughput. NOTE 3: For the backwards compatibility the Maximum packet loss rate UL may be bound to Maximum packet loss rate. |  |  |

* * * End of Changes * * * *