3GPP TSG-SA5 Meeting #161	S5-252611

Fukuoka, Japan, 19 - 23 May 2025

# Source:	CSCN

# Title:	IMS Call charging capability optimization

# Document for:	Discussion

Agenda Item:

1	Decision/action requested

CR to optimize the IMS call charging capability

2	References

[1] 3GPP TS 32.260 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects;

Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging

[2] 3GPP TS 32.298 3rd Generation Partnership Project;Technical Specification Group Services and System Aspects;

Telecommunication management; Charging management; Charging Data Record (CDR) parameter description

3	Rationale

Per TS 32.260 and TS 32.298, for normal session termination cases, when a session is terminated, a SIP BYE or a SIP 200 OK acknowledging the SIP BYE is sent by UE to CTF and the CTF shall trigger Charging Data Request[Stop]to CDF, with SIP-Request-Timestamp AVP set to the timestamp when it receives the SIP request. Then CDF shall generate a successful CDR with serviceDeliveryEndTimeStamp field retrieved from SIP-Request-TimeStamp AVP of Charging Data Request[Stop]. So serviceDeliveryEndTimeStamp field in CDR shall reflect the time stamp the UE terminates the call as described in Figure 1, which is a valid call duration.

Figure 1 Normal Session Termination

But in some rainy scenarios, such as IMS nodes internal error, CDF switchover, or network connection failure etc, CDF can not receive Charging Data Request[Stop]. In this case, CDF shall generate an incomplete CDR with aCRStopLost field set to TRUE and without serviceDeliveryEndTimeStamp field after a configured interval in CDF, which is described in Figure 2.

Figure 2 Charging Data Request[Stop] lost scenario

So for the Charging Data Request[Stop] lost scenario, as the incomplete CDR doesn't include serviceDeliveryEndTimeStamp field, the billing system can not get a valid call duration from it and subsequently causes revenue loss.

4	Detailed Proposal

Per TS 32.260, Charging Data Request [Interim] shall be sent by the network entity towards the CDF due to a session modification procedure (i.e. change in media) or expiration of the Charging Data Interim Interval, and CDF will use this information to generate partial CDRs or update session CDRs.

So CDF can record the timestamp of every Charging Data Request[Interim] it receives. For Charging Data Request[ Stop] lost scenario, CDF can use the timestamp carried in the latest Charging Data Request[Interim] to reflect the longest session living time of UE that CDF can get.

A new field can be added in CDR named as “lastACRInterimTimeStamp”, which corresponds to the timestamp carried in the latest Charging Data Request[Interim] and can be used to get a valid call duration as described in Figure 3. This field is only present when Charging Data Request[Stop] is lost in the session.

Figure 3 Charging Data Request[Stop] lost scenario after solution

If SIP-Response-Timestamp AVP is presen in the latest Charging Data Request [Interim], "lastACRInterimTimeStamp" field can be retrieved from the SIP-Response-Timestamp AVP of the latest Charging Data Request [Interim]. If SIP-Response-Timestamp AVP is not present, "lastACRInterimTimeStamp" filed can be retrieved from the Event-Timestamp AVP of the latest Charging Data Request [Interim].

Benefits:

1. For short call, there is no time limit partial CDR, so before this solution, billing system can not get a valid call duration for the whole call. After this solution, billing system can get a valid call duration as long as CDF can get.

2. For long duration call, CDF shall generate time limit partial CDR at a configured interval to avoid losing accounting information in abnormal termination cases.

For example, as described in Figure 4, if the time limit CDR generation interval in CDF is set to N*[Accounting Interim Interval], before this solution, in Charging Data Request[Stop] lost scenario, the possible call duration loss may be (N-1)*[Accounting Interim Interval]. The larger N is, the larger the call duration loss is. So in order to get less call duration loss, CDF is required to set a smaller value of CDR generation interval, which cases more time limit partial CDR to be generated. After this solution, the (N-1)*[Accounting Interim Interval] can be valid call duration as CDF record the time stamp of the last ACR, so the CDR generation interval in CDF can be set to a larger value, and CDF will generate less amount of time limit partial CDR.

Figure 4 Charging Data Request [Stop] lost scenario for long duration call