3GPP TSG-SA5 Meeting #161	S5-252691

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.663 | CR | 0028 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.663 Update sector equipment and antenna function definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Canada Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SectorEquipmentFunction and AntennaFunction definitions have been updated in 28.662 and scope has also been revised to include SBMA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the stage3 definitions for the updated IOCs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage3 would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2, A.3, B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Mirror of S5-252690. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# A.2	Mapping

## A.2.1	General mapping

See clause A.2.1 of [13].

## A.2.2	Information Object Class (IOC) mapping

### A.2.2.1	IOC SectorEquipmentFunction

Mapping from NRM IOC SectorEquipmentFunction attributes and associations to SS equivalent MOC SectorEquipmentFunction attributes

| IS Attribute | SS Attribute | SS Type |
| --- | --- | --- |
| id | id | string |
| fqBand | fqBand | short |
| eUTRANFqBands | eUTRANFqBands | GenericRanNRMAttributeTypes:: eUTRANFqBandsListType |
| nRFqBands | nRFqBands | GenericRanNRMAttributeTypes:: nRFqBandsListType |
| uTRANFDDFqBands | uTRANFDDFqBands | GenericRanNRMAttributeTypes:: uTRANFDDFqBandsListType |
| uTRANTDDFqBands | uTRANTDDFqBands | GenericRanNRMAttributeTypes:: uTRANTDDFqBandsListType |
| confOutputPower | confOutputPower | Short |
| theTmaList | theTmaList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| theAntennaList | theAntennaList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| theCellList | relatedCellList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| referencedBy | referencedBy | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |

### A.2.2.2	IOC AntennaFunction

Mapping from NRM IOC AntennaFunction attributes and associations to SS equivalent MOC AntennaFunction attributes

| IS Attribute | SS Attribute | SS Type |
| --- | --- | --- |
| id | id | string |
| beamTilt | beamTilt | short |
| retTiltValue | retTiltValue | short |
| bearing | bearing | short |
| retGroupName | retGroupName | string |
| elevation | elevation | short |
| height | height | short |
| maxAzimuthValue | maxAzimuthValue | short |
| minAzimuthValue | minAzimuthValue | short |
| horizBeamwidth | horizBeamwidth | short |
| vertBeamwidth | vertBeamwidth | short |
| latitude | latitude | short |
| longitude | longitude | short |
| theCellList | relatedCellList | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet |
| referencedBy | referencedBy | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet |

NOTE:	For all support qualifiers with the value "O", see attribute constraints in TS 28.622 [4].

2nd Change

# A.3	Solution Set definitions

## A.3.1	IDL definition structure

Clauses A.3.2 and A.3.3 define the MO classes for the Generic RAN NRM IRP.

## A.3.2	IDL specification "GenericRanNRMDefs.idl"

//File:GenericRanNRMDefs.idl

#ifndef _GENERICRANNRMDEFS_IDL_

#define _GENERICRANNRMDEFS_IDL_

#include "GenericNetworkResourcesNRMDefs.idl"

#pragma prefix "3gppsa5.org"

/**

* This module defines constants for each MO class name and

* the attribute names for each defined MO class.

*/

module GenericRanNRMDefs

{

/**

*  Definitions for MO class AntennaFunction

*/

interface AntennaFunction: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "AntennaFunction";

// Attribute Names

//

const string id = "id";

const string retTiltValue = "retTiltValue";

const string beamTilt = "beamTilt";

const string elevation = "elevation";

const string bearing = "bearing";

const string retGroupName = "retGroupName";

const string height = "height";

const string maxAzimuthValue = "maxAzimuthValue";

const string minAzimuthValue = "minAzimuthValue";

const string horizBeamwidth = "horizBeamwidth";

const string vertBeamwidth = "vertBeamwidth";

const string latitude = "latitude";

const string longitude = "longitude";

const string referencedBy = “referencedBy”;

};

/**

*  Definitions for MO class TmaFunction

*/

interface TmaFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "TmaFunction";

// Attribute Names

//

const string id = "id";

const string tmaSubunitNumber = "tmaSubunitNumber";

const string tmaStateFlag = "tmaStateFlag";

const string tmaFunctionFlag = "tmaFunctionFlag";

const string tmaMinGain = "tmaMinGain";

const string tmaMaxGain = "tmaMaxGain";

const string tmaResolution = "tmaResolution";

const string tmaGainFigure = "tmaGainFigure";

const string tmaNumberOfSubunits = "tmaNumberOfSubunits";

const string tmaBaseStationId = "tmaBaseStationId";

const string tmaSectorId = "tmaSectorId";

const string tmaAntennaBearing = "tmaAntennaBearing";

const string tmaInstalledMechanicalTilt = "tmaInstalledMechanicalTilt";

const string tmaSubunitType = "tmaSubunitType";

const string tmaSubunitRxFrequencyBand = "tmaSubunitRxFrequencyBand";

const string tmaSubunitTxFrequencyBand = "tmaSubunitTxFrequencyBand";

const string tmaGainResolution = "tmaGainResolution";

const string relatedCellList = "relatedCellList";

};

/*

* Definitions for MO class SectorEquipmentFunction

*/

interface SectorEquipmentFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SectorEquipmentFunction";

// Attribute Names

//

const string id = "id";

const string fqBand = "fqBand";

const string confOutputPower = "confOutputPower";

const string theTmaList = "theTmaList";

const string theAntennaList = "theAntennaList";

const string theCellList = "theCellList";

const string referencedBy = "referencedBy";

};

/*

* Definitions for MO class CommonBSFunction

*/

interface CommonBSFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "CommonBSFunction";

// Attribute Names

//

const string id = "id";

const string sharedTechnologies = "sharedTechnologies";

};

/*

* Definitions for MO class GSMCellPart

*/

interface GSMCellPart : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "GSMCellPart";

// Attribute Names

//

const string id = "id";

const string aRFCN = "aRFCN";

const string tsc = "tsc";

const string aTA = "aTA";

const string relatedSectorEquipment = "relatedSectorEquipment";

};

};

module GenericRanNRMAttributeTypes

{

typedef sequence<string> eUTRANFqBandsListType;

typedef sequence<string> nRFqBandsListType;

typedef sequence<string> uTRANFDDFqBandsListType;

typedef sequence<string> uTRANTDDFqBandsListType;

};

#endif //_GENERICRANNRMDEFS_IDL_

# B.3	Solution Set definitions

## B.3.1	XML definition structure

The overall description of the file format of configuration data XML files is provided by 3GPP TS 32.616 [7].

Annex B.3.3 of the present document defines the NRM-specific XML schema genericRanNrm.xsd for the Generic RAN Network Resources IRP NRM defined in 3GPP TS 28.662 [4] (except Repeater object).

XML schema genericRanNrm.xsd explicitly declares NRM-specific XML element types for the related NRM (except Repeater object) Annex B.3.4 of the present document defines the NRM-specific XML schema repeaterNrm.xsd for the Repeater object of the Generic RAN Network Resources IRP NRM defined in 3GPP TS 28.662 [4].

XML schema repeaterNrm.xsd explicitly declares NRM-specific XML element types for Repeater object defined in the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in 3GPP TS 32.616 [7].

## B.3.2	Graphical Representation

The graphical representation is not present in the current version of this specification.

## B.3.3	XML schema "genericRanNrm.xsd"

<?xml version="1.1" encoding="UTF-8"?>

<!--

3GPP TS 28.663 Generic RAN Network Resources IRP

Bulk CM Configuration data file NRM-specific XML schema

genericRanNrm.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema" xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm" xmlns:gn="http://www.3gpp.org/ftp/specs/archive/28_series/28.656#geranNrm"

xmlns:gr="http://www.3gpp.org/ftp/specs/archive/28_series/28.663#genericRanNrm"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.663#genericRanNrm" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.656#geranNrm"/>

<!-- Generic RAN Network Resources IRP NRM attribute related XML types -->

<simpleType name="angleValue">

<restriction base="short">

<minInclusive value="0"/>

<maxInclusive value="3600"/>

</restriction>

</simpleType>

<simpleType name="retGroupName">

<restriction base="string">

<maxLength value="80"/>

</restriction>

</simpleType>

<simpleType name="beamTilt">

<restriction base="string">

<maxLength value="80"/>

</restriction>

</simpleType>

<simpleType name="elevation">

<restriction base="short">

</restriction>

</simpleType>

<simpleType name="latitude">

<restriction base="short">

<maxLength value="80"/>

</restriction>

</simpleType>

<simpleType name="longitude">

<restriction base="string">

<maxLength value="80"/>

</restriction>

</simpleType>

<simpleType name="bearing">

<restriction base="short">

<minInclusive value="0"/>

<maxInclusive value="360"/>

</restriction>

</simpleType>

<simpleType name="tmaFunctionFlag">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="1"/>

</restriction>

</simpleType>

<simpleType name="tmaStateFlag">

<restriction base="unsignedShort">

<minInclusive value="0"/>

<maxInclusive value="1"/>

</restriction>

</simpleType>

<simpleType name="fourOctets">

<restriction base="hexBinary">

<length value="4"/>

</restriction>

</simpleType>

<complexType name="FqBandsList">

<sequence>

<element name="fqBand" type="string" minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</complexType>

<!-- Generic RAN Network Resources IRP NRM class associated XML elements -->

<element name="SectorEquipmentFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="fqBand" type="short" />

<element name="confOutputPower" type="short" minOccurs="0"/>

<element name="theTmaList" type="xn:dnList" />

<element name="theAntennaList" type="xn:dnList" />

<element name="theCellList" type="xn:dnList" />

<element name="referencedBy" type="xn:dnList" />

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="gr:SectorEquipmentFunctionOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="AntennaFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="retTiltValue" type="gr:angleValue" minOccurs="0"/>

<element name="bearing" type="gr:bearing" minOccurs="0"/>

<element name="beamTilt" type="gr:beamTilt" minOccurs="0"/>

<element name="elevation" type="gr:elevation" minOccurs="0"/>

<element name="latitude" type="gr:latitude" minOccurs="0"/>

<element name="longitude" type="gr:longitude" minOccurs="0"/>

<element name="retGroupName" type="gr:retGroupName" minOccurs="0"/>

<element name="height" type="short" minOccurs="0"/>

<element name="maxAzimuthValue" type="gr:angleValue" minOccurs="0"/>

<element name="minAzimuthValue" type="gr:angleValue" minOccurs="0"/>

<element name="horizBeamwidth" type="gr:angleValue" minOccurs="0"/>

<element name="vertBeamwidth" type="gr:angleValue" minOccurs="0"/>

<element name="referencedBy" type="xn:dnList" />								</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="gr:AntennaFunctionOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="TmaFunction"  substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="tmaSubunitNumber" type="unsignedShort" />

<element name="tmaStateFlag" type="gr:tmaStateFlag" />

<element name="tmaFunctionFlag" type="gr:tmaFunctionFlag" />

<element name="tmaMinGain" type="unsignedShort" />

<element name="tmaMaxGain" type="unsignedShort" />

<element name="tmaResolution" type="unsignedShort" />

<element name="tmaGainFigure" type="unsignedShort" />

<element name="tmaNumberOfSubunits" type="unsignedShort" />

<element name="tmaBaseStationId" type="string" minOccurs="0"/>

<element name="tmaSectorId" type="string" minOccurs="0"/>

<element name="tmaAntennaBearing" type="unsignedShort" minOccurs="0"/>

<element name="tmaInstalledMechanicalTilt" type="short" minOccurs="0"/>

<element name="tmaSubunitType" type="unsignedShort" minOccurs="0"/>

<element name="tmaSubunitRxFrequencyBand" type="gr:fourOctets" minOccurs="0"/>

<element name="tmaSubunitTxFrequencyBand" type="gr:fourOctets" minOccurs="0"/>

<element name="tmaGainResolution" type="unsignedShort" minOccurs="0"/>

<element name="relatedCellList" type="xn:dnList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="gr:TmaFunctionOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="GSMCellPart" substitutionGroup="gn:GsmCellOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="aRFCN" type="string"/>

<element name="tsc" type="long"/>

<element name="aTA" type="short"/>

<element name="relatedSectorEquipment" type="xn:dn"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="gr:GSMCellPartOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="CommonBSFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="sharedTechnologies" type="short"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="gr:CommonBSFunctionOptionallyContainedNrmClass"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SectorEquipmentFunctionOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="AntennaFunctionOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="TmaFunctionOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="GSMCellPartOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

<element name="CommonBSFunctionOptionallyContainedNrmClass" type="xn:NrmClass" abstract="true"/>

</schema>