3GPP TSG-SA5 Meeting #160	S5-252719

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0226 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.104 Multi Domain Resource Optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MDA capability to support analytics on different domains of the network and considers the cross domain impacts for network actions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces MDA support for multi-domain resource optimization |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No MDA support for Multi-domain resource optimization |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.x, 8.5.1.2, 8.5.x, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

#### 7.2.2.1	Service experience analysis

##### 7.2.2.1.1	Description

This MDA capability is for the service experience analysis.

##### 7.2.2.1.2	Use case

Service experience of end user is key indicator that directly reflects the user satisfaction degree. In 5G system, the diversity of network services is expanding all the time and the requirements of different services especially from vertical users are being standardized. Considering these diverse requirements and expectation from end user perspective (e.g. priorities of SLA related attributes such as latency, throughput, maximum number of users or different required values of these attributes), the service experience as a comprehensive indicator need to be extensively analysed.

##### 7.2.2.1.3	Requirements

Table 7.2.2.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-SER_EXP_MDA-01 | MDA capability for service experience analysis shall include identifying the source of service experience issue, e.g. RAN issue, CN issue, TN issue, UE issue, service provider issue. | Service experience analysis |
| REQ-SER_EXP_MDA-02 | MDA capability for service experience analysis shall include providing the analytics output with following information describing the current service experience aspects and potentially future prediction: - The predicted future service experience and/or observed service experience statistics. - Service experience degradation root cause analysis. | Service experience analysis |
| REQ-SER_EXP_MDA-03 | MDA capability for service experience analysis shall include providing the level of service experience. | Service experience analysis |
| REQ-SER_EXP_MDA-04 | MDA capability for service experience analysis shall include providing the recommendation for improving service experience. | Service experience analysis |
| REQ-SER_EXP_MDA-05 | MDA capability for service analysis should include the ability to provide service experience analysis across or within domains. | Service experience analysis |
| REQ-SER_EXP_MDA-06 | MDA capability for service analysis should include the ability to provide data correlation analytics across or within domains. | Service experience analysis |

Start of next change

### 7.2.x	Multi Domain Resource Optimization

##### 7.2.x.1	Description

MDA may provide analytics on different domains of the network, including radio, core and transport domains and on different functions in these domains including physical and virtual network functions.

##### 7.2.x.2	Use case

The behaviour of the different network domains (radio, core and transport) and functions may be coordinated, requiring that any optimization actions need to be coordinatedbetween these domains or functions. The MDA may need to support a multi-domain resource optimization capability that provides information on degree of correlation observed in the data from these network domains / functions.

Consider an example where the performance expectations for end-to-end network slice need to be fulfilled. To fulfil these requirements, the performance at relevant radio network, core network and transport network need to be considered and these different performance data from different domains need to be correlated. The information on the correlation between these domains provided by multi domain analytics capability can provide insights on how to fulfil the end-to-end performance expectations.

Multi domain resource optimization could also evaluate the related impact of one domain on the other based on which it derives recommendations for how to optimize resources among these domains in a coordinatedway. For example, to optimize the allocation of load among cells, multi domain resource optimization may provide a report that contains recommendations on changes to be concurrently made to the edge cloud resources hosting the cells functions (CU-CP/ CU-UP/ DU) as well as on the handover parameters of the cells to allow a set of users to be moved among those cells.

##### 7.2.x.3	Requirements

Table 7.2.x.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-MDA-MARO-1 | The MnS for MDA should include the capability to provide a report on the coordinated simultaneous optimization actions to be undertaken in multiple network domains including physical and virtual infrastructure. | Multi-domain resource optimization |
| REQ-MDA-MARO-2 | The MnS for MDA should have the capability to get data from different domains to prepare the correlation analytics across these domains. | Multi-domain resource optimization |

Start of next change

## 8.5	Data type definitions

### 8.5.1	RecommendedAction <<dataType>>

#### 8.5.1.1	Definition

This data type specifies the type of recommended action in the analytics output.

#### 8.5.1.2	Information elements

Table 8.5.1.2-1

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| recommended3GPPActions | It contains the recommendations actions concerning 3GPP defined operations on MOIs. | O | type: Recommended3GPPAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedNon3GPPActions | It contains the recommended actions related to non-3GPP operations for 3GPP management system to interact with non-3GPP management system. | O | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedHumanReadableActions | It contains the recommendations on human readable actions. NOTE: Further details of recommended human readable actions are not specified. | O | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| startTime | Start time for the time window when the specific action needs to be applied | O | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| endTime | End time for the time window when the specific action needs to be applied | O | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of change