3GPP TSG-SA5 Meeting #161	S5-252828

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | - | Rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to draftCR TS 28.105 Pre-specialised training and Fine-tuning use cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Intel, China Mobile, ZTE, Huawei, Rakuten, DTAG, AT&T, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The pre-specialised training use case has been discussed and agreed in the draft CR; however, the corresponding requirements are missing. In addition, the pre-specialised training use case should be accompanied by the Fine-tuning use case. This proposal introduces the Fine-tuning use case description and the corresponding requirements.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the missing Fine-tuning use case and corresponding requirements. - Provide further clarifications on Pre-specialised Training and Fine-tuning. - Add the missing corresponding terms. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is an input to Draft CR TS 28.105 in S5-251764. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ML model: a manageable representation of an ML model algorithm.

NOTE 1:	an ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output.

NOTE 2:	ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification.

NOTE 3:	ML model may include metadata. Metadata may include e.g. information related to the trained model, and applicable runtime context.

ML model training: a process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model.

ML model initial training: a process of training an initial version of an ML model.

ML model re-training: a process of training a previously trained version of an ML model and generate a new version.

NOTE 4:	a new version of a trained ML model supports the same type of inference as the previous version of the ML model, i.e., the data type of inference input and data type of inference output remain unchanged between the two versions of the ML model, but parameter values might be different for the re-trained model.

ML model pre-specialized training: the process of training an ML model on a dataset not specific to any type of inference.

NOTE x1: 	the pre-specialised trained model supports an inference scope that may be potentially adapted to support a list of inference types, such as MDA types in MDA, analytics types in NWDAF, type of AI/ML supported functions in NG-RAN, or vendor-specific extensions.

ML model Fine-tuning: the process of training a pre-specialised trained ML model to narrow down its inference scope to a new single inference type, generating a new ML model.

NOTE x2: 	the inference scope refers to a list of inference types that the ML model may be potentially adapted to support.

NOTE x3: 	the type of inference represents the specific type of ML inference supported by the model, such as MDA types in MDA, Analytics types in NWDAF, type of AI/ML supported functions in NG-RAN, or vendor-specific extensions.

Distributed training: a process of distributing the training workload across multiple ML training functions.

Federated Learning:  a distributed machine learning approach where the ML model is trained collaboratively by multiple ML training functions. This includes multiple FL clients, which perform training on local data, and one FL server, which aggregates model outcomes from the clients iteratively without exchanging data samples.

Horizontal Federated Learning:  a federated learning technique without exchanging/sharing local data set, wherein the local data set in different HFL clients for local model training have the same feature space for different samples.

FL Client: a training function that trains an ML model on local data and shares only the model updates with the FL server, preserving data privacy.

FL Server: a function that aggregates the ML model updates from FL Clients to produce a global ML model.

Reinforcement Learning: a machine learning approach in which an RL agent interacts with an RL environment by observing states, taking actions and receiving rewards as feedback. The RL agent learns a decision making policy by maximizing rewards over time through trial and error.

ML model joint training: a process of training a group of ML models.

ML training function: a logical function with ML model training capabilities.

ML model testing: a process of testing an ML model using testing data.

ML testing function: a logical function with ML model testing capabilities.

AI/ML inference: a process of running a set of input data through a trained ML model to produce set of output data, such as predictions.

NOTE 5:	the inference represents the process to realize the AI capabilities by utilizing a trained ML model and other AI enablers if needed, hence the AI/ML prefix is used when referring to inference as compared to training and testing.

AI/ML inference function: a logical function that employs trained ML model(s) to conduct inference.

AI/ML inference emulation: running the inference process to evaluate the performance of an ML model in an emulation environment before deploying it into the target environment.

ML model deployment: a process of making a trained ML model available for use in the target environment.

ML knowledge: the information representing the experience gained by the training an ML Model

NOTE 4: Examples of experience include statistics (e.g. a distribution) or summaries (e.g. tables) indicating the ML model’s recommended output for a given set of input data.

Next change

# 4a	AI/ML management functionality and service framework

## 4a.0	ML model lifecycle

AI/ML techniques are widely used in 5GS (including 5GC, NG-RAN, and management system), the generic AI/ML operational workflow shown in Figure 4a.0-1, highlights the main steps of an ML model lifecycle.

Figure 4a.0-1: ML model lifecycle

The ML model lifecycle includes training, testing, emulation, deployment, and inference. These steps are briefly described below:

-	ML model training: training, including initial training, re-training, pre-specialised training and fine-tunning. Training could be for a single ML model or a group of ML models. It may also include validation of the ML model(s) to evaluate the performance when the ML model(s) performs on the validation data. If the validation result does not meet the expectation (e.g., the variance is not acceptable), the MnS Producer may decide to re-train the ML model(s).

-	ML model testing: testing of a validated ML model to evaluate the performance of the trained ML model when it performs on testing data. If the testing result meets the expectations, the ML model may proceed to the next step If the testing result does not meet the expectations, the ML model needs to be re-trained.

-	AI/ML inference emulation: running an ML model for inference in an emulation environment. The purpose is to evaluate the inference performance of the ML model in the emulation environment prior to applying it to the target network or system. If the emulation result does not meet the expectation (e.g., inference performance does not meet the target, or the ML model negatively impacts the performance of other existing functionalities) the ML model needs to be re-trained.

NOTE:	The AI/ML inference emulation is considered optional and can be skipped in the ML model lifecycle.

-	ML model deployment: ML model deployment includes the ML model loading process (a.k.a. a sequence of atomic actions) to make a trained ML model available for use at the target AI/ML inference function.

ML model deployment may not be needed in some cases, for example when the training function and inference function are co-located.

-	AI/ML inference: performing inference using trained ML model(s) by the AI/ML inference function. The AI/ML inference may also trigger model re-training or update based on e.g., performance monitoring and evaluation.

NOTE:	depending on system implementation and AI/ML functionality arrangments, both AI/ML inference emulation and ML deployment steps may be skiped.

Next change

#### 6.2b.2.X3	ML Pre-specialised training

ML model pre-specialised training refers to the process of training an ML Model using a dataset that is not specific to any single type of inference. This means that the ML model training is not intended to support only one type of inference but rather leverages commonalities among multiple use cases. ML pre-specialised training can be appliedto AI/ML-based use cases specified in [2], and [3]. For example, an ML model could be pre- specialised trained using dataset from SLS analysis capability group covering type of inference including ServiceExperienceAnalysis, NetworkSliceThroughputAnalysis, NetworkSliceTrafficAnalysis, NetworkSliceLoadAnalysis and E2ElatencyAnalysis (see TS 28.104 []).

A pre-specialised trained model is not designed to conduct inference for a specific inference type, but this does not preclude the possibility for a pre-specialised model to conduct inference once it achieved performance requirement for a specific inference type.

A pre-specialised trained model can be fine-tuned to narrow down its inference scope, evolving into a new ML model with a single inference type.

The consumer may want to know which ML model could be pre-specialised trained, so the producer shall identify whether an ML model could be pre-specialised trained.

#### 6.2b.2.X4	ML Fine-tuning of pre-specialised trained model

Fine-tuning of a pre-specialised trained ML model narrows down its inference scope to support a single inference type. This means that a fine-tuned ML model can only support a single inference type for conducting inference.

Fine-tuning differs from re-training, where an ML model is re-trained on the same dataset and inference type for which it was previously trained.

The key distinction between re-training and fine-tuning is that re-training results in a new version of the same ML model for the same inference type, while fine-tuning produces a new ML model with an adapted inference scope or a single inference type.

Consumer may request fine-tuning of a pre-specialised trained ML model, and MLT MnS producer may evaluate whether the pre-specialised ML Model can be fine-tuned according to the training requirements.

Authentication procedures need to be established with appropriate tunnelling and authorisation mechanisms prior to an ML fine-tuning training request.

Next change

### 6.2b.3	Requirements for ML model training

Table 6.2b.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to request ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-02 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the data sources containing the candidate training data for ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-03 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the AI/ML inference name of the ML model to be trained. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-04 | The ML training MnS producer shall have a capability to provide the training result to the ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-05 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to configure the thresholds of the performance measurements and/or KPIs to trigger the re-training of an ML model. (See Note) | ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-06 | The ML training MnS producer shall have a capability to provide the version number of the ML model when it is generated by ML model re-training to the authorized ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-07 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage the training process, including starting, suspending, or resuming the training process, and configuring the ML context for ML model training. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2), ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-08 | The ML training MnS producer should have a capability to provide the grouping of ML models to an authorized ML training MnS consumer to enable coordinated inference. | ML model joint training (clause 6.2b.1.2.6) |
| REQ- ML_TRAIN-FUN-09 | The ML training MnS producer should have a capability to allow an authorized ML training MnS consumer to request joint training of a group of ML models. | ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-10 | The ML training MnS producer should have a capability to jointly train a group of ML models and provide the training results to an authorized consumer. | ML model joint training (clause 6.2b.2.6) |
| REQ-ML_SELECT-01 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to discover the properties of available ML models including the contexts under which each of the models were trained. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to select an ML model to be used for inference. | ML models and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to request for information and be informed about the available alternative ML models of differing complexity and performance. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-04 | The 3GPP management system shall have a capability to provide a selected ML model to the authorized ML training MnS consumer. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_TRAIN- MGT-01 | The ML training MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the specific ML model training, e.g. to modify the request or to delete the request. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model Training Processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-02 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage and configure one or more training processes, e.g. to start, suspend or restart the training. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer (e.g. the function/model different from the function that generated a request for ML model training) to request for a report on the outcomes of a specific training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to define the reporting characteristics related to a specific training request or training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have a capability to enable the ML training function to report to any authorized ML training MnS consumer about specific ML model training process and/or report about the outcomes of any such ML model training process. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an ML training function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer of AI/ML decisions (e.g. a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by an AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall have a capability to enable an authorized consumer to provide to the ML Training MnS producer, a training data quality score, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer of ML decisions (e.g. an AI/ML inference function) to provide to an authorized consumer of ML decisions (e.g. a controller) an AI/ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_VLD-01 | The ML training MnS producer should have a capability to validate the ML models during the ML model training process and report the performance of the ML models on both the training data and validation data to the authorized consumer. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-ML_VLD-02 | The ML training MnS producer should have a capability to report the ratio (in terms of quantity of data samples) of the training data and validation data used during the ML model training and validation process. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-TRAIN_EFF-01 | The 3GPP management system should have the capability to allow an authorized consumer to configure an ML training function to report the effectiveness of data used for model training. | Training data effectiveness reporting (clause 6.2b.2.8) |
| REQ-ML_TRAIN_PM-1 | The ML Training MnS producer should have a capability to allow an authorized consumer to get the capabilities about what kind of ML models the ML training function is able to train. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-2 | The ML Training MnS producer should have a capability to allow an authorized consumer to query what performance indicators are supported by the ML model training for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-3 | The ML Training MnS producer should have a capability to allow an authorized consumer to select the performance indicators from those supported by the ML training function for reporting the training performance for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-4 | The ML Training MnS producer should have a capability to allow an authorized consumer to provide the performance requirements for the ML model training using the selected the performance indicators from those supported by the ML training function. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-MLKTL-1 | The 3GPP management system should have a capability to enable an authorized MnS consumer to discover or request all or part of the available shareable knowledge at a given MLKTL MnS producer. | ML-Knowledge-based Transfer Learning (clause 6.2b.2.10.2.1) |
| REQ-MLKTL-2 | The 3GPP management system should have a capability for an MLKTL MnS producer to provide to an authorized MnS consumer all or part of its available shareable knowledge | ML-Knowledge-based Transfer Learning (clause 6.2b.2.10.2.1) |
| REQ-MLKTL-3 | The 3GPP management system should have a capability enabling an authorized MnS consumer to request a MLKTL MnS producer to initiate and execute a transfer learning instance to a specified ML model or ML-enabled function | ML-Knowledge-based Transfer Learning (clause 6.2b.2.10.2.2) |
| REQ-MLKTL-4 | The 3GPP management system should have a capability to enable an authorized MnS consumer to manage or control the knowledge request or the knowledge process or transfer learning process, e.g. to suspend, re-activate or cancel the ML Knowledge Request; or to adjust the description of the desired knowledge NOTE: An example MnS consumers include an operator or the function that generated the request for available Knowledge | ML-Knowledge-based Transfer Learning (clause 6.2b.2.10.2.2) |
| REQ-MLKTL-5 | The 3GPP management system should have a capability to enable an ML model or ML training function to register available knowledge to a shared knowledge repository, e.g. through a ML Knowledge Registration process | ML-Knowledge-based Transfer Learning (clause 6.2b.2.10.2.2) |
| REQ-MLKTL-6 | The 3GPP management system should have a capability enabling an authorized MnS consumer to request the Knowledge Repository to provide some or all the knowledge available for sharing based on specific criteria | ML-Knowledge-based Transfer Learning (clause 6.2b.2.10.2.2) |
| REQ-ML_TRAIN_CLUSTER-01 | The ML Training MnS producer should have a capability for an authorized MnS consumer to request training of a cluster of ML models as per clustering criteria associated to a set of multiple contexts from a previously trained ML model. | ML model training for multiple contexts (clause 6.2b.2.11) |
| REQ-ML_TRAIN-PRE-01 | The ML training MnS producer should have a capability allowing an authorized ML training MnS consumer to request pre-specialized training of a ML model. | ML Pre-specialized training (clause 6.2b.2.12) |
| REQ-ML_TRAIN_DST-01 | The 3GPP management system should enable an authorized consumer to provide information on the training dataset distribution. | Training data statistics (clause 6.2b.2.17) |
| REQ-ML_TRAIN_DST-02 | The 3GPP management system should enable an authorized consumer to provide information on the usage of outliers in the training dataset. | Training data statistics (clause 6.2b.2.17) |
| REQ-ML_DIST-TRNG-01 | The ML training MnS producer should have a capability allowing and authorized consumer to provide distributed training requirements to the MnS Producer. | Management of Distributed training (clause 6.2b.2.14) |
| REQ-ML_TRAIN_FL-1; The ML training MnS producer should have a capability allowing an authorized consumer to discover the FL roles (FL server or FL client) in Federated Learning.; Management of different roles in Federated Learning (Clause 6.2b.2.15.2.1). REQ-ML_TRAIN_FL-2; The ML training MnS producer should have a capability allowing an authorized consumer to provide FL training requirements to the MnS Producer.; Management of different roles in Federated Learning (Clause 6.2b.2.15.2.1). REQ-ML_TRAIN_FL-3; The ML training MnS producer should have a capability allowing an authorized consumer to provide requirements for selecting (including adding and removing) FL clients in Federated Learning to the MnS Producer.; Management of different roles in Federated Learning (Clause 6.2b.2.15.2.1). REQ-ML_TRAIN_FL-4; The ML training MnS producer should have a capability allowing an authorized consumer to get the performance of the global ML model on each participating FL Client.; Management of different roles in Federated Learning (Clause 6.2b.2.15.2.1). REQ-ML_TRAIN_FL-5; The ML training MnS producer should have a capability to report the information about the contribution of each FL client to the FL process to MnS consumer.; Management of different roles in Federated Learning (Clause 6.2b.2.15.2.1). REQ-ML_TRAIN_FL-6; The ML training MnS producer should have a capability to report the candidate FL Clients for the FL process.; Management of different roles in Federated Learning (Clause 6.2b.2.15.2.1). REQ-RL_TRAIN_01; The ML training MnS producer should have a capability allowing an authorized MnS consumer to query if RL training is supported.; Enabling Reinforcement Learning (6.2b.2.16.2.1). REQ-RL_TRAIN_02; The ML Training MnS producer for RL agent training component shall have a capability to report RL types (i.e., online RL, offline RL) to an authorized consumer.; Enabling Reinforcement Learning (6.2b.2.16.2.1). REQ-RL_TRAIN_03; The ML Training MnS producer for RL agent training component shall have a capability to allow an authorized consumer to get the type and scope of the RL environment for which an RL model has been trained.; Exploration in Reinforcement Learning (6.2b.2.16.2.2). REQ-RL_TRAIN_04; The ML Training MnS producer for RL agent training component shall have a capability to allow an authorized consumer to select the type of the RL environment for which an RL model is to be trained.; Exploration in Reinforcement Learning (6.2b.2.16.2.2). REQ-RL_TRAIN_05; The ML Training MnS producer for RL agent training component shall have a capability to allow an authorized consumer to provide the scope of the RL environment for which an RL model is to be trained.; Exploration in Reinforcement Learning (6.2b.2.16.2.2). REQ-RL_TRAIN_06; The ML training MnS producer should have a capability allowing an authorized MnS consumer to provide network performance requirements of performing RL training.; Exploration in Reinforcement Learning (6.2b.2.16.2.2). REQ-RL_TRAIN_07; The ML training MnS producer should have a capability to allow an authorized MnS consumer to specify the configuration range that the RL agent is allowed to explore.; Exploration in Reinforcement Learning (6.2b.2.16.2.2). REQ- ML_TRAIN-FT-x1; The ML training MnS producer should have a capability to enable an authorised consumer to request the fine-tuning of a pre-specialised trained ML model.; ML Pre-specialised training (clause 6.2b.2.X3), ML Fine-Tuning (clause 6.2b.2.X4). REQ- ML_TRAIN-FT-x2; The ML training MnS producer should have a capability allowing the consumer to specify the training type of an ML model training request such as pre-specialised training, and fine-tuning.; ML Pre-specialised training (clause 6.2b.2.X3), ML Fine-Tuning (clause 6.2b.2.X4). NOTE: The performance measurements and KPIs are specific to each type (i.e., the inference type that the ML model supports) of ML model. |

End of changes