3GPP TSG-SA5 Meeting #161	S5-252850

Fukuoka, Japan, 19 - 23 May 2025	revision of S5-252358

Source:	Nokia

Title:	Pseudo-CR on solution for Dynamic CCLs

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.2.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add solution for the use case of Dynamic composition of CCLs as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[3]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3".

[X]	3GPP TS 28.572: "Management and orchestration; Management of planned configurations".

* * * Next Change * * * *

### 6.3.1	ClosedControlLoop

#### 6.3.1.1	Definition

This IOC represents the closed control loop. It represents the information for controlling and monitoring a CCL associated with a stated scope.

The ClosedControlLoop is name-contained by SubNetwork or ManagedElement and is associated with a CCLreport that contains reported  information about the CCL. Accordingly, the report about a CCL can exist even when the CCL is deleted.

The capabilities of the CCL are contained in one or more CCLPurposes that describe what the CCL is capable of doing or can be configured to do - including information the network resources for which the CCL can execute decisions and actions. So, the ClosedControlLoop is associated with one or more CCLPurpose(s) that indicate(s) a list of characteristics that describe what a CCL can/is expected to be able to do. The purpose describes the type of functionality that can be executed including problem recovery and fault management .

The operational information about the CCL is contained in the CCLScope(s), so the ClosedControlLoop is associated with one or more CCLScope(s). The CCLScope defines what the CCL has been configured to read, evaluate, control, etc.

A CCL can be created from several components that are dynamically composed from a set of management services, each representing one component of the CCL. The attribute cCLComponents indicates the list of components which are combined to create a CCL.

The attribute  identifies the type of CCL that needs to be composed. The specific details of the purpose that is fulfilled by the CCL are then written into the CCL purpose.

#### 6.3.1.2	Attributes

The CCLControlLoop IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.1.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cCLComponentList | O | T | T | T | T |
| cCLType | O | T | T | T | T |
| cCLActionTrigger | M | T | T | F | T |
| desiredBehavior | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| cCLPurposeRefList | M | T | T | T | T |

#### 6.3.1.3	Attribute constraints

None

#### 6.3.1.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

### 6.3.x	CCLComponent <<dataType>>

6.3.x.1	Definition

This dataType defines a CCL component that can be used or has been used to dynamically compose a closed control loop by the MnS consumer.

6.3.x.2	Attributes

The CCLComponent IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cCLComponentRole | M | T | T | T | T |
| cCLComponentIdentification | M | T | T | F | T |

6.3.x.3	Attribute constraints

None

6.3.x.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

* * * Next Change * * * *

## 6.4	Attribute definitions

### 6.4.1	Attribute properties

Table 6.4.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| scopeType | It indicates the type of scope that represented by the particular scope instance. allowedValues: CCL_MEASUREMENT_SCOPE, CCL_TARGET_SCOPE, CCL_CONTROL_SCOPE, CCL_IMPACT_SCOPE Editor’s Note: The allowed values will be revisited | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLComponentList | It indicates the list of components ating as steps of the CCL, each either a MnF or a MnS producer whose services can be part of the CCL. The cCLComponent may have a role among MONITOR; ANALYSIS; DECISION; EXECUTION. Or OTHER. OTHER. Is used for example in the caes where a components fulfile more than 1 role or where the role can be siml y described by the four options. The cCLComponents are sequenced, i.e., cCLComponents is an ordred list. For example, if there are 2 steps that contribute to the analysis role, it is necessary to show how those steps are sequenced. The order in which they are listed indicates the order in which their services should be chained to complete the CCL | type: CCLComponent multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| cCLType | It indicates a type or Category of CCL that is to be instantiated or dynamically composition. It indicates the kind of capability that will be accomplished by the CCL instance, e.g. ENERGYOPTIMIZATION, SLICEASSURANCE, etc The specific details, characteristics and behavior of a CCL for a given CCL type are then written into the CCL purpose. Editor’s Note: Documentation and Allowed values will be revisited | type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLComponentRole | It indicates a role accomplished by CCL component. Allowed values: MONITOR; ANALYSIS; DECISION; EXECUTION, OTHER. Is used for example in the caes where a components fulfile more than 1 role or where the role can be siml y described by the four options | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLComponentIdentification | It indicates the entity accomplishing the component. It may be the the DN of an MOI or the combination of URI and DN that can be used to fulfil that role. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLActionTrigger | This defines the criteria/conditions under which the CCL is allowed to take actions. | Type: CCLTrigger multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| desiredBehavior | This will define the corresponding behavior of the CCL. The behaviors can be represented by an ENUM to include: - DECISION_ACTIVATION: The CCL executes the recommendations that it derives on to the network. - NOTIFY_RCOMMENDATION: The CCL starts processing input to derive recommendations but without the corresponding actions executed on the network. Instead, the recommendation is notified to the consumer who then considers whether it should be applied or not. - DO_NOTHING: do not do anything. | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

* * * Next Change * * * *

# 7	Procedures

## 7.C	Procedure for conditional composition of CCLs

Figure 7.C-1: Procedure and interactions for conditional composition of CCLs

Step 0: There exists an object exposing the MnS producer responsible for instantiating CCLs. This object may be represented by a subnetwork or managed element.

Step 1: The MnS consumer creates on the MnS producer responsible for instantiating CCLs the CCL composition operations desription which contains the details on the provisioning actions to be undertaken – in this case the operations for composing the CCL. These may include

- createMOI operations for instantiatng the objects to be used as components of the closd control loop, e.g., a PMJob to be used to collect data

- modifyMOI operations for configuring the instantiated components to enable thenm oprate as a single loop, e.g., to configure te PMJob to delvier data to an analytics sinstance

Step 2: The MnS consumer creates on the MnS producer responsible for instantiating CCLs the set of conditios to be evaluated fot composing the CCL. Thse conditions are created as an instance of TriggerConditionDescriptor defned in 28.572. TriggerConditionDescriptor describes the conditions that should be evaluated including performance, provisioning and fault management conditions

Step 3: The MnS producer monitors the network to detect when the conditions defined in TriggerConditionDescriptor evaluate to TRUE.

Step 4: If conditions in TriggerConditionDescriptor evaluate to TRUE, the MnS producer triggers execution of CCL composition operations.

Step 5: For the triggered CCL composition, the MnS producer may notify the conditions that triggere dteh composition or the composed CCL.

Step 6: The MnS producer executes the CCL composition operations through interaction with other management functions and services. When the composition is complete, the MnS producer may notify the MnS consumer of composed CCL.

* * * Next Change * * * *

## A.1.1	CCL NRM fragment (Figure 6.2.1.1-1)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class ClosedControlLoop <<InformationObjectClass>>

class CCLPurpose << ProxyClass >>

class CCLScope << InformationObjectClass >>

class CCLReport <<InformationObjectClass>>

class CCLComponent<<InformationObjectClass>>

ManagedEntity "1" *-- "*" ClosedControlLoop: <<names>>

ClosedControlLoop "1" *--> "*" CCLPurpose

ClosedControlLoop "1" *-- "*" CCLScope: <<names>>

ClosedControlLoop "1" *-- "*" CCLReport: <<names>>

ManagedEntity "1" *-- "*" CCLComponent: <<names>>

ClosedControlLoop "1" -r-> "*" CCLComponent

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

note top of CCLPurpose

Can be any of these CCL purposes:

NetworkProblemRecovery

FaultManagement

...

end note

@enduml

Source code for Figure 6.2.1.1-1 CCL NRM fragment

## A.1.2	CCL inheritance relationships (Figure 6.2.1.1-2)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class ClosedControlLoop <<InformationObjectClass>>

class CCLReport <<InformationObjectClass>>

class CCLScope <<InformationObjectClass>>

class CCLComponent<<InformationObjectClass>>

Top <|-- ClosedControlLoop

Top <|-- CCLScope

Top <|-- CCLReport

Top <|-- CCLComponent

@enduml

Source code for Figure 6.2.1.1-2 CCL inheritance relationships

Annex X (informative):
UML code for procedure diagrams

# X.2		UML code for CCL control procedure diagrams

This annex contains the PlantUML source code for the procedure diagrams related to CCL control in clause 7 of the present document.

## X.2.1	Procedure for conditional composition of CCLs (Figure 7.C-1)

@startuml Procedure for conditional composition of CCLs

skinparam Shadowing false

autonumber

skinparam monochrome true

participant "CCL Control MnS consumer" as MNSCS

participant "CCL Control MnS producer" as MNSPD

participant "Management functions" as MNFs

MNSCS -> MNSPD: create CCL composition desription

MNSCS -> MNSPD: create CCL composition conditions\n as an instance of TriggerConditionDescriptor

MNSPD -> MNSPD: Monitor conditions defined\n in TriggerConditionDescriptor

MNSPD -> MNSPD: If conditions in TriggerConditionDescriptor\n evaluate to TRUE, trigger execution\n of CCL composition operations

MNSPD -> MNSCS: Notify  conditions\n and triggering of composition.

Note over MNSPD, MNFs: execute CCL composition operations

MNSPD -> MNSCS: If composition is complete,\n Notify MnS consumer of composed CCL

@enduml

PlantUML source code for Figure 7.C-1 Procedure for conditional composition of CCLs

* * * End of Changes * * * *