3GPP TSG-SA5 Meeting #161	S5-252851

Fukuoka, Japan, 19 - 23 May 2025

Source:	Samsung

Title:	Solution for conflict management

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS 28.567

Version:	0.2.0

Work Item:	CCL

Comments

This contribution proposes the solution for the use case of CCL instantiation based on conditions.

Proposed Changes

* * * First Change * * * *

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-1: Relations for common information models for AI/ML management

Editor’s Note: The handling of Goal, targets or objectives for the general closed control loops is FFS

Figure 6.2.1-x: Conflict Management Control NRM fragment

### 6.2.2	Inheritance

Figure 6.2.2-1: Inheritance Hierarchy for Closed Control Loops

* * * Next Change * * * *

##### 6.3.a	ConflictManagementAndCoordinationEntity

6.3.a.1	Definition

This defines the conflict management functionality.

6.3.a.2	Attributes

The ConflictManagementCtrl IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cCLActionConflictsHandling | M | T | T | F | T |

6.3.a.3	Attribute constraints

None

6.3.a.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 6.3.b	CCLActionConflictsHandling <<datatype>>

6.3.b.1	Definition

This defines the handling of CCL action conflict between the two existing CCLs.

6.3.b.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| conflictInformation | M | T | T | F | T |
| conflictResolution | M | T | T | F | T |
| targetCCL | M | T | F | F | T |

6.3.b.3	Attribute constraints

None

6.3.b.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 6.3.c 	ConflictInformation <<datatype>>

6.3.c.1	Definition

This defines the information related with a conflicting CCLs that have been detected.

6.3.c.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| conflictingCCLId | M | T | T | F | T |
| conflictingActions | M | T | T | F | T |

6.3.c.3	Attribute constraints

None

6.3.c.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 6.3.d 	ActionConflictResolution <<datatype>>

6.3.d.1	Definition

This defines the information related with conflict resolution configured by the MnS Consumer.

6.3.d.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| conflictingCCLId | M | T | T | F | T |
| cCLGoalBreachPercentage | M | T | F | F | T |

6.3.d.3	Attribute constraints

None

6.3.d.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

* * * Next Change * * * *

### 6.3.1	ClosedControlLoop

#### 6.3.1.1	Definition

This IOC represents the closed control loop. It represents the information for controlling and monitoring a CCL associated with a stated scope.

The ClosedControlLoop is name-contained by SubNetwork or ManagedElement and is associated with a CCLreport that contains reported  information about the CCL. Accordingly, the report about a CCL can exist even when the CCL is deleted.

The capabilities of the CCL are contained in one or more CCLPurposes that describe what the CCL is capable of doing or can be configured to do - including information the network resources for which the CCL can execute decisions and actions. So, the ClosedControlLoop is associated with one or more CCLPurpose(s) that indicate(s) a list of characteristics that describe what a CCL can/is expected to be able to do. The purpose describes the type of functionality that can be executed including problem recovery and fault management .

The operational information about the CCL is contained in the CCLScope(s), so the ClosedControlLoop is associated with one or more CCLScope(s). The CCLScope defines what the CCL has been configured to read, evaluate, control, etc.

The attribute cCLPriority attribute provides the priority assigned to the CCL by the MnS Cosnumer.

#### 6.3.1.2	Attributes

The CCLControlLoop IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.1.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cCLPriority | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| cCLPurposeRefList | M | T | T | T | T |

#### 6.3.1.3	Attribute constraints

None

#### 6.3.1.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

* * * Next Change * * * *

## 6.4	Attribute definitions

### 6.4.1	Attribute properties

Table 6.4.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| scopeType | It indicates the type of scope that represented by the particular scope instance. allowedValues: CCL_MEASUREMENT_SCOPE, CCL_TARGET_SCOPE, CCL_CONTROL_SCOPE, CCL_IMPACT_SCOPE Editor’s Note: The allowed values will be revisited | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLActionConflictsHandling | This defines the handling of CCL action conflict between the two existing CCLs. | Type: cCLActionConflictsHandling multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictInformation | This defines the information related with a conflicting CCL. | Type: ConflictInformation multiplicity: * isOrdered: True isUnique: False defaultValue: None isNullable: False |
| conflictResolution | This defines the information related with conflict resolution. | Type: ConflictResolution multiplicity: * isOrdered: True isUnique: False defaultValue: None isNullable: False |
| targetCCL | The identification of the CCL that need to be deleted or updated to resolve conflict. This will be decided as per the information ConflictResolution. | Type: Dn multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingCCLId | This indicates the CCL identification | Type: Dn multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingActions | This provides the set of actions that have been taken by the CCL as part of the Execute step. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLPriority | This provides the priority of the CCL. This will be the numerical value between 1 to 10, with 1 being the least priority. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLMetricBreachPercentage | It defines the breach percentage per metric in terms of how bad the metric(s) is breached. For example, if the metric of guaranteed throughput is 200mbps and the actual throughput is coming to be 100mbps then the breach percentage would be 50%. The CCL that have higher percentage of breach will be prioritized | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

* * * Next Change * * * *

## A.1.2	CCL inheritance relationships (Figure 6.2.1.1-2)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class ClosedControlLoop <<InformationObjectClass>>

class CCLReport <<InformationObjectClass>>

class CCLScope <<InformationObjectClass>>

class ConflictManagementAndCoordinationEntity<<InformationObjectClass>>

Top <|-- ClosedControlLoop

Top <|-- CCLScope

Top <|-- CCLReport

Top <|-- ConflictManagementAndCoordinationEntity

@enduml

Source code for Figure 6.2.1.1-2 CCL inheritance relationships

* * * Next Change * * * *

# A.1.x	UML code for Figure 6.2.1-x

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class ConflictManagementAndCoordinationEntity<<InformationObjectClass>>

ManagedEntity "1" *-- "*" ConflictManagementAndCoordinationEntity: <<names>>

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

@enduml

* * * Next Change * * * *

* * * End of Changes * * * *