3GPP TSG-SA5 Meeting #161	S5-252916

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.537 | CR | 0039 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS28.537 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MADCOL_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR implements MADCOL_Ph2 draft CR into TS 28.537. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Description of management of external data - Description and requirement for condition based data collection - Update the description and requirements for clause 6.5 Discovery of management data - Add solution description for discovery of management data in clause 6.5 - Add Management data information in clause 5.3 Usage of MnS Registry for different deployment scenarios - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MADCOL_Ph2 content not available in Rel-19 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3, 6.1.1, 6.1.3, 6.4.1, 6.5.1, 6.5.3, 6.5.X (new), 6.5.X.1 (new), 6.5.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS28.622 CR 0566 |  |  |
| Other comments: |  | This CR has dependency to the implementation of the CR 0566 for TS 28.622 Tdoc no. S5-252275 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

### 5.3	Usage of MnS Registry for different deployment scenarios

Following are potential deployment scenarios for MnS Registry (i.e. act as discover MnS producer):

- Deployment scenario #1: Separate MnSRegistry. The MnS Registry and a concrete MnS producer are separate entities with a published interface between them. In this scenario, the MnS Registry is used to store the MnS information (i.e. MnSInfo) for MnS instances and management capability information for management data (i.e. MgmtDataInfo) provided by different MnFs, especially for different MnFs from different vendors. Following is one example, MnF_1 needs to register MnSInfo_1 and MgmtDataInfo_1 in the MnS Registry implemented in MnF_3, and MnF_2 needs to discover/retrieve the MnSInfo_1 and MgmtDataInfo_1 from MnF_3 to consume MnS_1 provided by MnF_1. In this scenario, MnS Registry needs to support both registry and discovery/retrieve capabilities.

Figure 5.3-1 Example of Separate MnSRegistry deployment scenario

- Deployment scenario #2: Embedded MnSRegistry. The MnsRegistry and a concrete MnS producer are hosted by the same entity with an implementation specific interface between them. The MnS Registry is implemented in an MnF and used to store the MnS Information (i.e. MnSInfo) for MnS instances and management capability information for management data (i.e. MgmtDataInfo) provided by the MnF itself. Following is one example, MnF_3 needs to retrieve/discover MnSInfo_1 and MgmtDataInfo_1 in the MnSRegistry implemented in MnF_1. In this scenario, MnS Registry supports discovery/retrieve capabilities only.

Figure 5.3-2 Example of Embedded MnSRegistry deployment scenario

Next change

# 6	Managing management data

## 6.1	Producing and reporting management data

### 6.1.1	Description

Management data is referring to data produced by radio access network functions, core network functions or management functions and used for management purposes. Management data specified by 3GPP for 5G management is classified into 5G performance measurements as defined by TS 28.552 [4], 5G end to end key performance indicators as defined by TS 28.554 [5] and Trace metrics as defined by TS 32.423 [8]. The combined performance measurements and key performance indicators are also called performance metrics.

Management data is produced on request. Therefore, the 3GPP management system needs to enable a data consumer to request management data to be produced. The data requester needs to specify the type of data to be produced as well as the radio access network functions, core network functions and management functions where the data shall be produced. The target managed object instances can be identified in multiple ways:

-	The requester can specify the target managed object instances based on the managed object tree (as defined in the 3GPP Network Resource Models) representing the network and management functions. The simplest approach is to directly identify the managed object instances where data shall be produced. More sophisticated approaches allow to specify one or more subtrees where data shall be produced and may allow also to specify managed object classes to select only object instances of specific classes.

-	The requester can specify one or multiple of the following selection criteria. The system needs to translate this information into the target managed object instances. The selection criteria need to be deterministic in such a way that the target node(s) can be selected unambiguously.

-	Area of interest: In a big network, it makes sense to specify a limited area for which data shall be produced. The area of interest can be expressed for example with a geographical area, one or several cells or one or several tracking areas. The target managed object instances represent network functions serving that area of interest. Geographical areas can be expressed for example with multiple longitude-latitude pairs that define a convex polygon. In the radio domain the geographical area needs to be mapped to the coverage area of cells supported by RAN NE(s). The managed object instances (e.g. NRCellCU, GNBDUFunction) providing service to these cells can be identified as target managed object instances. Of course, the coverage area of the target cell(s) will usually not exactly map to the described shape of the geographical area because, on the one side, the coverage area of cell has no sharp borders due to fast fading effects, on the other side, the coverage area of cells may vary slightly e.g. due to adaptation of the antenna downtilt angle or beamforming configurations. For the mapping between the geographical area and the corresponding managed object instances the cell coverage status at the time of the request shall be used. Later changes affecting the cell coverage shall not be reflected for the mapping.

-	Domain e.g RAN, CN: A consumer might only be interested in analysing and understanding the performance of a particular domain like RAN or Core e.g in case of recurrent issues, a consumer may want to have understanding of a particular domain only for further actions. In such a scenario, it should be possible to indicate the domain from where consumer wants measurements for its usage.

-	Traffic type e.g user plane or control plane: 5G brings clear separation (CUPS) of user plane and control plane in a network, a consumer may leverage it to identify target managed object instances to have measurements from. For example, the measurement report may be expected from user plane nodes only.

-	Slice type e.g eMBB, URLLC: Consumer may mention a particular slice type (eMBB, URLLC, mIoT, V2X, HMTC) as the selection criteria. It may help in narrowing down the target managed object instances, which are part of provided slice type(s).

This MnS to request management data in a simple way shall not be exposed at any network function.

The management data can be requested according to a certain time scheduler. The consumer can e.g. specify a start and stop time or can request for data at specific days of a week or specific hours of a day.

The management data can be requested according to certain conditions, in this case, the management data shall be produced if corresponding conditions are satisfied. The consumer may specify different management data producing control parameters (e.g. producing period) in different management data requests, and each request related to a certain condition. For example, the condition may indicate a performance abnormal scenario when a performance metric crosses a specified threshold. Normally, if the condition is not satisfied, the management data may be produced in a higher period. However, in a performance abnormal scenario, the MnS producer may provide the management data more frequently to enable network optimization, such that it may report the management data produced before the anomaly with a lower period.

After production the data needs to be reported to the data consumers. Reporting can be based on multiple reporting methods such as file or streaming. Data reporting needs to be requested by the data consumer. The requestor needs to specify the control parameters for reporting such as the reporting method and the address the data shall be delivered to.

A data consumer which is unaware of specific managed objects (e.g. NRCellCU, NRCellDU), is able to obtain the management data without relying on the specific managed objects using area based KPIs (e.g. downlink weak coverage ratio or high DL PRB load ratio for the associated area). An area based KPI is a type of the management data which is not associated to specific managed objects. In this case, data requestor specifies the list of names or categories of the area based KPIs for the interested areas. Based on the request, data producer needs to report the area based KPIs to the data consumer. The area based KPIs can be associated with sub-areas of the interested areas.

Depending on access rights and security settings, data consumers may be subject to restrictions regarding the data they can access.

Data is always produced in some context. The data describing this context is called context data. Context data contains information on all interrelated conditions in which the management data is produced. This includes for example the configuration of the measured network functions, information on the network entity where the network function is running such as vendor name or software version, but also alarms associated to the network function or load conditions.

Data consumers processing managemen data in an effort to accomplish some task typically benefit when taking context data into account. For that reason, data consumers should be able to obtain the context data for the management data they obtain. However, access to certain management data does not automatically imply access to all context data. Access to management data and access to context data may be subject to different data security and data protection considerations.

### 6.1.2	Void

### 6.1.3	Requirements

REQ-MDM-PR-1: The 3GPP management system shall enable an authorized data consumer to request management data (specified by 3GPP) to be produced.

REQ-MDM-PR-2: The 3GPP management system shall enable an authorized data consumer to request management data specified by 3GPP to be produced by certain managed object instance(s) only. The selection criteria to determine the managed object instance(s) shall be deterministic in such a way that the target node(s) can be selected unambiguously. The managed object instances can be targeted based on:

-	Area of interest (e.g. list of cells, list of tracking areas or geographical area).

-	Domain (CN or RAN).

-	User plane or control plane.

-	Slice type (e.g. eMBB, URLLC, mIoT, V2X, HMTC).

The MnS to request management data specified by 3GPP in a simple way shall not be exposed at any network function.

The mapping of geographical area to corresponding managed object instances reflects the cell coverage status at the time of the request.

REQ-MDM-PR-3: The 3GPP management system shall enable an authorized data consumer to request management data specified by 3GPP to be produced according to a certain time scheduler.

REQ-MDM-PR-4: The 3GPP management system shall enable an authorized data consumer to request management data (specified by 3GPP) to be reported to the requesting or another authorized data consumer.

REQ-MDM-PR-5: The 3GPP management system shall enable an authorized data consumer to obtain context data for management data. Access to management data does not imply access to context data. Different data privacy considerations may apply.

REQ-MDM-PR-6: The 3GPP management system shall support enabling an authorized data consumer to obtain management data which does not rely on specific managed objects.

REQ-MDM-PR-7: The 3GPP management system shall enable an authorized data consumer to request management data (specified by 3GPP) to be produced according to corresponding conditions.

NOTE: The term "management data specified by 3GPP" relates to

-	5G performance measurements as defined by TS 28.552 [4]

-	5G end to end key performance indicators as defined by TS 28.554 [5], and

-	Trace metrics as defined by TS 32.423 [8].

Next change

## 6.4	Managing external management data

### 6.4.1	Description

Management data which is specified by 3GPP (clause 6.1.1) can be enriched by additional data not specified by 3GPP. This so-called external management data can be produced by data sources of different nature (e.g. sensors) with different formats.

Note that management data that is not defined by 3GPP but specified based on 3GPP defined management data definition templates and frameworks (such as the measurement definition template in TS 32.404 [7]) is not considered as external management data.

External management data can be used for example as additional input for network optimization and prediction.

The management system should be able to manage this kind of data. That means data consumers should be able to request external management data to be produced and reported. The management system should provide support for storing it.

The definition of external data sources and the data formats they use is out of scope of this specification. This functionality is supported based on the assumption that the connection between the 3GPP external entities and 3GPP management system is set up enabling the 3GPP management system to obtain and store the external management data. The mechanism for setting up the connection between the 3GPP management system and external data sources and collection of the external management data is out of scope of this specification. This functionality focuses on the external management data discovery, request and delivery between MnS producer and MnS consumer. Following is an example to illustrate the framework for managing external management data.

Figure 6.4.1-1 Example to illustrate the framework for managing external management data.

The present document defines generic management mechanisms for external management data discovery, request and delivery, which can cope with any kind of external data sources and data formats.

### 6.4.2	Void

### 6.4.3	Requirements

REQ-MDM-ED-1: The 3GPP management system shall enable an authorized data consumer to request external management data to be produced.

REQ-MDM-ED-2: The 3GPP management system shall enable an authorized data consumer to request external management data to be reported to the requesting or another authorized data consumer.

REQ-MDM-ED-3: The 3GPP management system shall support the storing of produced external management data.

REQ-MDM-ED-4: The 3GPP management system shall enable an authorized data consumer to discover stored external management data.

REQ-MDM-ED-5: The 3GPP management system shall enable an authorized data consumer to retrieve stored external management data.

NOTE: The term "external management data" relates to data not specified by 3GPP.

## 6.5	Discovery of management data

### 6.5.1	Description

Discovery of management data mechanism allows MnS consumers to discover what management data can be produced by the 3GPP management system without direct involvement of those MnS services producing the data, which can be time and resource consuming process.

For this mechanism to work MnS producers as entities producing data, need to register what data they can produce by adding a corresponding record in the 3GPP management system. The MnS consumer needs to discover or retrieve what management data (represented by management data name) or what type of management data (represented by management data category) can be produced by MnS producer. In addition, the MnS consumer needs to know what reporting methods are supported for the specified management data.

NOTE: The term "management data produced by 3GPP management system" relates to

-	5G performance measurements as defined by TS 28.552 [4]

-	5G end to end key performance indicators as defined by TS 28.554 [5], and

-	Trace metrics as defined by TS 32.423 [8].

### 6.5.2	Void

### 6.5.3	Requirements

REQ-MDM-DIS-1: The 3GPP management system shall enable an authorized data consumer to discover what management data can be produced by 3GPP management system.

REQ-MDM-DIS-2: The 3GPP management system shall enable an authorized data consumer to discover what category of management data can be produced by 3GPP management system.

REQ-MDM-DIS-3: The 3GPP management system shall enable an authorized data consumer to obtain the reporting methods that are supported for the management data which can be produced by 3GPP management system.

### 6.5.X	Solutions

#### 6.5.X.1	Stage 2 definition

Following are the stage 2 definition for discovery of management data:

-	The operations and notifications of generic provisioning MnS defined in clause 11.1 in TS 28.532 [2].

-	The NRM fragments related to management data discovery (including MnsRegistry IOC, MgmtDataInfo IOC and MnsInfo) defined in clause 4.3.41, clause 4.3.42, and clause 4.3.Z in TS 28.622 [9].

#### 6.5.X.2	Stage 3 definition

Following are the stage 3 definition for discovery of management data:

-	RESTful HTTP-based solution set

-	RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in TS 28.532 [2]. OpenAPI document "TS28532_ProvMnS.yaml" in clause A.1 in TS 28.532 [2].

-	OpenAPI document " TS28623_MnSRegistryNrm.yaml" in clause 4.3 in TS 28.623 [10].

End of Changes