3GPP TSG-SA5 Meeting #161	S5-252926

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0711 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Add new filter for RRC connection number measurements for RedCap service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCap_OAM |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solution for the enhancement on the existing RRC number related measurements to support to monitoring the performance of RedCap service is investigated in clause 5.3.2 of TR 28.876 and the related normative work are recommended. This contribution proposes to add a new filter for RRC related measurements to implement the above recommendation accordingly in TS 28.552. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new filter for RRC number related measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements of RRC connection number measurements for RedCap service cannot be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.4.1, 5.1.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.1.4.1	Mean number of RRC Connections

a)	This measurement provides the mean number of users in RRC connected mode for each NR cell during each granularity period. The measurement is optionally split into subcounters per PLMN ID, and optionally filterable for UE type.

b)	SI.

c)	This measurement is obtained by sampling at a pre-defined interval, the number of users in RRC connected mode for each NR cell and for each PLMN ID, and then taking the arithmetic mean. The measurement is optionally filterable for UE type.

d)	Each measurement is a  single integer value. If the optional measurement is performed, the number of measurements is equal to the number of supported PLMNs.

e)   RRC.ConnMean, or optionally RRC.ConnMean.PLMN, where PLMN identifies the PLMN ID, or RRC.ConnMean_UEType or RRC.ConnMean.PLMN_UEType, where UEType refers to allowed values related to RedCap UE type of uECellBarredAccess in TS 28.541 clause 4.4.1.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the number of RRC connections in connected mode during the granularity period.

5.1.1.4.2	Max number of RRC Connections

a)	This measurement provides the maximum number of users in RRC connected mode for each NR cell during each granularity period. The measurement is optionally split into subcounters per PLMN ID, and optionally filterable for UE type.

b)	SI.

c)	This measurement is obtained by sampling at a pre-defined interval, the number of users in RRC connected mode for each NR cell and for each PLMN ID, and then taking the maximum. The measurement is optionally filterable for UE type.

d)	Each measurement is a single integer value. If the optional measurement is performed, the number of measurements is equal to the number of supported PLMNs.

e)	RRC.ConnMax, or optionally RRC.ConnMax.PLMN, where PLMN identifies the PLMN ID, or RRC.ConnMax_UEType or RRC.ConnMax.PLMN_UEType, where UEType refers to allowed values related to RedCap UE type of uECellBarredAccess in TS 28.541 clause 4.4.1.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the number of RRC connections in connected mode during the granularity period.

End of Change