3GPP TSG-SA5 Meeting #161	S5-253008

Fukuoka, Japan, 19 - 23 May 2025	revision of S5-252368

Source:	Nokia

Title:	Pseudo-CR on Example realizations of CCLs

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.2.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add example realizations of CCLs as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 4	Concepts and overview

## 4.2	Functional steps of a closed control loop

A closed control loop may manage any managed entity, e.g. a network resource or a communication service as described in TS 28.535 [2] and TS 28.536 [3]. Generally, the control loop consists of the steps Monitoring/data collection, Analysis, Decision and Execution. The adjustment of the resources of the managed entity used is completed by the continuous iteration of the steps in a management control loop.

Figure 4.2-1: Steps of a Control Loop
a) the four functional steps; and
b) 2 steps combined into a single management function

-	The "Monitoring/data collection" step is responsible for collecting and pre-processing data from managed entities or from external sources.

-	The "Analysis" step derives insights from the available data obtained in the monitoring/data collection step. The insights provide answers to the questions, for example What is likely to happen, what has happened and why.

-	The "Decision" step is responsible for deriving workflows from insights provided by the analysis step. It decides which reactive, proactive or predictive actions should be taken in consideration of insights obtained in the analysis step.

-	The execution step manages the activation of commands on the controlled resources or entities. The decision step should decide which actions are required, but not necessarily how they should be taken in the managed entities. So, the translation from actions to commands is a responsibility of the execution step.

## 4.2	Realisation of closed control loops

The CCL may be composed on demand into what is called an open-box CCL from discrete entities in the Management system that accomplish the functional steps, e.g., using capabilities offered by the Management system. The CCLs steps (or the components accomplishing them) as well as the interactions among them, are based on 3GPP management services. E.g., the CCL may apply one or more PM jobs for the Monitoring/data collection step and  apply an MDA capability for an analysis stage.

However, the CCL (e.g., in any form in Figure 4.2-1) can be pre-integrated into what are called Closed-Box CCLs. where, the CCLs components and their interactions are assembled prior to instantiating the CCL in the Management system. They are not accessible to the MnS consumers, only the CCL’s external interactions and capabilities, e.g., the control interface to define the CCL scopes is accessible to the MnS consumers.

* * * End of Changes * * * *