3GPP TSG-SA5 Meeting #162	S5-253395

Goteborg, Sweden, 25 - 29 August 2025

Source:	China Mobile

Title:	Rel-19 pCR TS 28.561 Update NDT data generation

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	1.0.0

Work Item:	NDT

Comments

This contribution is proposed to update NDT data generation to improve quality.

Proposed Changes

* * * First Change * * * *

5.4.2.2	Using NDT to generate ML training data

ML training usually requires large amounts of data to guarantee good performance of the ML models. In general, the ML training data for network related use cases is obtained through historical network management data. For instance, assuming that there is a ML model supporting MDA SLS analysis described in TS 28.104 clause 7.2.2, the raw feature of training data could be the enabling data, such as UL/DL throughput, uplink/downlink delay, etc., as specified in clause 8.4.2 of TS 28.104.

However, obtaining data from the network has the following limitations:

-	The quantity of issues happened in actual mobile network is limited.

-	The variety of issues happened in actual mobile network is limited. There could be corner network issues that hardly happen in live network.

Sufficient ML training data plays a key role in training a useful ML model. The more training data provided, the better the performance of the ML model. To overcome these challenges, the MnS consumer can send a request to the NDT to generate data with an indication of data requirements, e.g. data types, required data periods, data sampling periods, etc. When receiving the request from the MnS consumer, the MnS Producer creates the NDT function instance and determines the simulated network objects and data (e.g., PM data, CM data) according to data requirements. The simulated network objects include simulated network areas and managed objects (e.g., network functions, S-NSSAI) in NDT. The MnS Producer could send a notification to the MnS Consumer. The notification includes the estimated time when the simulated objects and generated data can be provided. The MnS Producer receives the feedback information from the MnS consumer indicating whether the estimated time meets the requirements. And then, according to the simulated network objects and data, the MnS Producer collects the data from the physical network used for NDT job simulation.

Subsequently, based on the collected data, the MnS Producer creates/activates network NDT function, executes the network simulation and generates data corresponding to the request by using the NDT. The MnS producer sends a report with the generated data to the MnS consumer, which can be used to enhance model accuracy by providing a wide range of training examples reflecting potential real network conditions.

* * * End of Changes * * * *