3GPP TSG-SA5 Meeting #162 	S5-253405

Stor-Göteborg, Sweden, 25th August 2025 - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0716 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Corrections on SDT measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As indicted by incoming LS S5-253290, RAN2 has SDT related measurements specified in R2-2504742. However, in TS28.522, SDT related measurements are not inline with the RAN2 agreed CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections on SDT measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.23.7, 5.1.1.23.8, 5.1.1.23.9, 5.1.1.23.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

##### 5.1.1.23.7	Mean number of Active UEs in RRC_INACTIVE with ongoing SDT procedure in the DL per cell

a)	This measurement provides the mean number of active UEs (in RRC_INACTIVE state) in the DL in an NRCellDU. The measurement is calculated per PLMN ID and per QoS level  and per supported S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained by aggregating the measurement "Mean number of Active UEs in RRC_INACTIVE with ongoing SDT procedure  in the DL per DRB per cell" (see clause 4.2.1.3a.2 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI) and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MeanRRCInactiveUeDl, or

DRB.MeanRRCInactiveUeDl_Filter, 
where filter is a combination of PLMN and QoS and SNSSAI,

where PLMN represents the PLMN ID, QoS represents the mapped 5QI, and SNSSAI represents S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

*** START OF NEXT CHANGE ***

##### 5.1.1.23.8	Max number of Active UEs in RRC_INACTIVE with ongoing SDT procedure in the DL per cell

a)	This measurement provides the max number of active UEs (in RRC_INACTIVE state) in the DL in an NRCellDU. The measurement is calculated per PLMN ID and per QoS level  and per supported S-NSSAI.

b)	DER (n=1).

c)	This measurement is defined according to measurement "Max number of Active UEs in RRC_INACTIVE with ongoing SDT procedure in the DL per DRB per cell" (see clause 4.2.1.3a.3 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI) and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MaxRRCInactiveUeDl, or

DRB.MaxRRCInactiveUeDl_Filter, 
where filter is a combination of PLMN and QoS and SNSSAI,

where PLMN represents the PLMN ID, QoS represents the mapped 5QI, and SNSSAI represents S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

*** START OF NEXT CHANGE ***

##### 5.1.1.23.9	Mean number of Active UEs in RRC_INACTIVE with ongoing SDT procedure in the UL per cell

a)	This measurement provides the mean number of active UEs (in RRC_INACTIVE state) in the UL in an NRCellDU. The measurement is calculated per PLMN ID and per QoS and per supported S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained by aggregating the measurement "Mean number of Active UEs (in RRC_INACTIVE state) in the UL per DRB per cell" (see clause 4.2.1.3a.4 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI) and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MeanRRCInactiveUeUl, or

DRB.MeanRRCInactiveUeUl_Filter, 
where filter is a combination of PLMN and QoS and SNSSAI,

where PLMN represents the PLMN ID, QoS represents the mapped 5QI, and SNSSAI represents S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

*** START OF NEXT CHANGE ***

##### 5.1.1.23.10	Max number of Active UEs in RRC_INACTIVE with ongoing SDT procedure in the UL per cell

a)	This measurement provides the max number of active UEs (in RRC_INACTIVE state) in the UL in an NRCellDU. The measurement is calculated per PLMN ID and per QoS level, per S-NSSAI and per supported PLMN ID.

b)	DER (n=1)

c)	This measurement is defined by the measurement "Max number of Active UEs in RRC_INACTIVE with ongoing SDT procedure  in the UL per DRB per cell" (see clause 4.2.1.3a.5 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI) and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of supported S-NSSAIs.
[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MaxRRCInactiveUeUl, or

DRB.MaxRRCInactiveUeUl_Filter, 
where filter is a combination of PLMN and QoS and SNSSAI,
where PLMN represents the PLMN ID, QoS represents the mapped 5QI, and SNSSAI represents S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

*** END OF CHANGE ***