TSG-SA5 Meeting #162	S5-253475

# Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: compr ehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to draftCR Rel-19 TS 28.105 Correct inference related attributes in MLModel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MLModel IOC currently has the following inference related attributes: - aiMLInferenceName (type: AIMLInferenceName, SQ: M, cardinality: 1) - mlCapabilitiesInfoList (type: MLCapabilityInfo, SQ: M, cardinality:1..*), with MLCapabilityInfo having these attributes: - aiMLInferenceName (type: AIMLInferenceName, SQ: M, cardinality: 1) - capabilityName (type: String, SQ: O, cardinality: 1) - mlCapabilityParameters (type: AttributeValuePair, SQ: O, cardinality: *) - inferenceScope (type: AIMLInferenceName, SQ: CM, cardinality: *) As noticed, there exists redundancies between “aiMLInferenceName”, “mlCapabilitiesInfo[i].aiMLInferenceName” and “inferenceScope” attributes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove “aiMLInferenceName” and “inferenceScope” attributes from MLModel IOC, leaving “mlCapabilitiesInfoList” only. The reason is that the information of the first two attributes is already contained in the third attribute. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Redudancy information on ML model capabilities, leading to misconfiguration on producer side and confusion when discovery on consumer side. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2a.2.1.2, 7.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

#### 7.2a.2.1	MLModel

##### 7.2a.2.1.1	Definition

This IOC represents the ML model. ML model algorithm or ML model are not subject to standardization. It is name-contained by MLModelRepository.

This MLModel MOI can be created by the system (MnS producer) or pre-installed. The MnS consumer can request the system to delete the MLModel MOI.

The MLModel contains 3 types of contexts - TrainingContext, ExpectedRunTimeContext and RunTimeContext which represent status and conditions of the MLModel. These contexts are of mLContext <<dataType>>, see clauses 7.4.3 and 7.5.1 for details.

It also contains a reference named retrainingEventsMonitorRef which is a pointer to ThresholdMonitor MOI. This indicates the list of performance measurements and the corresponding thresholds that are monitored and used to identify the need for re-training by the MnS Producer. After the MLModel MOI has been instantiated, the MnS Consumer can request MnS producer to instantiate a ThresholdMonitor MOI and update the reference in the MLModel MOI that can be used by the MnS producer to decide on the re-training of the MLModel. The MnS producer can be ML Training MnS producer or AI/ML Inference MnS Producer.

The ML model includes information about its applicable type of training, which includes pre-specialised training, fine-tuning, or re-training.

##### 7.2a.2.1.2	Attributes

The MLModel IOC includes attributes inherited from Top IOC (defined in TS 28.622 [12]) and the following attributes:

Table 7.2a.2.1.2-1

| Attribute name | Support Qualifier | isReadable |  | isWritable |  | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mLModelId | M | T |  | F |  | F | T |
| mLModelVersion | M | T |  | F |  | F | T |
| expectedRunTimeContext | M | T |  | T |  | F | T |
| trainingContext | CM | T |  | F |  | F | T |
| runTimeContext | O | T |  | F |  | F | T |
| supportedPerformanceIndicators | O | T |  | F |  | F | T |
| mLCapabilitiesInfoList | M | T |  | F |  | F | T |
| mLTrainingType | M | T |  | F |  | F | T |
| Attribute related to role |  |  |  |  |  |  |  |
| retrainingEventsMonitorRef | O | T |  | T |  | F | T |
| aIMLInferenceReportRefList | O |  | T |  | F | F | T |
| usedByFunctionRefList | O |  | T |  | F | F | T |

##### 7.2a.2.1.3	Attribute constraints

Table 7.2a.1.3-1

| Name | Definition |
| --- | --- |
| trainingContext | Condition: The trainingContext represents the status and conditions related to training and should be added when training is completed. |

##### 7.2a.2.1.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

Next Change

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mLModelId | It identifies the ML model. It is unique in each MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| candidateTrainingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aIMLInferenceName | It indicates the type of inference that the ML model supports. allowedValues: see clause 7.4.10 | type: AIMLInferenceName multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.aIMLInferenceName | It indicates the type of inference that the ML model conducting inference. allowedValues: see clause 7.4.10 | type: AIMLInferenceName multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mDAType | It indicates the type of inference that the ML model for MDA supports. The detailed definition and corresponding allowed values for mDAType see TS 28.104 [2]. | type: MDAType (TS 28.104 [2]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nwdafAnalyticsType | It indicates the type of inference that the ML model for NWDAF supports. The detailed definition and corresponding allowed values for nwdafAnalyticsID see NwdafEvent in TS 29.520 [20]. | type: NwdafEvent (TS 29.520 [20]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ngRanInferenceType | It indicates the type of inference that the ML model for NG-RAN supports. The detailed definition and corresponding allowed values for ngRanInferenceType see clause 7.4a.1 | type: NgRanInferenceType multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vSExtensionType | It indicates the type of inference that is vendor's specific extension. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| usedConsumerTrainingData | It provides the address(es) where lists of the consumer-provided training data are located, which have been used for the ML model training. It may include the information about the effectiveness of training data, which indicates the consumer-provided training data is useful or not. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trainingRequestRef | It is the DN(s) of the related MLTrainingRequest MOI(s). | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trainingProcessRef | It is the DN(s) of the related MLTrainingProcess MOI(s) that produced the MLTrainingReport. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingReportRef | It is the DN of the MLTrainingReport MOI that represents the reports of the ML model training. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lastTrainingRef | It is the DN of the MLTrainingReport MOI that represents the reports for the last training of the ML model(s). | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| modelConfidenceIndication | It indicates the average confidence value (in unit of percentage) that the ML model would perform for inference on the data with the same distribution as training data. Essentially, this is a measure of degree of the convergence of the trained ML model. allowedValues: { 0..100 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingRequestSource | It identifies the entity that requested to instantiate the MLTrainingRequest MOI. This attribute is the DN of a managed entity, otherwise, it is a String. | type: <<Choice>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.requestStatus | It describes the status of a particular ML model training request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTrainingProcessId | It identifies the training process. It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| priority | It indicates the priority of the training process. The priority may be used by the ML model training to schedule the training processes. Lower value indicates a higher priority. allowedValues: { 0..65535 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| terminationConditions | It indicates the conditions to be considered by the ML training MnS producer to terminate a specific training process. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| progressStatus | It indicates the status of the process. allowedValues: N/A. | type: ProcessMonitor multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLUpdateProcess.cancelProcess | It allows the ML update MnS consumer to cancel the ML update process. Setting this attribute to "TRUE" cancels the ML update process. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLUpdateProcess.suspendProcess | It allows the ML update MnS consumer to suspend the ML update process. Setting this attribute to "TRUE" suspends the ML update process. The process can be resumed by setting this attribute to “FALSE” when it is suspended. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLModelVersion | It indicates the version number of the ML model. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceRequirements | It indicates the expected performance for a trained ML model when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| modelPerformanceTraining | It indicates the performance score of the ML model when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLTrainingProcess.progressStatus.progressStateInfo | It provides the following specialization for the “progressStateInfo“ attribute of the “ProcessMonitor“ data type for the “MLTrainingProcess.progressStatus“. When the ML model training is in progress, and the " mLTrainingProcess.progressStatus.status " is equal to "RUNNING", it provides the more detailed progress information. allowedValues for " mLTrainingProcess.progressStatus.status " = "RUNNING": - “COLLECTING_DATA” - “PREPARING_TRAINING_DATA” - “TRAINING” + DN of the MLModel being trained The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLING" are vendor specific. The allowed values for " mLTrainingProcess.progressStatus.status " = "NOT_STARTED" are vendor specific. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inferenceOutputName | It indicates the name of an inference output of an ML model. allowedValues: the name of the MDA output IEs (see 3GPP TS 28.104 [2]), name of analytics output IEs of NWDAF (see TS 23.288 [3]), RAN inference output IE name(s), and vendor's specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceMetric | It indicates the performance metric used to evaluate the performance of an ML model, e.g. "accuracy", "precision", "F1 score", etc. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceScore | It indicates the performance score (in unit of percentage) of an ML model when performing inference on a specific data set (Note). The performance metrics may be different for different kinds of ML models depending on the nature of the model. For instance, for numeric prediction, the metric may be accuracy; for classification, the metric may be a combination of precision and recall, like the "F1 score". allowedValues: { 0..100 }. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.cancelRequest | It allows the ML training MnS consumer to cancel the ML model training request. Setting this attribute to "TRUE" cancels the ML model training request. The request can be resumed by setting this attribute to "FALSE" when it is suspended. Cancellation is possible when the requestStatus is the "NOT_STARTED", " IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingRequest.suspendRequest | It allows the ML training MnS consumer to suspend the ML model training request. Setting this attribute to "TRUE" suspends the ML model training process. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.cancelProcess | It allows the ML training MnS consumer to cancel the ML model training process. Setting this attribute to “TRUE“ cancels the ML model training process. Cancellation is possible when the “mLTrainingProcess.progressStatus.status“ is not the “FINISHED“ state. Setting the attribute to “FALSE“ has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.suspendProcess | It allows the ML training MnS consumer to suspend the ML model training process. Setting this attribute to "TRUE" suspends the ML model training process. The process can be resumed by setting this attribute to “FALSE” when it is suspended. Suspension is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| inferenceEntityRef | It describes the target entities that will use the ML model for inference. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dataProviderRef | It describes the entities that have provided or should provide data needed by the ML model e.g. for training or inference | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| areNewTrainingDataUsed | It indicates whether new training data are used for the ML model training. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingDataQualityScore | It indicates numerical value that represents the dependability/quality of a given observation and measurement type. The lowest value indicates the lowest level of dependability of the data, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| decisionConfidenceScore | It is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. The lowest value indicates the lowest level of dependability of the decisions, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedRuntimeContext | This describes the context where an MLModel is expected to be applied. allowedValues: N/A | type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingContext | This specifies the context under which the MLModel has been trained. allowedValues: N/A | type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| runTimeContext | This specifies the context where the MLmodel or model is being applied. allowedValues: N/A | type: MLContext multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.mLModelRef | It identifies the DN of the MLModel requested to be trained. | type: DN multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLTrainingReport.mLModelGeneratedRef | It identifies the DN of the MLModel generated by the ML model training. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLModelRepositoryRef | It identifies the DN of the MLModelRepository. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLRepositoryId | It indicates the unique ID of the ML repository. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| modelPerformanceValidation | It indicates the performance score of the ML model when performing on the validation data. allowedValues: N/A | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dataRatioTrainingAndValidation | It indicates the ratio (in terms of quantity of data samples) of the training data and validation data used during the training and validation process. It is represented by the percentage of the validation data samples in the total training data set (including both training data samples and validation data samples). The value is an integer reflecting the rounded number of percent * 100. allowedValues: { 0 .. 100 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTestingRequest.requestStatus | It describes the status of a particular ML testing request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTestingRequest.cancelRequest | It allows the ML testing MnS consumer to cancel the ML testing request. Setting this attribute to "TRUE" cancels the ML testing request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTestingRequest.suspendRequest | It allows the ML testing MnS consumer to suspend the ML testing request. Setting this attribute to "TRUE" suspends the ML testing request. The request can be resumed by setting this attribute to “FALSE” when it is suspended. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTestingRequest.mLModelRef | It identifies the DN of the MLModel requested to be tested. | type: DN Multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| modelPerformanceTesting | It indicates the performance score of the ML model when performing on the testing data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLTestingResult | It provides the address where the testing result is provided. The detailed testing result format is vendor specific. allowedValues: N/A. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| testingRequestRef | It identifies the DN of the MLTestingRequest MOI. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedPerformanceIndicators | This parameter lists specific PerformanceIndicator(s) of an ML model. allowedValues: N/A. | type: SupportedPerfIndicator multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceIndicatorName | It indicates the identifier of the specific performance indicator. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isSupportedForTraining | It indicates whether the specific performance indicator is supported a performance metric of ML model training for the ML model. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| isSupportedForTesting | It indicates whether the specific performance indicator is supported a performance metric of ML model testing for the ML model. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLUpdateProcessRef | It is the DN of the mLUpdateProcess MOI that represents the process of updating an ML model. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateRequestRefList | It is the list of DN of the MLUpdateRequest MOI that represents an ML update request. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLUpdateReportRef | It is the DN of the MLUpdateReport MOI that represents an ML update report. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateReportingPeriod | It specifies the time duration upon which the MnS consumer expects the ML update is reported. | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availMLCapabilityReport | It represents the available ML capabilities. allowedValues: N/A. | type: AvailMLCapabilityReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| updatedMLCapability | It represents the updated ML capabilities. allowedValues: N/A. | type: AvailMLCapabilityReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availMLCapabilityReportID | It identifies the available ML capability report. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| newCapabilityVersionId | It indicates the specific version of AI/ML capabilities to be applied for the update. It is typically the one indicated by the MLCapabilityVersionID in a newCapabilityVersion | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mlCapabilityVersionId | It indicates the version of ML capabilities that is available for the update. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceGainThreshold | It defines the minimum performance gain as a percentage that shall be achieved with the capability update, i.e., the difference in the performances between the existing capabilities and the new capabilities should be at least performanceGainThreshold otherwise the new capabilities should not be applied. Allowed value: float between 0.0 and 100.0 | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectedPerformanceGains | It indicates the expected performance gain if/when the AI/ML capabilities of the respective network function are updated with/to the specific set of newly available AI/ML capabilities. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| updateTimeDeadline | It indicates the maximum as stated in the MLUpdate request that should be taken to complete the update | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLUpdateReport.mLModelRefList | It indicates the DN of MLModel instances that can be updated. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLUpdateRequest.requestStatus | It describes the status of a particular ML update request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLUpdateRequest.cancelRequest | It allows the MnS consumer to cancel the ML update request. Setting this attribute to "TRUE" cancels the ML update request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLUpdateRequest.suspendRequest | It allows the MnS consumer to suspend the ML update request. Setting this attribute to "TRUE" suspends the ML update request. The request can be resumed by setting this attribute to “FALSE” when it is suspended. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| memberMLModelRefList | It identifies the list of member ML models within an ML model coordination group. | type: DN multiplicity: 2..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| MLTrainingRequest.mLModelCoordinationGroupRef | It identifies the DN of the MLModelCoordinationGroup requested to be trained. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingReport.mLModelCoordinationGroupGeneratedRef | It identifies the DN of the MLModelCoordinationGroup generated by ML model joint training. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTestingRequest.mLModelCoordinationGroupRef | It identifies the DN of the MLModelCoordinationGroup requested to be tested. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| retrainingEventsMonitorRef | It indicates the DN of the ThresholdMonitor MOI that indicates the performance measurements and its corresponding thresholds to be used by MnS producer to initiate the re-training of the MLModel. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLModelLoadingRequest.requestStatus | It describes the status of a particular ML model loading request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLModelLoadingRequest.cancelRequest | It allows the MnS consumer to cancel the ML model loading request. Setting this attribute to "TRUE" cancels the ML model loading. Cancellation is possible when the requestStatus is the "NOT_STARTED", " IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLModelLoadingRequest.suspendRequest | It allows the MnS consumer to suspend the ML model loading request. Setting this attribute to "TRUE" suspends the ML model loading request. The request can be resumed by setting this attribute to “FALSE” when it is suspended. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLModelToLoadRef | It identifies the DN of a trained MLModel requested to be loaded to the target inference function(s). | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| policyForLoading | It provides the policy for controlling ML model loading triggered by the MnS producer. This policy contains two thresholds in the thresholdList attribute. The first threshold is related to the ML model to be loaded, and the second threshold is related to the existing ML model being used for inference. | type: AIMLManagementPolicy multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdList | It provides the list of threshold. | type: ThresholdInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLModelLoadingProcess.progressStatus.progressStateInfo | It provides the following specialization for the "progressStateInfo" attribute of the "ProcessMonitor" data type for the "MLModelLoadingProcess.progressStatus". When the ML model loading is in progress, and the " MLModelLoadingProcess.progressStatus.status " is equal to "RUNNING", it provides the more detailed progress information. allowedValues for " MLModelLoadingProcess.progressStatus.status " = "RUNNING": The allowed values for " MLModelLoadingProcess.progressStatus.status " = "CANCELLING" are vendor specific. The allowed values for " MLModelLoadingProcess.progressStatus.status " = "NOT_STARTED" are vendor specific. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLModelLoadingProcess.cancelProcess | It allows the MnS consumer to cancel the ML model loading process. Setting this attribute to "TRUE" cancels the process. Cancellation is possible when the "MLModelLoadingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLModelLoadingProcess.suspendProcess | It allows the MnS consumer to suspend the ML model loading process. Setting this attribute to "TRUE" suspends the process. The process can be resumed by setting this attribute to "FALSE" when it is suspended. Suspension is possible when the "MLModelLoadingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLModelLoadingRequestRef | It identifies the DN of the associated MLModelLoadingRequest. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLModelLoadingPolicyRef | It identifies the DN of the associated MLModelLoadingPolicyRef. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| loadedMLModelRef | It identifies the DN of the MLModel that has been loaded to the inference function. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| activationStatus | It describes the activation status. allowedValues: ACTIVATED, DEACTIVATED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AIMLManagementPolicy.managedActivationScope | It provides a list of sub scopes for which ML inference is activated as triggered by a policy on the MnS producer. For example, the sub scopes may be a list of cells or of geographical areas. The list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope if the policy evaluates to true. allowedValues: N/A | type: ManagedActivationScope multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AIMLInferenceFunction.managedActivationScope | It provides a list of sub scopes for which ML inference is activated as triggered by a policy on the MnS producer. For example, the sub scopes may be a list of cells or of geographical areas. The list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope if the policy evaluates to true. allowedValues: N/A | type: AIMLManagementPolicy multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ManagedActivationScope.dNList | It indicates the list of DN, the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | type: DN multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.timeWindow | It indicates the list of time window; the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | type: TimeWindow multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.geoPolygon | It indicates the list of GeoArea, the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | type: GeoArea multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| usedByFunctionRefList | It provides the DNs of the functions supported by the AIMLInferenceFunction. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferenceOutputId | It identifies an inference output within an AIMLinferenceReport. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferenceOutputs | It indicates the Outputs that have been derived by the AIMLInferenceFunction instance from a specific ML model. Each ML model, inferenceOutputs may be a set of values. allowedValues: N/A. | type: InferenceOutput multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferencePerformance | It indicates the performance score of the ML model during Inference. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferenceOutputTime | It indicates the time at which the inference output is generated. allowedValues: N/A | type: DateTime multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| outputResult | It indicates the result of an inference. | type: AttributeValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: Null isNullable: False |
| mLCapabilitiesInfoList | It indicates information about what an ML model can generate inference for. allowedValues: N/A. | type: MLCapabilityInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| capabilityName | It indicates the name of a capability for which an ML model can generate inference. The capability is defined by Mns producer which can be traffic analysis capability, coverage analysis capability, mobility analysis capability or vendor specific extensions. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLCapabilityParameters | It indicates a set of optional parameters that apply for an aIMLInferenceName capabilityName. allowedValues: N/A | type: AttributeValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aIMLInferenceReportRefList | It indicates a list of DN of the AIMLInferenceReport MOI that represents an AIML inference report. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| mLModelRefList | It identifies the list of MLModel DN. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| mLKnowledge | It indicates an instance of ML Knowledge available at the ML training function. | type: MmLKnowledge multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| mLKnowledgeName | It identifies the ML Knowledge. It is unique in each MnS producer. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| KnowledgeType | It identifies the type of ML Knowledge as either a Statistic, a regression or a Table of input-output value(s) Allowed values: TABLE , STATISTIC, REGRESSION | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| PredictorResponseArray | It identifies the predictor and corresponding response data for a piece of ML Knowledge. For exapme, it represents one of the following: - the input and output data for a table - the predictor and response for a statistic, - the input and output data for a regression NOTE: The nature of the data is not scope of this specification | type: pair<String, String> multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| inferenceExplanationInfo | It indicates the inference explanation information of the ML model Inference results. E.g. the critical features in the training or inference data. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| mLTrainingType | It indicates the type of ML training (e.g., initial-training, re-training, pre-specialised training, fine-tuning) requested by the consumer. allowed values: initial training, pre-specialised training, re-training, fine-tuning. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| expectedInferenceScope | It indicates the inference capabilities that the ML model is expected to support, where the inference scope contains a list of aIMLInferenceName that the ML model can be potential adapted to support. | type: AIMLInferenceName multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| distributedTrainingExpectation | It indicates distributed traning expectations provided by MnS consumer. allowedValues: N/A. | type: DistributedTrainingExpectation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| expectedTrainingTime | It indicates the expected training duration provided by MnS consumer, in unit of minites. allowedValues: Integer | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| dataSplitIndication | This is a Boolean attribute specifying whether the provided training data should be split or not. The value FALSE specify that the training data shall not be spilt. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |  |
| suggestedTrainingNodeList | It indicates a list of suggested training nodes provided by MnS consumer. allowedValues: Not applicable. | type: DN multiplicity: 0..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |  |
| trainingDataStatisticalProperties | It indicates the training data statistical properties to be considered by the MnS producer when training an ML model. | type: DataStatisticalProperties multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| uniformlyDistributedTrainingData | It indicates the need for using training data that are uniformly distributed according to the different aspects (e.g., equivalent data samples for each UE in the training data, equivalent data samples for each type of slice in the training data, equivalent data samples from each GeoArea in the training data) of the aIMLinferenceName. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |  |
| trainingDataWithOrWithoutOutliers | It indicates that the training data samples should consider or disregard data samples that are at the extreme boundaries of the value range. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |  |
| potentialImpactInfo | This datatype define the potential network impacts due to the inference output results | type: PotentialImpactInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| impactedScope | This will specify the scope of affect, the inference output may have on the network including entities performing the recommended actions in the inference output and entities impacted due to implementation of the recommended actions The choice attribuite dNList defines Identifier of the network functions that may be affected by the output result of the inference function. The choice attribute timeWindow defines a time duration indicating that the related network function(s) may be affected during this time duration by the inference output result. The choice attribute geoPolygon defines a Geographical location indicating that the network function(s) in that location may be affected by the inference output result. | type: ManagedActivationScope multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| impactedPM | This will identify the potential performance metrics that may be degraded/improved due to the implementation of recommendations provided as part of inference output. | type: ImpactedPM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| pMIdentifier | This indicates the performance measurement or the KPI that may be impacted by the ML Model. This will be the name of PM and KPI as defined in 3GPP TS 28.552 and 28.554 respectively (e.g. for Managing NG-RAN AI/ML-based distributed Load Balancing function, the PM can be measurements related to MLB, UE throughput and Radio resource utilization etc). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| supportedLearningTechnology | It identifies the learning technologies including Reinforcement Learning, Federated Learning and Distributed training which supported by the ML training function. | type: SupportedLearningTechnology multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| rLRequirement | It identifies the expected performanc and performed scope for the ML model training when Reinforcement Learning is supported. | type: RLRequirement multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| learningTechnologyName | It indicates a list of learning technology names used to represent the learning technics supported by the ML training function. allowedValues: RL, FL, DL where RL indicates of Reinforcement Learning, FL indicates of Federated Learning and DL indicates of Distributed training. | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| supportedEnvironment | It indicates the supported RL environments. When the ML training MnS producer supports RL, this attribute is included in the SupportedLearningTechnology datatype, which indicates the supported environment of the ML training function for ML model training. allowedValues: SIMULATION ENVIONMENTS, REAL NETWORK ENVIONMENTS. | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| supportedInferenceNameList | It indicates a list of inference name that the learning technologies can be applied. allowedValues: see clause 7.4.10 | type: AIMLInferenceName multiplicity: 1..* isOrdered: False isUnique: N/A defaultValue: None isNullable: False |  |
| rLEnvironmentType | It indicates the simulated environment or real network where the ML model should be traind. allowedValues: SIMULATION ENVIONMENTS, REAL NETWORK ENVIONMENTS | type: Enum multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| rLEnvironmentScope | It indicates the specific environment scope for the entities that the RL process should be performed, i.e, where the RL agent is located. | type: EnvironmentScope multiplicity: 1..* isOrdered: False isUnique: N/A defaultValue: None isNullable: False |  |
| rLImpactedScope | It indicates the specific environment scope for the entities that may be impacted by the RL process, i.e., scope may be impacted by actions of the RL agent. | type: EnvironmentScope multiplicity: 1..* isOrdered: False isUnique: N/A defaultValue: None isNullable: False |  |
| rLPerformanceRequirements | It indicates a list of thresholds for the network performance requirements, when the RL training process(es) is performed. | type: ThresholdInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| clusteringInfo | It containes information that indicates the clustering criteria for the ML Models that can be grouped together for training | type: ClusteringCriteria multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| ClusteringCriteria.performanceMetric | This defines clustering criteria based on the performance metric for which the ML model is mainly evaluated. That is, the models, which intend to achieve same performance characteristic (e.g. accuracy, precision, F1 score etc) can be clustered together for training. It indicates the performance metric used to evaluate the performance of an ML model allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| taskType | This defines grouping criteria based on the task the ML model is trained for. For example, this can be aIMLInferenceName or capabilityName as defined in 3GPP TS 28.105. Note: Whether the taskType can be aIMLInferenceName here is FFS. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| allowedClusterTrainingTime | This defines the combined time limit within which the training of ML models cluster shall be completed. A cluster of ML models takes more time to train together as compared to time taken for training an individual ML model. The criteria allows accommodating only those ML models whose training time does not exceed the set combined time limit | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |
| preferredModelDiversity | This defines the consumer preferred model diversity types that is to be considered for models clustering. For example, decision trees, neural networks, linear regression and like so | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| clusteringInfo | It containes information that indicates the clustering criteria for the ML Models that can be grouped together for training | type: ClusteringCriteria multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |  |
| NOTE: When the performanceScore is to indicate the performance score for ML model training, the data set is the training data set. When the performanceScore is to indicate the performance score for ML validation, the data set is the validation data set. When the performanceScore is to indicate the performance score for ML model testing, the data set is the testing data set. |  |  |

End of Changes