3GPP TSG-SA5 Meeting #162 	S5-253479

Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

# Source:	Rakuten Mobile

# Title:	Usage of X2HOBlackList in NR-NRM

# Document for:	Discussion and Endorsement

Agenda Item:	6.1

# 1	Decision/action requested

The group is asked to discuss and endorse to this proposal.

# 2	References

[1]	3GPP TS 28.541 "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[2]	3GPP TS 37.340 "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Overall Description; Stage-2"

[3]	3GPP TS 38.300 "NR; NR and NG-RAN Overall description; Stage-2"

# 3	Rationale

## 3.1	Problem statement

In TS 28.541[1], the usage of x2HOBlockList is not clear.

- The attribute x2HOBlockList is defined as part of GNBCUCPFunction IOC in clause 4.3.2.2.

- Attribute constraints for x2HOBlockList are defined in clause 4.3.2.3. As described in this clause, this attribute is conditional for the case where the UE is setup with Mult-Radio Dual Connectivity with EPC. This clause also references clause 4.1.2 of TS 37.340 which describes the use case where E-UTRAN supports dual connectivity and eNB acts as the Master Node and en-gNB is the Secondary Node.

- As per 38.300 [3], clause 9.3, when UE is connected to EUTRA and EPC, the handover is via N26 interface between MME and AMF. No X2 involvement is prescribed for this use case.

## 3.2	Analysis

There is no case where gNB uses X2 interface for handover to eNB. Even if the MR-DC is considered and UE is connected to EPC, there is no X2 HO defined between en-gNB and eNB. Handovers between gNB and eNB always use NG/S1 interfaces and the handover happens through the core network.

## 3.3	History

The following table describes past TDocs which track the progression of this attribute in NR-NRM.

| TDoc | Our remarks | TDoc Final Status | TS 28.541 version | SA5 meeting number |
| --- | --- | --- | --- | --- |
| S5-197837.zip | Introduction of blacklists (X2XnHOBlackList) | Approved | Updated in 16.2.0 | TSGS5_128 |
| S5-202084.zip | Discussion paper explaining the split to separate lists for X2 and XN handovers block lists | Endorsed | N/A | TSGS5_130e |
| S5-202303.zip | New CR with split parameter for HO block lists | Approved | Updated in 16.5.0 | TSGS5_130e |

# 4	Detailed proposal

This DP proposes to endorse following proposals:

- Update TS 28.541[2] and remove the attribute X2HOBlockList in clauses 4.3.2.2, 4.3.2.3, 4.4.1.

- Update stage 3 _3gpp-nr-nrm-gnbcucpfunction.yang to remove x2HOBlackList

- Update stage 3 TS28541_NrNrm.yaml to remove x2HOBlockList