3GPP TSG-SA5 Meeting #162	S5-253702

Goteborg, Sweden, 25 - 29 August 2025

Source:	China Unicom

Title:	Rel-20 pCR TR 28.886 Add new use case on radio resource optimization based on per SSB usage

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 28.886

Version:	0.0.0

Work Item:	FS_eMDAS_Ph4

Comments

As agreed in S5-253072, following WT is supposed to be investigated:

WT-3.1 Study possible new MDAS capabilities to provide analytics on the status/performance/usage of the managed network and to recommend optimization or repair actions.

Current use cases in TS 28.104 do not have radio resource optimization based on per SSB usage. This use case is proposed to provide intelligently adjust the SSB beam configuration to improve the efficiency of radio resource.

Proposed Changes

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 28.104: "Management and orchestration;Management Data Analytics (MDA)".

* * * Next Change * * * *

# 4	Use cases

## 4.y	Use case#y: Radio resource optimization based on per SSB usage

### 4.y.1	Description

NR adopts massive MIMO technology, the base station uses directional beams in the initial access common channel and RRC_CONNECTED state. A common signal/channel for initial access of the UE is transmitted in a synchronization signal block (SSB). SSB is designed into different beam patterns to correlate the number of SSB beams and the different directions of SSB beams.

Figure 1 is a single sideband beam that meets a cell accessed by users, Figure 2 is a time division SSB beam that meets the access requirements of users according to multi-user distribution. Time division means that the flexible SSB beam is modified and deleted in the target time period. Figure 3 is calculating the beam-level user number, the beam-level user occupancy time in the service cell, and the user distribution is estimated. Figure 4 calculates the interference of neighboring cells received by each beam to estimate the potential neighboring cell users.

SSB beam meets the user's access coverage requirements. In view of the demand scenario with a large number of users and concentrated packet traffic, it is difficult to accurately modify or disable the SSB beam based on traffic statistics; the beam-level user number and beam-level user occupancy time need to be considered. For example, as shown in the Figure, if the area where the expected number of users is concentrated can be covered by N beams, then X gain beams can be disabled. This energy-saving technology depends on the accuracy of where the expected business requirements come from geographically, so it can be associated with the SSB beam coverage area. In this use case, the beam-level user number and beam-level user occupancy time are the main criteria for defining user number management. It is hoped that MDA analysis can be used to obtain the prediction of per SSB usage at the granularity level of geographical coordinates. Then, this information can be used to intelligently adjust the SSB beam configuration to improve the efficiency of radio resource.

| Figure 1: Illustration of SSB beams covering full cell coverage area. | Figure 2: Illustration of reduced number of SSB beams covering only hotspot area time division. |
| --- | --- |
| Figure 3: Prediction of user number and user occupancy time per beam | Figure 4: Prediction of potential user number per beam |

### 4.y.2	Potential requirements

REQ-RRO-FUN-1: MDA capability for radio resource optimization shall include providing the predicted SSB-level usage for the area to recommend the SSB beam configuration.

* * * End of Changes * * * *