3GPP TSG-SA5 Meeting #162	S5-253775

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.201 | CR | 0027 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.201 Correction to information for MOCN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_MOCN_NetShare |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It should be clarified that the PLMN Identifier in the Network Sharing Charging Information is POP PLMN Identifier not MOP, because the MOP PLMN Identifier has been captured in the charging data request. The refences for network sharing in charging data request are wrong. There is no need for a separate QoS for network sharing since this can be handled by the network slicing. The binding for network slice performance and analytics converged charging can be used for network sharing charging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update PLMN Identifier to POP PLMN Identifier 2. Correcting the network sharing references and adding the network sharing to the binding description. 3. Removing the editor’s note on QoS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Network Sharing Charging would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.1.1.2, 6.2.1.4, 6.2.1.5, 6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is revison of S5-253572 |  |  |  |  |  |  |  |  |

First Change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

CEF	Charging Enablement Function

CGF	Charging Gateway Function

CHF	Charging Function

MnS	Management Service

NSPA	Network Slice Performance and Analytics

NWDAF	Network Data Analytics Function

PEC	Post Event Charging

POP	Participating Operator

Next Change

#### 6.1.1.2	Charging Data Request message

Table 6.1.1.2-1 illustrates the basic structure of a Charging Data Request message from the CEF as used for performance and analytics based charging.

Table 6.1.1.2-1: Charging Data Request message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Tenant Identifier | OM | This field if present is the identifier of subscriber of network slice and may in the case of 5G MOCN network sharing represent the participating operator, e.g. PLMN Id. |
| NF Consumer Identification | M | Described in TS 32.290 [57]. |
| NF Functionality | M | Described in TS 32.290 [57]. |
| NF Name | OC | Described in TS 32.290 [57]. |
| NF Address | OC | Described in TS 32.290 [57]. |
| NF PLMN ID | OC | Described in TS 32.290 [57]. |
| Invocation Timestamp | M | Described in TS 32.290 [57]. |
| Invocation Sequence Number | M | Described in TS 32.290 [57]. |
| One-time Event | OC | Described in TS 32.290 [57]. |
| One-time Event Type | OC | Described in TS 32.290 [57]. |
| Service Specification Information | OC | Described in TS 32.290 [57]. |
| Multiple Unit Usage | OC | Described in TS 32.290 [57], with the exception that quota management is not applicable. |
| Rating Group | M | Described in TS 32.290 [57] |
| Used Unit Container | OC | Described in TS 32.290 [57] |
| Local Sequence Number | OM | Described in TS 32.290 [57] |
| NSPA Container Information | OC | This field holds the network slice performance and analytics container specific information described in clause 6.2.1.3. |
| Network Sharing Container Information | OC | This field holds 5G MOCN network sharing container specific information described in clause 6.2.1.5. |
| NSPA Charging Information | OC | This field holds the network slice information, which is reported to the CHF described in clause 6.2.1.2. |
| Network Sharing Charging Information | OC | This field holds 5G MOCN network sharing specific information described in clause 6.2.1.4. |

Next Change

#### 6.2.1.4	Definition of Network Sharing Charging Information

Specific charging information used for 5G MOCN network sharing charging is provided within the Network Sharing Charging Information.

The detailed structure of the Network Sharing Charging Information can be found in table 6.2.1.4.1.

Table 6.2.1.4.1: Structure of Network Sharing Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| PLMN Identifier | M | This field holds POP PLMN that the performance information is related to. |
| Single NSSAI | OC | This field holds network slice information the performance analytics information is related to. |

Next Change

#### 6.2.1.5	Definition of Network Sharing Container Information

Specific charging information used for 5G MOCN network sharing charging is provided within the Network Sharing Container Information.

The detailed structure of the Network Sharing Container Information can be found in table 6.2.1.5.1.

Table 6.2.1.5.1: Structure of Network Sharing Container Information

| Information Element | Category | Description |
| --- | --- | --- |
| Downlink Data Volume | OC | This field holds downlink cell PDCP SDU Data Volume as described in clause 5.1.3.6.2 of TS 28.552 [270]. |
| Uplink Data Volume | OC | This field holds uplink cell PDCP SDU Data Volume as described in clause 5.1.3.6.2 of TS 28.552 [270]. |
| Number of PDU Sessions requested | OC | This field holds the number of PDU sessions requested by the gNB as described in clause 5.1.1.5.1 of TS 28.552 [270]. |
| Number of PDU Sessions successful | OC | This field holds the number of PDU sessions successfully setup by the gNB from AMF as described in clause 5.1.1.5.2 of TS 28.552 [270]. |

Next Change

## 6.3	Bindings for network slice performance and analytics converged charging

This mapping between the Information Elements, resource attributes and CHF CDR parameters for network slice performance and analytics or network sharing converged charging is described in clause 7 of TS 32.291 [51].

End of Change