3GPP TSG-SA5 Meeting #162	S5-253957

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025             revision of S5-253224

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0551 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.623 YANG stage-3 corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-07-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | YANG SS is not matching the approved stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update YANG code to match existing stage 2. Clean up of YANG additional checks warning. Update Forge reference as dictated by "Working Methods". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and Stage 3 mismatch; interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Forge |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1790 at commit c1b44d8caf03caab88e46c614cc9703913d274a4 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]	3GPP TS 28.622: “Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

[5]	Void

[6]	Void

[7]	Void

[8]	Void

[9]	Void.

[10]	Void

[11]	Void

[12]	Void

[13]	Void

[14]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[15]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[16]	IETF RFC 8528: "YANG Schema Mount".

[17]	Management and Orchestration APIs Stage 3 Repository https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel19_SA109/

[18]	RFC 8525: "YANG Library"

[19]	RFC 6022: "YANG Module for NETCONF Monitoring"

[20]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[21]	3GPP TS 32.161: "Management and orchestration; JSON expressions (Jex)".

[22]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[23]	IETF RFC 7950: “The YANG 1.1 Data Modeling Language”

[24]	IETF RFC 6991: “Common YANG Data Types”

Next change

*** START OF CHANGE 25 ***

*** yang-models/_3gpp-common-ep-rp.yang ***

<CODE BEGINS>

module _3gpp-common-ep-rp {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-ep-rp";

prefix "eprp3gpp";

import _3gpp-common-yang-types { prefix types3gpp ; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-measurements { prefix meas3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Common/basic class/grouping to be inherited/reused.

This IOC represents an end point of a link used across a reference

point between two network entities.

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference

"3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)

3GPP TS 28.620

Umbrella Information Model (UIM)";

revision 2023-09-18 { reference CR-0271 ; }

revision 2020-06-08 { reference "CR-0092"; }

revision 2019-06-17 {

description "Initial revision";

}

grouping EP_RPGrp {

description "Abstract class, represents an end point of a link used

across a reference point between two network entities.

For naming the subclasses of EP_RP, the following rules shall apply:

-  The name of the subclassed IOC shall have the form 'EP_<rp>',

where <rp> is a string that represents the name of the reference point.

Thus, two valid examples of EP_RP subclassed IOC names would be:

EP_S1 and EP_X2.";

leaf userLabel {

type string;

description "A user-friendly (and user assignable) name of this object.";

}

leaf farEndEntity {

config false;

type types3gpp:DistinguishedName;

}

}

grouping EP_Common {

uses EP_RPGrp;

uses meas3gpp:SupportedPerfMetricGroupGrp;

list localAddress {

description "Local IP address and VLAN ID.";

key "ipAddress vlanId";

min-elements 1;

max-elements 1;

uses types3gpp:AddressWithVlan;

}

leaf remoteAddress {

description "Remote IP address.";

mandatory true;

type inet:ip-address;

}

}

}

<CODE ENDS>

*** END OF CHANGE 25 ***

*** START OF CHANGE 26 ***

*** yang-models/_3gpp-common-externaldata.yang ***

<CODE BEGINS>

module _3gpp-common-externaldata {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-externaldata";

prefix "exdata3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-inet-types { prefix inet; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the external data.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623 Generic Network Resource Model (NRM)";

revision 2025-07-19 { reference CR-0551; }

grouping ExternalDataScopeGrp {

list geoAreas {

description "It describes the concrete geographical area(s)";

key idx;

leaf idx { type uint32; }

uses types3gpp:GeoAreaGrp;

}

leaf-list objectInstancesIncluded {

type types3gpp:DistinguishedName;

description "List of managed object instances to which the described

data are related. Each object instance is identified by its DN.";

}

leaf-list objectInstancesExcluded {

type types3gpp:DistinguishedName;

description "List of managed object instances which are not considered

in relation to the described data. Each object instance is identified

by its DN.";

}

}

grouping ExternalDataTypeGrp {

description "Represents the IOC ExternalDataType";

leaf externalDataType {

type string;

mandatory true;

description "Type of external management data as defined by the

implementation.

Examples: 'Electronic Map', 'Camara Data', 'UE path', 'Camera Photo',

'Event Schedule”";

}

leaf-list mediaLocation {

type inet:uri;

description "URI of the media which includes the transfer protocol.

Examples:

sftp://companyA.com/datastore/fileName.xml

https://companyA.com/ManagedElement=1/Files=1/File=1";

}

leaf externalDataTypeSchema {

type inet:uri;

mandatory true;

description "URI of the schema to parse a type of external management data.

The detailed schema definition for the different types of external

management data is out of scope of this specification.";

}

list externalDataScope {

description "It describes the concrete scope which the external management

data is applicable.";

key idx;

leaf idx { type uint32; }

uses ExternalDataScopeGrp;

}

}

augment /subnet3gpp:SubNetwork {

list ExternalDataType {

description "This IOC specifies a type of external management data and

the associated meta data.

External management data is data which enrich 3GPP specified management

data and management data specified based on 3GPP defined management data

definition templates and frameworks. External management data can be

produced by data sources of different nature (e.g. sensors) with different

formats. Details see clause 6.4.1 in TS 28.537.

MnS producer may use this IOC to describe external management data.

For example, MnS producer publishes all external management data which

are available.

MnS consumer may use this IOC to configure, discover or request external

management data. For example, MnS consumer discovers and requests certain

types of external management data.

The ExternalDataType IOC can be name-contained by SubNetwork.

Attribute externalDataType defines the type of external management data.

Attribute mediaLocation indicates the address from which the described

external management data can be retrieved.

The value of mediaLocation, if present, can provide one or several

directories or the address where the described external management data

can be retrieved. The different directories can be used to group the

external management data, e.g., one directory per geographical area or

per time period. For example, the value of the mediaLocation attribute

can be given by

sftp://companyA.com/datastore/weatherforecasts/

In this case a potential file with name 'weatherforecastMunicht0.xml'

needs to be retrieved using 'sftp' from

sftp://companyA.com/datastore/weatherforecasts/weatherforecastMunicht0.xml.

Attribute externalDataTypeSchema indicates the URI where the MnS consumer

can get the schema to parse the external management data.

Attribute externalDataScope is used to describe the concrete scope (e.g.,

geographical areas) which the external management data is applicable.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalDataTypeGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 26 ***

*** START OF CHANGE 27 ***

*** yang-models/_3gpp-common-managed-function.yang ***

<CODE BEGINS>

module _3gpp-common-managed-function {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-common-managed-function;

prefix mf3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-measurements { prefix meas3gpp; }

import _3gpp-common-trace { prefix trace3gpp; }

import _3gpp-stm-nrm-stmfunction { prefix stm3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "The module defines a base class/grouping for major 3GPP

functions.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference

"3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)

3GPP TS 28.620

Umbrella Information Model (UIM)";

revision 2025-08-13 { reference CR-0551; }

revision 2025-04-19 { reference "CR-0541"; }

revision 2024-11-19 { reference "Add STM"; }

revision 2024-02-24 { reference CR-0346; }

revision 2023-09-18 { reference CR-0271; }

revision 2023-04-26 { reference CR-0250; }

revision 2023-02-14 { reference "CR-0234"; }

revision 2022-11-02 { reference "CR-0753"; }

revision 2022-10-24 { reference CR-0196;   }

revision 2022-01-07 { reference "CR-0146"; }

revision 2021-01-25 { reference "CR-0122"; }

revision 2020-09-30 { reference "CR-bbbb"; }

revision 2020-08-06 { reference "CR-0102"; }

revision 2020-08-03 { reference "CR-0095"; }

revision 2020-06-23 { reference "CR-085"; }

revision 2020-06-08 { reference "CR-0092"; }

revision 2019-11-21 { reference "S5-197275, S5-197735"; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-18 { reference  "Initial revision"; }

feature MeasurementsUnderManagedFunction {

description "The MeasurementSubtree shall be contained under ManageElement";

}

feature TraceUnderManagedFunction {

description "The TraceSubtree shall be contained under ManagedFunction";

}

feature StmCtrlUnderManagedFunction {

description "The StmCtrlSubtree shall be contained under ManagedFunction";

}

grouping OperationGrp {

description "This data type represents an operation.";

reference "3gpp TS 28.622";

leaf name {

type string;

mandatory true;

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

leaf-list allowedNFTypes {

type string;

min-elements 1;

description "The type of the managed NF service instance

The specifc values allowed are described in TS 23.501";

}

leaf operationSemantics {

type enumeration {

enum REQUEST_RESPONSE;

enum SUBSCRIBE_NOTIFY;

}

config false;

mandatory true;

yext3gpp:inVariant;

description "Semantics type of the operation.";

reference "3GPP TS 23.502";

}

}

grouping ManagedNFServiceGrp {

description "A ManagedNFService represents a Network Function (NF) service.";

reference "Clause 7 of 3GPP TS 23.501.";

leaf userLabel {

type string;

description "A user-friendly (and user assignable) name of this object.";

}

leaf nFServiceType {

config false;

mandatory true;

type string;

description "The type of the managed NF service instance

The specifc values allowed are described in clause 7.2 of TS 23.501";

yext3gpp:notNotifyable;

yext3gpp:inVariant;

}

list sAP {

key "host port";

min-elements 1;

max-elements 1;

description "The service access point of the managed NF service instance";

uses types3gpp:SAPGrp;

}

list operations {

key name;

min-elements 1;

uses OperationGrp ;

description "Set of operations supported by the managed NF

service instance";

}

leaf administrativeState  {

type types3gpp:BasicAdministrativeState;

default LOCKED;

description "Permission to use or prohibition against using the instance";

}

leaf operationalState  {

type types3gpp:OperationalState;

config false;

mandatory true;

description "Describes whether the resource is installed and working";

}

leaf usageState {

type types3gpp:usageState ;

config false;

mandatory true;

description "Describes whether the resource is actively in use at a

specific instant, and if so, whether or not it has spare

capacity for additional users.";

}

leaf registrationState {

type enumeration {

enum REGISTERED;

enum DEREGISTERED;

}

config false;

}

}

grouping Function_Grp {

description "A base grouping for 3GPP functions.";

leaf userLabel {

type string;

description "A user-friendly (and user assignable) name of this object.";

}

}

grouping VnfParametersGrp {

description "This <<dataType>> represents the parameter set of the VNF

instance(s) corresponding to an NE.";

leaf vnfInstanceId {

type string ;

description "VNF instance identifier";

reference "ETSI GS NFV-IFA 008 v2.1.1:

Network Functions Virtualisation (NFV); Management and Orchestration;

Ve-Vnfm reference point - Interface and Information Model Specification

section 9.4.2

ETSI GS NFV-IFA 015 v2.1.2: Network Functions Virtualisation (NFV);

Management and Orchestration; Report on NFV Information Model

section B2.4.2.1.2.3";

}

leaf vnfdId {

type string ;

description "Identifier of the VNFD on which the VNF instance is based.

The absence of the leaf or a string length of zero for vnfInstanceId

means the VNF instance(s) does not exist (e.g. has not been

instantiated yet, has already been terminated).";

reference "ETSI GS NFV-IFA 008 v2.1.1:

Network Functions Virtualisation (NFV); Management and Orchestration;

Ve-Vnfm reference point - Interface and Information Model Specification

section 9.4.2";

}

leaf flavourId {

type string ;

description "Identifier of the VNF Deployment Flavour applied to this

VNF instance.";

reference "ETSI GS NFV-IFA 008 v2.1.1:

Network Functions Virtualisation (NFV) Management and Orchestration";

}

leaf autoScalable {

type boolean ;

description "Indicator of whether the auto-scaling of this

VNF instance is enabled or disabled.";

}

}

grouping PeeParametersGrp {

description "This <<dataType>> represents the parameter list for the

control and monitoring of power, energy and environmental parameters

of ManagedFunction instance(s).";

leaf idx { type uint32; }

leaf siteIdentification {

type string;

mandatory true;

description "The identification of the site where the

ManagedFunction resides.";

}

leaf siteLatitude {

type decimal64 {

fraction-digits 4;

range "-90.0000..+90.0000";

}

description "The latitude of the site where the ManagedFunction

instance resides, based on World Geodetic System (1984 version)

global reference frame (WGS 84). Positive values correspond to

the northern hemisphere. This attribute is optional

for BTSFunction, RNCFunction, GNBDUFunction and

NRSectorCarrier instance(s).";

}

leaf siteLongitude {

type decimal64 {

fraction-digits 4;

range "-180.0000..+180.0000";

}

description "The longitude of the site where the ManagedFunction

instance resides, based on World Geodetic System (1984 version)

global reference frame (WGS 84). Positive values correspond to

degrees east of 0 degrees longitude. This attribute is optional

for BTSFunction, RNCFunction, GNBDUFunction and

NRSectorCarrier

instance(s).";

}

leaf siteAltitude {

type decimal64 {

fraction-digits 4;

}

description "The altitude of the site where the ManagedFunction

instance resides, in the unit of meter. This attribute is

optional for BTSFunction, RNCFunction, GNBDUFunction and

NRSectorCarrier instance(s).";

}

leaf siteDescription {

type string;

mandatory true;

description "An operator defined description of the site where

the ManagedFunction instance resides.";

}

leaf equipmentType {

type string;

mandatory true;

description "The type of equipment where the managedFunction

instance resides.";

reference "clause 4.4.1 of ETSI ES 202 336-12";

}

leaf environmentType {

type string;

mandatory true;

description "The type of environment where the managedFunction

instance resides.";

reference "clause 4.4.1 of ETSI ES 202 336-12";

}

leaf powerInterface {

type string;

mandatory true;

description "The type of power.";

reference "clause 4.4.1 of ETSI ES 202 336-12";

}

}

grouping ManagedFunctionGrp {

description "Abstract root class to be inherited/reused by classes

representing 3GPP functions.

Anywhere this grouping is used by classes inheriting from ManagedFunction

the list representing the inheriting class needs to include all

contained classes of ManagedFunction too. Contained classes are

either

- augmented into the Function class or

- shall be included in the list representing the inheriting class

using the grouping  ManagedFunctionContainedClasses:

1) EP_RP solved using augment

2) uses mf3gpp:ManagedFunctionContainedClasses;

";

uses Function_Grp;

list vnfParametersList {

key "vnfInstanceId";

uses VnfParametersGrp;

description "This attribute contains the parameter set of the VNF

instance(s) corresponding to an NE. The presence of this attribute

indicates that the ManagedFunction represented by the MOI is a

virtualized function ";

}

list peeParameterslist{

key "idx";

uses PeeParametersGrp;

description "This attribute contains the parameter list for the control

and monitoring of power, energy and environmental parameters of

ManagedFunction instance(s). ";

}

leaf priorityLabel {

type uint32;

}

uses meas3gpp:SupportedPerfMetricGroupGrp;

leaf-list supportedTraceMetrics {

type string;

config false;

description "List of trace metrics. When this attribute is contained in

a managed object it defines the trace metrics supported for this

object and all descendant objects.

Trace metrics include trace messages, MDT measurements

(Immediate MDT, Logged MDT, Logged MBSFN MDT), RLF and RCEF reports,

see TS 32.422. Trace metrics are identified with their metric

identifier. The metric identifier is constructed as defined

in clause 10 of TS 32.422.";

}

}

grouping ManagedFunctionContainedClasses {

description "A grouping used to containe classes (lists) contained by

the abstract IOC ManagedFunction";

list ManagedNFService {

description "Represents a Network Function (NF)";

reference "3GPP TS 23.501";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ManagedNFServiceGrp;

}

}

uses meas3gpp:MeasurementSubtree {

if-feature MeasurementsUnderManagedFunction ;

}

uses trace3gpp:TraceSubtree {

if-feature TraceUnderManagedFunction ;

}

uses stm3gpp:StmCtrlSubTree {

if-feature StmCtrlUnderManagedFunction ;

}

}

}

<CODE ENDS>

*** END OF CHANGE 27 ***

*** START OF CHANGE 28 ***

*** yang-models/_3gpp-common-managementdatacollection.yang ***

<CODE BEGINS>

module _3gpp-common-managementdatacollection {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-managementdatacollection";

prefix "mgtdatcol3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types {prefix types3gpp; }

import _3gpp-common-yang-extensions {prefix yext3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Handling management data collection

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2025-07-19 { reference CR-0551; }

revision 2025-02-07 { reference "CR-0461 CR-0505"; }

revision 2024-04-12 { reference CR-0346; }

revision 2023-11-18 { reference CR-0299 ; }

revision 2023-11-06 { reference "CR-0305"; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2023-02-14 { reference "CR-0234"; }

revision 2022-11-04 { reference "CR-0194"; }

grouping ManagementDataGrp {

description "This <<choice>> defines the management data which is

requested. It is a choice between

-   a list of data categories (attribute mgtDataCategory) This may include

'COVERAGE', 'CAPACITY', 'MOBILITY', 'ENERGY_EFFICIENCY', 'ACCESSIBILITY'

etc. The mapping of exact measurement with the requested category will be

done at the producer and is implementation specific.

-   a list of management data identified with their name (attribute

'mgtDataName'). The management data name presents a specific single

measurement (e.g. by selecting 'RRU.PrbTotDl', see TS 28.552 or

'immediateMdt.nr.m1', see TS 32.422) or a set of measurements (e.g.

measurement families such as RRU (radio resource utilization) or

MM (mobility management) in case of PM, see TS 28.552 [20], or group of

measurements such as 'immediateMdt.nr' in case of MDT, see TS 32.422).";

choice managementData {

case mgtDataCategory {

leaf-list category  {

type mgtDataCategoryType;

min-elements 1;

max-elements 5; // The ENUM contains 5 possible values

description "This attributes defines the type of management data that

are requested.

Allowed values for data category are COVERAGE, CAPACITY,

ENERGY_EFFICIENCY, MOBILITY, ACCESSIBILITY. The data categories

will map to certain measurement families defined in TS 28.552, see

below. In addition to the below mappings, MnS producer may map the

provided categories to any additional proprietary management data,

as appropriate.

- The COVERAGE category will map to measurement families of MR

(measurements related to Measurement Report) and L1M (measurements

related to Layer 1 Measurement).

- The CAPACITY category will map to measurement family RRU

(measurements related to Radio Resource Utilization).

- The ENERGY_EFFICIENCY category will map to measurement family PEE

(measurements related to Power, Energy and Environment).

- The MOBILITY category will map to measurement family MM

(measurements related to Mobility Management).

- The ACCESSIBILITY category will map to measurement family CE

(measurements related to Connection Establishment).";

}

}

case mgtDataName {

leaf-list name {

type string;

min-elements 1;

description "List of management data identified by name.

The list may include metrics or set of metrics defined

in TS 28.552, TS 28.554 and TS 32.422.

The metrics are identified with their names/identifiers.

For performance measurements defined in TS 28.552 the name is

constructed as the bullet e) of the measurement definition.

For KPIs defined in TS 28.554 the name is defined according to the

KPI definitions template as the component designated with a).

For trace metrics (including trace messages, MDT measurements

(Immediate MDT, Logged MDT, Logged MBSFN MDT), RRC, RLF and RCEF

reports) defined in TS 32.422, the name (metric identifier) is

defined in clause 10 of TS 32.422.

For non-3GPP specified managment data the name is defined

elsewhere.";

}

}

mandatory true;

}

}

grouping GeoAreaToCellMappingGrp {

description "Represents the GeoAreaToCellMapping data type.

The mapping of the geographical area to cells is performed at

instantiation of the IOC.";

list geoArea {

description "It specifies the geographical area using the cordinates of

the corners of a convex polygon.";

key idx;

max-elements 1;

leaf idx {

type string;

}

uses types3gpp:GeoAreaGrp;

}

leaf associationThreshold {

type uint8 {

range 1..100;

}

yext3gpp:inVariant;

description "It specifies the threshold of coverage area in

percentage whether a cell belongs to the geographical area or not.

If this attribute is absent, the location of the base station

antenna determines whether a cell belongs to the geographical

area or not.";

}

}

grouping AreaOfInterestGrp {

description "It specifies a location(s) from where the management data

shall be collected.";

list geoAreaToCellMapping {

description "Contains a geographical area and an association threshold.

The geo-area is defined as a convex polygon using the attribute

'geoArea'.

The MnS producer shall map the geographical area to cells. There are

two evaluation criteria whether a cell belongs to a geographical area

or not. If attribute 'associationThreshold' is absent, the location

of the base station antenna determines the belonging. If attribute

'associationThreshold' is configured, the coverage area is considered.

The attribute 'associationThreshold' determines the lower boundary of

the coverage ratio. For example, if the 'associationThreshold' is

configured to 60%, a cell shall be considered as included in the

geographical area if at least 60% of the coverage area of that

cell overlaps with the specified geographical area.

The mapping of the geographical area to cells is performed at

instantiation of the IOC.";

key idx;

leaf idx { type string; }

uses GeoAreaToCellMappingGrp;

}

list taiList {

description "Tracking Area Identity list";

key idx;

min-elements 1;

max-elements 8;

leaf idx { type string; }

uses types3gpp:TaiGrp;

}

leaf-list nrCellIdList {

type string;

description "List of NR cells identified by NG-RAN CGI";

}

leaf-list eutraCellIdList {

type string;

description "List of E-UTRAN cells identified by E-UTRAN-CGI";

}

leaf-list utraCellIdList {

type string;

description "List of UTRAN cells identified by UTRAN CGI";

}

}

grouping NodeFilterGrp {

list areaOfInterest {

key idx;

leaf idx {

type string;

}

uses AreaOfInterestGrp;

description "It specifies a location(s) from where the management data

shall be collected. It is defined in terms of TAI(s).";

}

leaf networkDomain {

type enumeration {

enum CN;

enum RAN;

}

// mandatory false

description "It specifies the network domain of the target node. This

will also result in collecting appropriate management data from the

nodes belonging to the specified domain.";

}

leaf cpUpType {

type enumeration {

enum CP;

enum UP;

}

// mandatory false

description "It specifies the traffic type of the target node. This will

also result in collecting appropriate management data from the nodes

handling the specified traffic (e.g AMF for CP and UPF for UP).";

}

leaf sst {

type uint8;  // TS 28.003 clause 28.4.

description "It specifies the slice service type (SST) of which the slice

subnet should be targeted. Please refer to 3GPP TS 23.501: 'System

Architecture for the 5G System'";

}

leaf-list objectInstances {

type types3gpp:DistinguishedName;

yext3gpp:inVariant;

description "List of managed object instances. Each object instance is

identified by its DN.";

}

}

typedef mgtDataCategoryType {

type enumeration {

enum COVERAGE;

enum CAPACITY;

enum ENERGY_EFFICIENCY;

enum MOBILITY;

enum ACCESSIBILITY;

}

description "";

}

grouping ManagementDataCollectionGrp {

uses ManagementDataGrp {

description "This attribute defines the list of management data that are

requested.";

}

list targetNodeFilter {

key idx;

leaf idx {

type string;

}

min-elements 1;

description "Set of information to target the Object Instance to collect

the measurements from.";

uses NodeFilterGrp;

}

list collectionTimeWindow {

key "startTime endTime";

max-elements 1;

description "Collection time window for which the management data

should be reported.";

uses types3gpp:TimeWindowGrp;

}

list reportingCtrl {

key idx;

leaf idx {

type string;

}

min-elements 1;

max-elements 1;

uses types3gpp:ReportingCtrl;

description "Selecting the reporting method and defining associated

control parameters.";

}

leaf dataScope {

type enumeration {

enum SNSSAI;

enum 5QI;

}

// mandatory false; [Implicit]

description "It specifies whether the required data is reported per

S-NSSAI or per 5QI.";

}

leaf condition {

type string;

mandatory true;

yext3gpp:inVariant;

description "Logical expression of one or several condition(s).

The actual syntax and capabilities of condition is SS specific. However,

each SS should support condition consisting of one or several assertions

that may be grouped using the logical operators AND, OR and NOT. Only if

the whole expression of condition evaluates TRUE, the attribute

conditionStatus will be TRUE.

Each assertion is a pointer to a Boolean parameter or a logical

expression of attribute existence or attribute value comparison

('equal to X, less than Y' etc.).

An empty string is not allowed.";

}

list processMonitor {

config false;

min-elements 1;

max-elements 1;

key idx;

leaf idx { type string; }

description "Indicates the process of the ManagementDataCollection MOI.";

}

leaf consolidateOutput {

type boolean;

mandatory true;

yext3gpp:inVariant;

description "Indicates whether the management data collection output will

be consolidated into a single file per reporting period.";

}

leaf jobId {

type string;

yext3gpp:inVariant;

description "Identifier of a PerfMetricJob, a TraceJob or a QMCJob.";

}

}

augment /subnet3gpp:SubNetwork {

list ManagementDataCollection {

key id;

uses top3gpp:Top_Grp ;

container attributes {

uses ManagementDataCollectionGrp;

}

description "This IOC represents a management data collection request

job. The requested data is of kind Trace, MDT (Minimization

of Drive Test), RLF (Radio Link Failure) report, RCEF (RRC Connection

Establishment Failure) report, RRC report,

PM (performance measurements), KPI

(end-to-end key performance indicators) or a combination of these.

The attribute managementData defines the management data which shall

be reported. This may either include a list of data categories or a

list of management data identified with their name. For further details

see TS 28.622 clause 4.3.50.

The targetNodeFilter attribute can be

used to target object instance(s) producing the required management

data. It is assumed that the consumer may not have detailed knowledge

of the network and hence may not identify the exact object instance

producing the required management data. In this case consumer can

request management data, specified by 3GPP, produced by certain network

function(s) based on a particular location, the domain (CN or RAN) of

the network function, and the handled traffic (CP or UP) of the network

function.

To activate the production of the requested data, a MnS consumer has to

create a ManagementDataCollection object instance on the MnS producer.

The attribute jobId is used to identify a management data collection

request.

The production and reporting of the management data can be constrained

by conditions such that only when the conditions are satisfied shall

management data collection be enabled. For example, a MnS consumer can

request to create two ManagementDataCollection instances. One can be

configured with high data producing and reporting period on a set of

conditions (e.g, to reduce transmission cost when network performance

metric is in normal range). Another can be configured with low data

producing and reporting period on another set of conditions (e.g. to

enable network optimization when network performance metric is in

abnormal range).

The MnS producer may derive multiple jobs (PerfMetricJob, TraceJob)

from a single ManagementDataCollection job for collecting the required

management data. The jobId of the request is used to identify the

output.  The value is also used to correlate the derived jobs which

will have the same jobId value as the ManagementDataCollection.

If the MnS producer receives the collected data from multiple sources,

it will consolidate the data into a set of management data for

reporting based on the value of the attribute consolidateOutput.

For consolidation of file-based management data the attribute

consolidateOutput controls:

-   True: the MnS Producer will combine the file output from jobs used

to collect the required management data into a single output file as

follows:

- File is in compressed format, i.e. zip.

-  File shall contain individual output files from each configured job

- Each file retains its original filename

-  Each file retains it original content

-  Consolidated filename uses naming convention defined in,

TS 28.532 clause 11.3.2.1.4 with the following:

-  <Type> is a combination of the management data types included in

the consolidated output

-  <UniqueIdList> is omitted

-  False: the MnS Producer will not combine the output from jobs used to collect the required management data.  The MnS Consumer will receive separate output from the derived jobs.

Subject to the reporting method, the MnS Consumer may receive file

related notifications.  When consolidated output is selected the

MnS Producer will create file notifications for the consolidated files.

When consolidated output is not selected, the MnS Producer will create

the notification subscriptions on behalf of the MnS Consumer and the

MnS Consumer will receive notifications directly from the derived jobs.

The attribute collectionTimeWindow specifies the time window for which

the management data should be reported. The attributes startTime and

the endTime can be in the past, present or in the future.

The attribute reportingCtrl specifies the method and associated control

parameters for reporting the produced management data to MnS consumers.

Three methods are available: file-based reporting with selection of the

file location by the MnS producer, file-based reporting with selection

of the file location by the MnS consumer and stream-based reporting.

The attribute dataScope configures, whether the management data should

be reported per S-NSSAI or per 5QI or per PLMN, if applicable.

The attribute processMonitor allows the MnS consumer to monitor the

status of the management data collection represented by the object

ManagementDataCollection.

The MnS producer indicates in the attribute progressStateInfo the

state of the management data collection:

-  NOT_STARTED

- RUNNING

- CANCELING

and indicates in the attribute resultStateInfo:

-  FINISHED

-  FAILED

-  PARTIALLY_FAILED

-  CANCELLED";

}

}

}

<CODE ENDS>

*** END OF CHANGE 28 ***

*** START OF CHANGE 29 ***

*** yang-models/_3gpp-common-measurements.yang ***

<CODE BEGINS>

module _3gpp-common-measurements {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-measurements";

prefix "meas3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-files { prefix files3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines Measurement and KPI related groupings

Any list/class intending to use this should include 2 or 3 uses statements

controlled by a feature:

A)

+++     feature MeasurementsUnderMyClass {

+++       description 'Indicates whether measurements and/or KPIs are supported

+++       for this class.';

+++     }

B) include the attribute measurementsList and/or kPIsList indicating the

supported measurment and KPI types and GPs. Note that for classes

inheriting from ManagedFunction, EP_RP or SubNetwork these attributes are

already inherited, so there is no need to include them once more. E.g.

+++     grouping MyClassGrp {

+++        uses meas3gpp:SupportedPerfMetricGroup;

+++     }

C) include the class PerfmetricJob to control the measurements/KPIs. E.g.

list MyClass {

container attributes {

uses MyClassGrp;

}

+++       uses meas3gpp:MeasurementSubtree {

+++         if-feature MeasurementsUnderMyClass ;

+++       }

}

Measurements can be contained under ManagedElement, SubNetwork, or

any list representing a class inheriting from Subnetwork or

ManagedFunction.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2025-08-01 { reference CR-0551; }

revision 2025-05-19 { reference CR-0532; }

revision 2025-02-07 { reference CR-0461; }

revision 2025-01-29 { reference CR-0512; }

revision 2024-11-01 { reference CR-0488; }

revision 2024-05-06 { reference CR-0359; }

revision 2024-02-24 { reference CR-0346; }

revision 2023-11-18 { reference "CR-0299 CR-0305"; }

revision 2023-09-18 { reference CR-0271; }

revision 2023-04-26 { reference CR-0250; }

revision 2023-02-18 { reference "CR-0240"; }

revision 2023-02-14 { reference "CR-0234"; }

revision 2022-11-04 { reference "CR-0212 CR-0194"; }

revision 2022-10-24 { reference CR-0196; }

revision 2022-09-30 { reference CR-0191; }

revision 2021-07-22 { reference "CR-0137"; }

revision 2020-11-06 { reference "CR-0118"; }

revision 2020-09-04 { reference "CR-000107"; }

revision 2020-06-08 { reference "CR-0092"; }

revision 2020-05-31 { reference "CR-0084"; }

revision 2020-03-11 { reference "S5-201581, SP-200229"; }

revision 2019-11-21 { reference "S5-197275, S5-197735"; }

revision 2019-10-28 { reference "S5-193516"; }

revision 2019-06-17 { reference " "; }

feature FilesUnderPerfMetricJob {

description "Files shall be contained under PerfMetricJob";

}

grouping ThresholdInfoGrp {

description "Defines a single threshold level.";

leaf-list performanceMetrics {

type string;

min-elements 1;

description "List of performance metrics.

Performance metrics include measurements defined in TS 28.552 and KPIs

defined in TS 28.554 [28]. Performance metrics can also be specified

by other SDOs, or be vendor specific. Performance metrics are

identified with their names.

For measurements defined in TS 28.552 the name is constructed as the

bullet e) of the measurement definition.

A name can also identify a vendor specific performance metric or a

group of vendor specific performance metrics.";

}

leaf thresholdDirection {

type enumeration {

enum UP;

enum DOWN;

enum UP_AND_DOWN;

}

must '. = "UP_AND_DOWN" or  not(../hysteresis)' {

error-message "In case a threshold with hysteresis is configured, the "

+"threshold direction attribute shall be set to 'UP_AND_DOWN'.";

}

mandatory true;

description "Direction of a threshold indicating the direction for which

a threshold crossing triggers a threshold.

When the threshold direction is configured to 'UP', the associated

treshold is triggered only when the performance metric value is going

up upon reaching or crossing the threshold value. The treshold is not

triggered, when the performance metric is going down upon reaching or

crossing the threshold value.

Vice versa, when the threshold direction is configured to 'DOWN', the

associated treshold is triggered only when the performance metric is

going down upon reaching or crossing the threshold value. The treshold

is not triggered, when the performance metric is going up upon reaching

or crossing the threshold value.

When the threshold direction is set to 'UP_AND_DOWN' the treshold is

active in both direcions.

In case a threshold with hysteresis is configured, the threshold

direction attribute shall be set to 'UP_AND_DOWN'.";

}

leaf thresholdValue {

type union {

type int64;

type decimal64 {

fraction-digits 2;

}

}

mandatory true;

description "Value against which the monitored performance metric is

compared at a threshold level in case the hysteresis is zero";

}

leaf hysteresis {

type union {

type uint64;

type decimal64 {

fraction-digits 2;

range "0..max";

}

}

must '. >= 0';

description "Hysteresis of a threshold. If this attribute is present

the monitored performance metric is not compared against the

threshold value as specified by the thresholdValue attribute but

against a high and low threshold value given by

threshold-high = thresholdValue + hysteresis

threshold-low = thresholdValue - hysteresis

When going up, the threshold is triggered when the performance metric

reaches or crosses the high threshold value. When going down, the

hreshold is triggered when the performance metric reaches or crosses

the low threshold value.

A hysteresis may be present only when the monitored performance

metric is not of type counter that can go up only. If present

for a performance metric of type counter, it shall be ignored.";

}

}

grouping SupportedPerfMetricGroupGrp {

list SupportedPerfMetricGroups {

config false;

key idx;

description "Captures a group of supported performance metrics and

associated parameters related to their production and reporting.

A SupportedPerfMetricGroup attribute which is part of an MOI may

define performanceMetrics for any MOI under the subtree contained

under that MOI, e.g. SupportedPerfMetricGroup on a ManagedElement

can specify supported metrics for contained ManagedFunctions

like a GNBDUFunction.";

leaf idx { type uint32; }

leaf-list performanceMetrics {

type string;

min-elements 1;

description "Performance metrics include measurements defined in

TS 28.552 and KPIs defined in TS 28.554.

Measurements are identified by name.

For measurements defined in TS 28.552 the name is constructed as

the bullet e) of the measurement definition.

For KPIs defined in TS 28.554 the name is defined in the KPI

definitions template as the component designated with e).

For non-3GPP specified measurements the name is defined

elsewhere.";

}

leaf-list granularityPeriods {

type uint32 {

range 1..max ;

}

units seconds;

description "Granularity periods supported for the associated

measurement types. The period is defined in seconds.";

}

leaf-list reportingMethods {

type enumeration {

enum FILE_BASED_LOC_SET_BY_PRODUCER;

enum FILE_BASED_LOC_SET_BY_CONSUMER;

enum STREAM_BASED;

}

min-elements 1;

}

leaf-list reportingPeriods {

type uint32 {

range 1..max ;

}

units seconds;

description "Reporting periods supported for the associated

measurement types. The period is defined in seconds.";

}

}

}

grouping PerfMetricJobGrp {

description "Represents the attributes of the IOC PerfMetricJob";

leaf administrativeState {

default UNLOCKED;

type types3gpp:BasicAdministrativeState ;

description "Enable or disables production of the metrics";

}

leaf operationalState {

config false;

mandatory true;

type types3gpp:OperationalState ;

description "Indicates whether the PerfMetricJob is working.";

}

leaf jobId {

type string;

description "Id for a PerfMetricJob job.";

yext3gpp:inVariant;

}

leaf-list performanceMetrics {

type string;

min-elements 1;

description "Performance metrics include measurements defined in

TS 28.552 and KPIs defined in TS 28.554. Performance metrics can

also be those specified by other SDOs or vendor specific metrics.

Performance metrics are identfied with their names. A name can also

identify a vendor specific group of performance metrics.

For measurements defined in TS 28.552 the name is constructed as

the bullet e) of the measurement definition.

For KPIs defined in TS 28.554 the name is defined in the KPI

definitions template as the component designated with e).";

}

leaf granularityPeriod {

type uint32 {

range 1..max ;

}

units seconds;

mandatory true;

description "Granularity period used to produce measurements. The value

must be one of the supported granularity periods for the metric.";

}

leaf-list objectInstances {

type types3gpp:DistinguishedName;

}

leaf-list rootObjectInstances {

type types3gpp:DistinguishedName;

description "Each object instance designates the root of a subtree that

contains the root object and all descendant objects.";

}

uses types3gpp:ReportingCtrl {

refine "reportingCtrl/file-based-reporting/fileReportingPeriod" {

must '(number(.)*"60") mod number(../granularityPeriod) = "0"' {

error-message

"The time-period must be a multiple of the granularityPeriod.";

}

}

}

leaf _linkToFiles {

type string ;

config false;

mandatory true;

yext3gpp:notNotifyable ;

description "Link to a 'Files' object.";

yext3gpp:inVariant;

}

choice conditiona-or-schedule {

leaf schedulerRef {

type types3gpp:DistinguishedName;

description "Pointer to a Scheduler object.";

}

leaf conditionMonitorRef {

type types3gpp:DistinguishedName;

description "Pointer to a ConditionMonitor object.";

}

}

}

grouping ThresholdMonitorGrp {

description "A threshold monitor that is created by the consumer for

the monitored entities whose measurements are required by consumer

to monitor.";

leaf administrativeState {

default UNLOCKED;

type types3gpp:BasicAdministrativeState ;

description "Enables or disables the ThresholdMonitor.";

}

leaf operationalState {

config false;

mandatory true;

type types3gpp:OperationalState ;

description "Indicates whether the ThresholdMonitor is working.";

}

list thresholdInfoList {

key idx;

min-elements 1;

leaf idx { type uint32 ; }

uses ThresholdInfoGrp;

description "List of threshold info.";

}

leaf monitorGranularityPeriod {

type uint32 {

range "1..max";

}

units second;

mandatory true;

description " Granularity period used to monitor measurements for

threshold crossings. ";

}

leaf-list objectInstances {

type types3gpp:DistinguishedName;

yext3gpp:notNotifyable;

}

leaf-list rootObjectInstances {

type types3gpp:DistinguishedName;

description "Each object instance designates the root of a subtree that

contains the root object and all descendant objects.";

yext3gpp:notNotifyable;

}

}

grouping MeasurementSubtree {

description "Contains classes that define measurements.

Should be used in all  classes (or classes inheriting from)

- SubNnetwork

- ManagedElement

- ManagedFunction

If a YANG module wants to augment these classes/list/groupings they must

augment all user classes!

If a class uses this grouping in its list it shall also use the

grouping SupportedPerfMetricGroupGrp to add SupportedPerfMetricGroup as

an attribute to its grouping";

list PerfMetricJob {

description "This IOC represents a performance metric production job. It

can be name-contained by SubNetwork, ManagedElement, or ManagedFunction.

To activate the production of the specified performance metrics, a MnS

consumer needs to create a PerfMetricJob instance on the MnS producer.

For ultimate deactivation of metric production, the MnS consumer should

delete the job to free up resources on the MnS producer.

For temporary suspension of metric production, the MnS consumer can

manipulate the value of the administrative state attribute. The MnS

producer may disable metric production as well, for example in overload

situations. This situation is indicated by the MnS producer with setting

the operational state attribute to disabled. When production is resumed

the operational state is set back to enabled.

The jobId attribute can be used to associate metrics from multiple

PerfMetricJob instances. The jobId can be included when reporting

performance metrics to allow a MnS consumer to associate received

metrics for the same purpose.  For example, it is possible to configure

the same jobId value for multiple PerfMetricJob instances required to

produce the measurements for a specific KPI.

The attribute performanceMetrics defines the performance metrics to be

produced and the attribute granularityPeriod defines the granularity

period to be applied.

All object instances below and including the instance name-containing

the PerfMetricJob (base object instance) are scoped for performance

metric production. Performance metrics are produced only on those object

instances whose object class matches the object class associated to the

performance metrics to be produced.

The attributes objectInstances and rootObjectInstances allow to restrict

the scope. When the attribute objectInstances is present, only the object

instances identified by this attribute are scoped. When the attribute

rootObjectInstances is present, then the subtrees whose root objects are

identified by this attribute are scoped. Both attributes may be present

at the same time meaning the total scope is equal to the sum of both

scopes. Object instances may be scoped by both the objectInstances and

rootObjectInstances attributes. This shall not be considered as an error

by the MnS producer.

When the performance metric requires performance metric production on

multiple managed objects, which is for example the case for KPIs, the

MnS consumer needs to ensure all required objects are scoped. Otherwise

a PerfMetricJob creation request shall fail.

The attribute reportingCtrl specifies the method and associated control

parameters for reporting the produced measurements to MnS consumers.

Three methods are available: file-based reporting with selection of the

file location by the MnS producer, file-based reporting with selection

of the file location by the MnS consumer and stream-based reporting.

For file-based reporting, all performance metrics that are produced

related to a 'PerfMetricJob' instance for a reporting period shall be

stored in a single reporting file.

When the administrative state is set to 'UNLOCKED' after the creation

of a 'PerfMetricJob' the first granularity period shall start. When

the administrative state is set to 'LOCKED' or the operational state

to 'DISABLED', the ongoing reporting period shall be aborted, for

streaming the ongoing granularity period. When the administrative

state is set back to 'UNLOCKED' or the operational state to 'ENABLED'

a new reporting period period shall start, in case of streaming a new

granularity period.

Changes of all other configurable attributes shall take effect only at

the beginning of the next reporting period, for streaming at the

beginning of the next granularity period.

When the 'PerfMetricJob' is deleted, the ongoing reporting period shall

be aborted, for streaming the ongoing granularity period.

A PerfMetricJob creation request shall fail, when the requested

performance metrics, the requested granularity period, the requested

repoting method, or the requested combination thereof is not supported

by the MnS producer.

Creation and deletion of PerfMetricJob instances by MnS consumers is

optional; when not supported, PerfMetricJob instances may be created and

deleted by the system or be pre-installed.";

key id;

uses top3gpp:Top_Grp ;

container attributes {

uses PerfMetricJobGrp ;

}

uses files3gpp:FilesSubtree {

if-feature FilesUnderPerfMetricJob;

}

}

list ThresholdMonitor {

key id;

description "Represents a threshold monitor for performance metrics.

It can be contained by SubNetwork, ManagedElement, or ManagedFunction.

A threshold monitor checks for threshold crossings of performance metric

values and generates a notification when that happens.

The ThresholdMonitor shall be used only when NRM based threshold

monitoring is supported.

To activate threshold monitoring, a MnS consumer needs to create a

ThresholdMonitor instance on the MnS producer. For ultimate deactivation

of threshold monitoring, the MnS consumer should delete the monitor to

free up resources on the MnS producer.

For temporary suspension of threshold monitoring, the MnS consumer can

manipulate the value of the administrative state attribute. The MnS

producer may disable threshold monitoring as well, for example in

overload situations. This situation is indicated by the MnS producer with

setting the operational state attribute to disabled. When monitoring is

resumed the operational state is set again to enabled.

All object instances below and including the instance containing the

ThresholdMonitor (base object instance) are scoped for performance

metric production. Performance metrics are monitored only on those

object instances whose object class matches the object class associated

to the performance metrics to be monitored.

The optional attributes objectInstances and rootObjectInstances allow to

restrict the scope. When the attribute objectInstances is present, only

the object instances identified by this attribute are scoped. When the

attribute rootObjectInstances is present, then the subtrees whose root

objects are identified by this attribute are scoped. Both attributes may

be present at the same time meaning the total scope is equal to the sum

of both scopes. Object instances may be scoped by both the objectInstances

and rootObjectInstances attributes. This shall not be considered as an

error by the MnS producer.

Multiple thresholds can be defined for multiple performance metric sets

in a single monitor using thresholdInfoList. The attribute

monitorGranularityPeriod defines the granularity period to be applied.

The value is a supported GP for the measurements being monitored.

Threshold crossing behaviour is as defined in [54], Annex F.

Each threshold is identified with a number (key) called thresholdLevel.

A threshold is defined using the attributes thresholdValue ,

thresholdDirection and hysteresis.

When hysteresis is absent or carries no information, a threshold is

triggered when the thresholdValue is reached or crossed. When hysteresis

is present, two threshold values are specified for the threshold as

follows: A high treshold value equal to the threshold value plus the

hysteresis value, and a low threshold value equal to the threshold value

minus the hysteresis value. When the monitored performance metric

increases, the threshold is triggered when the high threshold value is

reached or crossed. When the monitored performance metric decreases, the

threshold is triggered when the low threshold value is reached or crossed.

The hsyteresis ensures that the performance metric value can oscillate

around a comparison value without triggering each time the threshold when

the threshold value is crossed.

Using the thresholdDirection attribute a threshold can be configured in

such a manner that it is triggered only when the monitored performance

metric is going up or down upon reaching or crossing the threshold.

A ThresholdMonitor creation request shall be rejected, if the performance

metrics requested to be monitored, the requested granularity period, or

the requested combination thereof is not supported by the MnS producer.

A creation request may fail, when the performance metrics requested to be

monitored are not produced by a PerfMetricJob.

Creation and deletion of ThresholdMonitor instances by MnS consumers is

optional; when not supported, ThresholdMonitor instances may be created

and deleted by the system or be pre-installed.";

uses top3gpp:Top_Grp ;

container attributes {

uses ThresholdMonitorGrp ;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 29 ***

*** START OF CHANGE 30 ***

*** yang-models/_3gpp-common-mnsregistry.yang ***

<CODE BEGINS>

module _3gpp-common-mnsregistry {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-mnsregistry";

prefix "mnsregist3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-managementdatacollection { prefix mgtdatcol3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the MNSRegistry IOC and its

contained classes that ares part of the Generic Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623 Generic Network Resource Model (NRM)";

revision 2025-07-19 { reference CR-0551; }

revision 2023-09-30 { reference CR-0278; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2023-05-10 { reference CR-0243; }

revision 2023-02-14 { reference CR-0234; }

revision 2021-11-23 { reference "S5-216090"; }

revision 2021-10-18 { reference "S5-215263"; }

revision 2021-08-29 { reference "Initial revision, S5-214388"; }

grouping MgmtDataInfoGrp {

description "Represents the MgmtDataInfo IOC.";

uses mgtdatcol3gpp:ManagementDataGrp {

description "Defines the list of management data that can be supported.

The management data is a choice between:

- a list of data categories (attribute mgtDataCategory)

- a list of management data identified with their name

(attribute 'mgtDataName').";

}

leaf-list supportedGranularityPeriods {

type int32;

units seconds;

description "Granularity periods supported for the production of

associated management data. The period is defined in seconds.";

}

leaf-list supportedReportingPeriods {

type uint32;

units seconds;

description "Reporting periods supported for the associated management

data. The period is defined in seconds.";

}

leaf historicalDataPeriod {

type uint64;

units seconds;

description "This attribute describes the maximum period of the requested

historical data. The period is defined in seconds.

When the value of this attribute is NULL, which means the capability of

querying historical data is not supported.";

}

leaf-list supportedReportingMethod {

type enumeration {

enum FILE;

enum STREAM;

}

min-elements 1;

description "List of supported reporting methods for the associated

management data.";

}

leaf-list supportedDataScope {

type enumeration {

enum SNSSAI;

enum 5QI;

enum PLMN;

}

min-elements 1;

description "List of supported sub counter capabilities for the associated

management data";

}

leaf-list supportedDataRequestMnSRef {

type types3gpp:DistinguishedName;

min-elements 1;

description "List of DN of MnSInfo for the MnS instance(s) which can be

used to request the associated management data";

}

leaf-list supportedDataReportingMnSRef {

type types3gpp:DistinguishedName;

min-elements 1;

description "List of DN of MnSInfo for the MnS instance(s) which can be

used to report the associated management data";

}

}

grouping MnsScopeGrp {

description "This <<choice>> specifies the list of managed object instances

that can be accessed using the Management Service. These managed object

instances can be represented with one of the following options:

-  A list of DNs, i.e. representing managed object instances identified

by the DN.

-  A list of GeoAreas, i.e. representing managed object instances covered

by the specified geographical areas. In the present document, the

geoAreasList is only used for MnS consumer to retrieve MnS

scope of the specified MnS instance.

-  A list of TAIs, i.e. representing managed object instances covered

by the specified TAIs.";

choice MnsScope {

mandatory true;

leaf-list objectInstanceList {

type types3gpp:DistinguishedName;

yext3gpp:inVariant;

yext3gpp:notNotifyable;

description "This attribute describes list of DNs for the managed

object instances that can be accessed using the Management Service.

If a complete SubNetwork can be accessed using the Management Service,

this attribute may contain the DN of the SubNetwork instead of the

DNs of the individual managed entities within the SubNetwork.

If a complete ManagedElement can be accessed using the Management

Service, this attribute may contain the DN of the ManagedElement

instead of the DNs of the individual managed entities within the

ManagedElement.";

}

list geoAreasList {

yext3gpp:inVariant;

yext3gpp:notNotifyable;

description "This attribute describes geographical areas for the

managed object instances that can be accessed using the Management

Service.";

key idx;

leaf idx { type uint32; }

uses types3gpp:GeoAreaGrp;

}

list taiList {

yext3gpp:inVariant;

yext3gpp:notNotifyable;

description "This attribute describes the list of Tracking Area

Identities (TAI) for the managed object instances that can be accessed

using the Management Service.";

key idx;

leaf idx { type uint32; }

uses types3gpp:TaiGrp;

}

}

}

grouping MnsInfoGrp {

description "Represents the MnsInfo IOC.";

leaf mnsLabel {

description "Human-readable name of management service.";

mandatory true;

type string;

}

leaf mnsType {

description "Type of management service.";

type enumeration {

enum ProvMnS;

enum FaultSupervisionMnS;

enum StreamingDataReportingMnS;

enum FileDataReportingMnS;

}

}

leaf mnsVersion {

description "Version of management service.";

type string;

}

leaf mnsAddress {

description "Addressing information for Management Service operations.";

mandatory true;

type string;

}

leaf-list mnsCapability {

type union {

type enumeration {

enum NR_PROVISIONING;

enum 5GC_PROVISIONING;

enum NETWORK_SLICING_PROVISIONING;

enum EDGE_COMPUTING_PROVISIONING;

enum PERFORMANCE_METRIC_COLLECTION_CONTROL;

enum PERFORMANCE_METRIC_DATA_REPORT;

enum PERFORMANCE_METRIC_THRESHOLD_MONITOR_CONTROL;

enum PERFORMANCE_METRIC_THRESHOLD_NOTIFICATION;

enum FAULT_CONTROL;

enum FAULT_NOTIFICATION;

enum TRACE_MDT_DATA_COLLECTION_CONTROL;

enum TRACE_MDT_DATA_REPORT;

enum QOE_DATA_COLLECTION_CONTROL;

enum QOE_DATA_REPORT;

enum FILE_RETRIEVAL;

enum FILE_DOWNLOAD;

enum SUBSCRIPTION_CONTROL;

enum HEARTBEAT_CONTROL;

enum HEARTBEAT_NOTIFICATION;

enum ML_MODEL_MANAGEMENT;

enum MANAGEMENT_DATA_ANALYTIC;

enum RANSC_MANAGEMENT;

enum SON_POLICY;

enum COMMUNICATION_SERVICE_ASSURANCE_CONTROL;

enum INTENT_DRIVEN_MANAGEMENT;

enum MNS_REGISTRY_AND_DISCOVERY;

enum MNS_ACCESS_CONTROL_MANAGEMENT;

enum DSO_RAPID_RECOVERY_AND_THRESHOLD_MONITORING;

}

type string;

}

description "It describes the types of management capabilities of the

MnS instance provided by the MnS producer. The detailed description for

above enum values see Annex F in TS 28.533.

Note: vendor extension values are allowed for the attribute

'mnsCapability'";

}

list mnsScope {

description "This attribute defines the information about the

management scope of the Management Service. The management scope is

used to represent  the set of managed object instances that can be

accessed using the Management Service.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses MnsScopeGrp;

}

leaf-list mgmtDataInfoRef {

type types3gpp:DistinguishedName;

yext3gpp:notNotifyable;

min-elements 1;

description "List of DN of MgmtDataInfo instance(s) which are associated

the MnSInfo which represent a management service instance";

}

}

grouping MnsRegistryGrp {

description "Currently no own attributes defined.";

}

augment "/subnet3gpp:SubNetwork" {

list MnsRegistry {

description "This IOC is a container for MnsInfo IOC-s. It can be

contained only by SubNetwork IOC. A SubNetwork IOC can contain

only one instance of MnsRegistry.

The IOC is instantiated by the system.";

key id;

max-elements 1;

uses top3gpp:Top_Grp;

container attributes {

uses MnsRegistryGrp;

}

list MnsInfo {

description "This IOC represents an available Management Service (MnS)

and provides the data required to support its discovery.

It is name-contained by MnsRegistry.

This information is used by the consumer to discover the producers

of specific Management Services and to derive the addresses of the

Management Service.

Attributes mnsLabel, mnsType, and mnsVersion are used to describe

the Management Service.

Attribute mnsAddress is used to provide addressing information for

the Management Service operations.

Attribute mnsScope is used to provide information about the

management scope of the Management Service. The management scope

is used to represent the set of managed object instances that can

be accessed using the Management Service.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses MnsInfoGrp;

}

}

list MgmtDataInfo {

description "Describes the management capabilities for a set of

management data, which can be used for management data discovery.

MgmtDataInfo  is contained by MnSRegistry and associated to MnSInfo.

Multiple MgmtDataInfo instances maybe created for different sets of

management data, it is MnS producer’s decision.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses MgmtDataInfoGrp;

}

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 30 ***

*** START OF CHANGE 31 ***

*** yang-models/_3gpp-common-subscription-control.yang ***

<CODE BEGINS>

module _3gpp-common-subscription-control {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-subscription-control";

prefix "subscr3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-yang-types { prefix yang; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines IOCs for subscription and heartbeat control.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.623";

revision 2025-07-19 { reference CR-0551; }

revision 2025-05-13 { reference "CR-0548"; }

revision 2025-03-24 { reference "CR-0516 0518"; }

revision 2024-05-18 { reference CR-0359; }

revision 2024-01-18 { reference "CR-0309 CR-0329"; }

revision 2023-09-18 { reference CR-0271; }

revision 2023-08-10 { reference "CR0257 CR0260"; }

revision 2022-10-20 { reference CR-0196;   }

revision 2021-01-16 { reference "CR-0120"; }

revision 2020-08-26 { reference "CR-0106"; }

revision 2019-11-29 { reference "S5-197648 S5-197647 S5-197829 S5-197828"; }

grouping ScopeGrp {

description "This <<dataType>> allows to select data nodes in an object

tree whose root is identified by the so called base managed object

instance. The identification of the base object instance is not part of

this <<dataType>> and needs to be specified by other means. The base

managed object instance is typically a managed object instance in an

object tree.

The 'scopeType' and the 'scopeLevel' attributes allow to select managed

object instances. Attributes, attribute fields and attribute elements

cannot be selected.

The 'dataNodeSelector' attribute allows to select managed object

instances, attributes, attribute fields, attribute elements, or

attribute field elements. Its value contains a solution set specific

expression for specifying the data nodes to be selected.";

choice scope {

description "Describes which object instances are selected with

respect to a base object instance.";

case type-level {

leaf scopeType {

type enumeration {

enum BASE_ONLY;

enum BASE_ALL;

enum BASE_NTH_LEVEL;

enum BASE_SUBTREE;

}

mandatory true;

description "If the optional scopeLevel parameter is not supported

or absent, allowed values of scopeType are BASE_ONLY and BASE_ALL.

The value BASE_ONLY indicates only the base object is selected.

The value BASE_ALL indicates the base object and all of its

subordinate objects (incl. the leaf objects) are selected.

If the scopeLevel parameter is supported and present, allowed

values of scopeType are BASE_ALL, BASE_ONLY, BASE_NTH_LEVEL

and BASE_SUBTREE.

The value BASE_NTH_LEVEL indicates all objects on the level,

which is specified by the scopeLevel parameter, below the base

object are selected. The base object is at scopeLevel zero.

The value BASE_SUBTREE indicates the base object and all of its

subordinate objects down to and including the objects on the level,

which is specified by the scopeLevel parameter, are selected.

The base object is at scopeLevel zero.";

}

leaf scopeLevel {

when '../scopeType = "BASE_NTH_LEVEL" or ../scopeType = "BASE_SUBTREE"';

type uint16;

mandatory true;

description "See description of scopeType.";

}

}

case dataNodeSelector {

container dataNodeSelector {

description "The value shall follow the rules of RFC 8641

filter-spec";

reference "RFC 8641 section 5. and ietf-yang-push.yang";

choice dataNodeSelector {

anydata datastore-subtree-filter {

description "Subtree filter";

reference

"RFC 6241: Network Configuration Protocol (NETCONF),  Section 6";

}

leaf datastore-xpath-filter {

type yang:xpath1.0;

description "XPath-filter";

reference

"XML Path Language (XPath) Version 1.0

(https://www.w3.org/TR/1999/REC-xpath-19991116)

RFC 7950: The YANG 1.1 Data Modeling Language,

Section 10";

}

}

}

}

}

}

typedef NotificationTypes {

description "An extensible enumeration of notification types";

type union {

type enumeration {

enum notifyMOICreation;

enum notifyMOIDeletion;

enum notifyMOIAttributeValueChanges;

enum notifyMOIChanges;

enum notifyEvent;

enum notifyNewAlarm;

enum notifyAckStateChanged;

enum notifyComments;

enum notifyCorrelatedNotificationChanged;

enum notifyChangedAlarmGeneral;

enum notifyClearedAlarm;

enum notifyAlarmListRebuilt;

enum notifyPotentialFaultyAlarmList;

enum notifyFileReady;

enum notifyFilePreparationError;

enum notifyThresholdCrossing ;

enum notifyPotentialFaultyDataNodeTree;

enum notifyDataNodeTreeSyncRecommended;

}

type string;

}

}

grouping NtfSubscriptionControlGrp {

description "Attributes of a specific notification subscription";

leaf notificationRecipientAddress {

type string;

mandatory true;

}

leaf-list notificationTypes {

type NotificationTypes;

description "Defines the types of notifications that are candidates

for being forwarded to the notification recipient.

If the notificationTypes attribute is not supported or not present

all candidate notifications types are forwarded to the notification;

discriminated by notificationFilter attribute.";

}

list scope {

description "Scopes (selects) data nodes in an object tree.";

key idx;

max-elements 1;

leaf idx { type string; }

uses ScopeGrp;

}

leaf notificationFilter {

type string;

description "Defines a filter to be applied to candidate notifications

identified by the notificationTypes attribute.

If notificationFilter is present, only notifications that pass the

filter criteria are forwarded to the notification recipient; all other

notifications are discarded.

The filter can be applied to any field of a notification.

The format of the string shall confrm to a

JSON expressions (Jex) 'JexConditionsExpr'";

reference "3GPP TS 32.161";

}

leaf lastSequenceNo {

type uint64;

config false;

mandatory true;

description "The sequence number of the last notification that was sent

by a 'NtfSubscriptionControl' instance.";

}

leaf operationalState {

type types3gpp:OperationalState;

config false;

mandatory true;

description "Operational state of manged object instance.

The operational state describes if an object instance is operable

('ENABLED') or inoperable ('DISABLED').

This state is set by the object instance or the MnS producer and

is hence READ-ONLY.";

}

leaf-list availabilityStatus {

type types3gpp:AvailabilityStatus;

config false;

description "The availability status provides additional information

about the operational state";

}

}

grouping HeartbeatControlGrp {

description "Attributes of HeartbeatControl.";

leaf heartbeatNtfPeriod {

type uint32;

mandatory true;

units seconds;

description "Specifies the periodicity of heartbeat notification emission.

The value of zero has the special meaning of stopping the heartbeat

notification emission.";

}

leaf triggerHeartbeatNtf {

type boolean;

default false;

description "Setting this attribute to 'true' triggers an immediate

additional heartbeat notification emission. Setting the value to

'false' has no observable result.

The periodicity of notifyHeartbeat emission is not changed.

After triggering the heartbeat the system SHALL set the value

back to false.";

yext3gpp:notNotifyable;

}

}

grouping SubscriptionControlSubtree {

description "Contains notification subscription related classes.

Should be used in all classes (or classes inheriting from)

- SubNetwork

- ManagedElement

If some YAM wants to augment these classes/list/groupings they must

augment all user classes!";

list NtfSubscriptionControl {

description "NtfSubscriptionControl represents a notification

subscription of a notification recipient.

The scope attribute is used to select managed object instances included

in the subscription. The base object instance of the scope is the

object instance name-containing the NtfSubscriptionControl instance.

When the scope attribute is absent, all objects below and including

the base object are scoped. The notifications related to the selected

managed object instances are candidates to be sent to the address

specified by the notificationRecipientAddress attribute.

The notificationTypes attribute and notificationFilter attribute

allow MnS consumers to control which candidate notifications are

sent to the notificationRecipientAddress.

If the notificationTypes attribute is present, its value identifies

the notification types that are candidates to be sent to the

notificationRecipientAddress. If the notificationTypes attribute is

absent, notifications of all types are candidates to be sent to

notificationRecipientAddress. Notification types supported in the

NtfSubscriptionControl.notificationTypes attribute are the ones

listed in the attribute SupportedNotifications.notificationTypes.

If supported, the notificationFilter attribute defines a filter that

is applied to the set of candidate notifications. The filter is

applicable to all parameters of a notification. Only candidate

notifications that pass the filter criteria are sent to the

notificationRecipientAddress. If the notificationFilter attribute is

absent, all candidate notificatios are sent to the

notificationRecipientAddress.

To receive notifications, a MnS consumer has to create a

NtfSubscriptionControl instance on the MnS producer. A MnS consumer

can create a subscription for another MnS consumer since it is not

required the notificationRecipientAddress be his own address.

When a MnS consumer does not wish to receive notifications any more

the MnS consumer shall delete the corresponding NtfSubscriptionControl

instance.

When a subscription is created and the notification scope inludes

the created subscription object and the subscribed notification types

include notifications reporting object creation (notifyMOICreation

or notifyMOIChanges), the first notification sent related to the

new subscription shall report the creation of the

NtfSubscriptionControl instance. Likewise, when a subscription is

deleted and the notification scope includes the deleted subscription

object and the subscribed notification types include notifications

reporting object deletion (notifyMOIDeletion or notifyMOIChanges),

the last notification sent related to the subscription shall report

the deletion of the NtfSubscriptionControl instance.

If multiple NtfSubscriptionControl instances are configured to send

the same notification to the same notificationRecipientAddress, then

a separate notification message shall be sent for each such

NtfSubscriptionControl instance.

A 'NtfSubscriptionControl' class optionally supports adding a sequence

number parameter to the notification header. For each  notification

sent from a 'NtfSubscriptionControl' instance to the configured

notification recipient, the sequence number is increased by one.

This allows the notification recipient to detect notifications lost or

reordered in transit. The sequence number sent last is reflected in

the 'lastSequenceNo' attribute. This allows the MnS consumer to check

if he missed notifications in situations where he did not receive

any notification for some time.

The 'operationalState' attribute represents the operability of the

subscription. The 'availabilityStatus' further qualifies the

opertational state. Both attributes are set by the MnS producer.

If the 'operationalState' is set to 'ENABLED' and 'availabilityStatus'

has no value the subscription process is fully operational and

notifications are forwarded to the subscribed consumer.

If the 'operationalState' is set to 'ENABLED' and 'availabilityStatus'

is set to 'DEGRADED' the subscription process is degraded. There is

no guarantee that all notifications, that should be forwarded to the

notification recipient, are forwarded.

If the 'operationalState' is set to 'DISABLED' and 'availabilityStatus'

is set to 'FAILED' the subscription process is not operational, and

no notifications are sent to the notification recipient.

If the 'operationalState' is set to 'DISABLED' and 'availabilityStatus'

is set to 'DEPENDENCY' the subscription process itself has no problems

but some other process on which it depends such as downstream

processes in the event channel that feeds events into the subscription

process. As a result, events are not delivered to the subscription

process and no notifications are sent to the notification recipient.

When the subscription process is disabled or degraded and becomes

fully operational again, the MnS producer sends- the related data node

tree change notifications to subscribed MnS consumers. The MnS consumer

may use the reception of these notifications as a trigger to synchronize

his data node tree with the data node tree on the MnS producer.

However, the state change notifications contain no information about

which part of the data node tree should be synchronized. For this

purpose the 'notifyDataNodeTreeSyncRecommended' (TS 28.532 [27]) and

'alarmListRebuilt' (TS 28.111 [58]) notifications are provided.

Creation and deletion of NtfSubscriptionControl instances by MnS

consumers is optional; when not supported, the NtfSubscriptionControl

instances may be created and deleted by the system or be

pre-installed.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NtfSubscriptionControlGrp;

}

list HeartbeatControl {

description "MnS consumers (i.e. notification recipients) use heartbeat

notifications to monitor the communication channels between themselves

and MnS producers configured to emit notifications.

A HeartbeatControl instance allows controlling the emission of

heartbeat notifications by MnS producers. The recipients of heartbeat

notifications are specified by the notificationRecipientAddress

attribute of the NtfSubscriptionControl instance containing the

HeartbeatControl instance.

Note that the MnS consumer managing the HeartbeatControl instance

and the MnS consumer receiving the heartbeat notifications may not be

the same.

As a pre-condition for the emission of heartbeat notifications, a

HeartbeatControl instance needs to be created. Creation of an instance

with an initial non-zero value of the heartbeatNtfPeriod attribute

triggers an immediate heartbeat notification emission. Creation of an

instance with an initial zero value of the heartbeatPeriod attribute

does not trigger an emission of a heartbeat notification. Deletion of

an instance does not trigger an emission of a heartbeat notification.

Once the instance is created, heartbeat notifications are emitted with

a periodicity defined by the value of the heartbeatNtfPeriod

attribute. No heartbeat notifications are emitted if the value is

equal to zero. Setting a zero value to a non zero value, or a non zero

value to a different non zero value, triggers an immediate heartbeat

notification, that is the base for the new heartbeat period. Setting a

non zero value to a zero value stops emitting heartbeats immediately;

no final heartbeat notification is sent.

Creation and deletion of HeartbeatControl instances by MnS Consumers

is optional; when not supported, the HeartbeatControl instances may be

created and deleted by the system or be pre-installed.

Whether and when to emit heartbeat notifications is controlled by

HeartbeatControl. Subscription for heartbeat is not supported via the

NtfSubscriptionControl.";

max-elements 1;

key id;

uses top3gpp:Top_Grp;

container attributes {

uses HeartbeatControlGrp;

}

}

}

}

grouping SupportedNotificationsGrp {

description "Attributes of SupportedNotifications.";

leaf-list notificationTypes {

type NotificationTypes;

config false;

description "List of notification types supported by the MnS producer";

}

leaf-list notificationProtocols {

type enumeration {

enum HTTP;

enum HTTP_VES_ENCAPS;

}

config false;

min-elements 1;

description "List of protocols supported for notifications.";

reference "3GPP TS 28.532";

}

}

grouping SupportedNotificationsSubtree {

description "Contains SupportedNotifications.";

list SupportedNotifications {

description "SupportedNotifications represents the notification related

capabilities of a MnS producer.

The notificationTypes attribute lists notificationTypes supported

by the MnSProducer. Specific IOCs can be the source of a specific

but not necessary every supported notificationType.

The notificationProtocols attribute identifies the notification

transport protocols supported by a MnS producer.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SupportedNotificationsGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 31 ***

*** START OF CHANGE 32 ***

*** yang-models/_3gpp-common-trace.yang ***

<CODE BEGINS>

module _3gpp-common-trace {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-trace";

prefix "trace3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-extensions {prefix yext3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-files { prefix files3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types {prefix types5g3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Trace handling

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)" ;

revision 2025-06-25 { reference "CR-0551" ; }

revision 2025-05-07 { reference "CR-0532 CR-0536 CR-0540" ; }

revision 2025-02-07 { reference "CR-0504" ; }

revision 2025-02-05 { reference "CR-0461" ; }

revision 2024-11-25 { reference "CR-0492" ; }

revision 2024-11-07 { reference "CR-0480"; }

revision 2024-10-01 { reference "CR-0457"; }

revision 2024-08-13 { reference "CR-0387 CR-0377"; }

revision 2024-08-07 { reference "CR-0405 CR-0421 CR-0379"; }

revision 2024-05-12 { reference "CR-0401"; }

revision 2024-05-06 { reference CR-0359; }

revision 2024-04-06 { reference "CR-0342"; }

revision 2024-01-29 { reference "CR-0316"; }

revision 2023-11-06 { reference "CR-0290 CR-0294"; }

revision 2023-11-03 { reference CR-0302 ; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2023-08-10 { reference CR-0261; }

revision 2023-04-26 { reference CR-0250; }

revision 2023-02-18 { reference "CR-0234"; }

revision 2023-02-16 { reference "CR-0233"; }

revision 2022-09-30 { reference CR-0191 ; }

revision 2022-04-27 { reference "CR-0159"; }

revision 2021-10-18 { reference "CR-0139"; }

revision 2021-07-22 { reference "CR-0137"; }

revision 2021-01-25 { reference "CR-0122"; }

revision 2020-11-16 { reference "CR-0117"; }

revision 2020-08-06 { reference "CR-0102"; }

feature FilesUnderTraceJob {

description "Files shall be contained under TraceJob";

}

grouping FreqInfoGrp {

description "Represents the FreqInfo dataType.

This <<dataType>> defines the RF reference frequency and the frequency

operating bands used in a cell for a given direction (UL or DL) in FDD

or for both UL and DL directions in TDD";

leaf arfcn {

type uint32 {

range 0..3279165;

}

mandatory true;

description "RF Reference Frequency as defined in TS 38.104,

clause 5.4.2.1. The frequency provided identifies the absolute

frequency position of the reference resource block (Common RB 0)

of the carrier. Its lowest subcarrier is also known as Point A.";

}

leaf-list freqBands {

type uint32 {

range 1..1024;

}

min-elements 1;

description "List of NR frequency operating bands. Primary NR

Operating Band as defined in TS 38.104, clause 5.4.2.3.

The value 1 corresponds to n1, value 2 corresponds to NR operating

band n2, etc.";

}

}

grouping AreaConfigGrp {

description "Represents the AreaConfig dataType.

This <<dataType>> defines the area for which measurement logging should

be performed. It is described by a list of cells and a list of

frequencies.";

list freqInfo {

key arfcn;

min-elements 1;

max-elements 32;

description "It specifies the carrier frequency and bands used in

a cell.";

uses FreqInfoGrp ;

}

leaf-list pciList {

type uint32 {

range 0..1007;

}

min-elements 1;

max-elements 32;

description "List of neighbour cells subject for MDT scope.";

}

}

grouping AreaScopeGrp {

description "This <<dataType>> defines an area scope.";

choice AreaScopeChoice {

leaf-list eutraCellIdList {

type string;

min-elements 1;

max-elements 32;

description "List of E-UTRAN cells identified by E-UTRAN-CGI";

}

leaf-list nrCellIdList {

type string;

min-elements 1;

max-elements 32;

description "List of NR cells identified by NG-RAN CGI";

}

leaf-list tacList {

type types3gpp:Tac;

min-elements 1;

max-elements 8;

description "Tracking Area Code list";

}

list sliceIdList {

description "Network Slice Id list";

key idx;

uses types5g3gpp:PLMNInfo;

min-elements 1;

max-elements 16384;

leaf idx { type string; }

}

list taiList {

description "Tracking Area Identity list";

key idx;

min-elements 1;

max-elements 8;

leaf idx { type string; }

uses types3gpp:TaiGrp;

}

}

list nPNIdentityList {

description "list of NPN IDs of in NR. It is either a list of PNI-NPNs

identified by CAG ID with associated plmn-Identity or a list of SNPN

identified by Network ID with associated plmn-Identity";

key idx;

min-elements 1;

uses types3gpp:NpnIdGrp;

leaf idx { type string; }

}

}

grouping ExcessPacketDelayThresholdsGrp {

description "Represents the ExcessPacketDelayThresholds dataType.

This <<dataType>> defines a excess packet delay threshold information

to enable the calculation of the PDCP Excess Packet Delay in the

uplink in case of M6 uplink measurements are requested. The excess

packet delay threshold information is specified with the 5QI value

and excess packet delay threshold value.";

leaf fiveQIValue {

type uint8;

mandatory true;

description "It indicates 5QI value.";

}

leaf excessPacketDelayThresholdValue {

type decimal64 {

fraction-digits 2;

range 0.25|0.5|1|2|4|5|10|20|30|40|50|60|70|80|90|100|150|300|500 ;

}

mandatory true;

units milliseconds;

description "Value of excess packet delay threshold

for M6 UL measurement in milliseconds.";

}

}

grouping TraceReferenceGrp {

description "Represents the TraceReference dataType.

This <<dataType>> defines a globally unique identifier, which uniquely

identifies the Trace Session that is created by the TraceJob. It is

composed of the MCC, MNC (resulting in PLMN identifier) and the

trace identifier.";

uses types3gpp:PLMNId;  // mcc+mnc

leaf traceId {

type string {

pattern "[0-9a-fA-F]{6}";

}

mandatory true;

description "An identifier, which identifies the Trace

(together with MCC and MNC). This is a 3 byte Octet String,

expressed in hexadecimal form e.g., '1a7b3d').";

}

}

grouping MbsfnAreaGrp {

description "Represents the MbsfnArea dataType.

This <<dataType>> defines a MBSFN area. It is composed of the MBSFN Area

identifier and the carrier frequency (EARFCN).";

leaf mbsfnAreaId {

type uint32 {

range 1..max;

}

mandatory true;

description "MBSFN Area Identifier";

}

leaf earfcn{

type uint32 {

range 1..max;

}

mandatory true;

description "Carrier Frequency";

}

}

grouping TraceConfigGrp {

description "Defines the configuration parameters of TraceJob

which are specific for Trace or combined Trace and Immediate MDT.

The attribute listOfNeTypes specifies the network elements to be

traced. The optional attribute listOfInterfaces allows to specify

the individual interfaces of the network elements to be recorded.

The attribute traceDepth allows to configure the level of detail

of the information which shall be recorded.  For trace the reporting

is event based, where the triggering event is configured with

attribute triggeringEvents. For each triggering event the first and

last message (start/stop triggering event) to record  are specified.";

list listOfInterfaces {

key idx;

description "Specifies the interfaces that need to be traced in the given

ManagedEntityFunction.The attribute is applicable only for Trace. In

case this attribute is not used, it carries a null semantic.";

reference "Clause 5.5 of 3GPP TS 32.422 for additional details on the

allowed values.";

leaf idx { type uint32 ; }

leaf-list MSCServerInterfaces {

type enumeration {

enum A ;

enum Iu-CS ;

enum Mc ;

enum MAP-G ;

enum MAP-B ;

enum MAP-E ;

enum MAP-F ;

enum MAP-D ;

enum MAP-C ;

enum CAP ;

}

}

leaf-list MGWInterfaces {

type enumeration {

enum Mc ;

enum Nb-UP ;

enum Iu-UP ;

}

}

leaf-list RNCInterfaces {

type enumeration {

enum Iu-CS ;

enum Iu-PS ;

enum Iur ;

enum Iub ;

enum Uu ;

}

}

leaf-list SGSNInterfaces {

type enumeration {

enum Gb ;

enum Iu-PS ;

enum Gn ;

enum MAP-Gr ;

enum MAP-Gd ;

enum MAP-Gf ;

enum Ge ;

enum Gs ;

enum S6d ;

enum S4 ;

enum S3 ;

enum S13 ;

}

}

leaf-list GGSNInterfaces {

type enumeration {

enum Gn ;

enum Gi ;

enum Gmb ;

}

}

leaf-list S-CSCFInterfaces {

type enumeration {

enum Mw ;

enum Mg ;

enum Mr ;

enum Mi ;

}

}

leaf-list P-CSCFInterfaces {

type enumeration {

enum Gm ;

enum Mw ;

}

}

leaf-list I-CSCFInterfaces {

type enumeration {

enum Cx ;

enum Dx ;

enum Mg ;

enum Mw ;

}

}

leaf-list MRFCInterfaces {

type enumeration {

enum Mp ;

enum Mr ;

}

}

leaf-list MGCFInterfaces {

type enumeration {

enum Mg ;

enum Mj ;

enum Mn ;

}

}

leaf-list IBCFInterfaces {

type enumeration {

enum Ix ;

enum Mx ;

}

}

leaf-list E-CSCFInterfaces {

type enumeration {

enum Mw ;

enum Ml ;

enum Mm ;

enum Mi-Mg ;

}

}

leaf-list BGCFInterfaces {

type enumeration {

enum Mi ;

enum Mj ;

enum Mk ;

}

}

leaf-list ASInterfaces {

type enumeration {

enum Dh ;

enum Sh ;

enum ISC ;

enum Ut ;

}

}

leaf-list HSSInterfaces {

type enumeration {

enum MAP-C ;

enum MAP-D ;

enum Gc ;

enum Gr ;

enum Cx ;

enum S6d ;

enum S6a ;

enum Sh ;

}

}

leaf-list EIRInterfaces {

type enumeration {

enum MAP-F ;

enum S13 ;

enum MAP-Gf ;

}

}

leaf-list BM-SCInterfaces {

type enumeration {

enum Gmb ;

}

}

leaf-list MMEInterfaces {

type enumeration {

enum S1-MME ;

enum S3 ;

enum S6a ;

enum S10 ;

enum S11 ;

enum S13 ;

}

}

leaf-list SGWInterfaces {

type enumeration {

enum S4 ;

enum S5 ;

enum S8 ;

enum S11 ;

enum Gxc ;

}

}

leaf-list PDN_GWInterfaces {

type enumeration {

enum S2a ;

enum S2b ;

enum S2c ;

enum S5 ;

enum S6b ;

enum Gx ;

enum S8 ;

enum SGi ;

}

}

leaf-list eNBInterfaces {

type enumeration {

enum S1-MME ;

enum X2 ;

}

}

leaf-list en-gNBInterfaces {

type enumeration {

enum S1-MME ;

enum X2 ;

enum Uu ;

enum F1-C ;

enum E1 ;

}

}

leaf-list AMFInterfaces {

type enumeration {

enum N1 ;

enum N2 ;

enum N8 ;

enum N11 ;

enum N12 ;

enum N14 ;

enum N15 ;

enum N20 ;

enum N22 ;

enum N26 ;

}

}

leaf-list AUSFInterfaces {

type enumeration {

enum N12 ;

enum N13 ;

}

}

leaf-list NEFInterfaces {

type enumeration {

enum N29 ;

enum N30 ;

enum N33 ;

}

}

leaf-list NRFInterfaces {

type enumeration {

enum N27 ;

}

}

leaf-list NSSFInterfaces {

type enumeration {

enum N22 ;

enum N31 ;

}

}

leaf-list PCFInterfaces {

type enumeration {

enum N5 ;

enum N7 ;

enum N15 ;

}

}

leaf-list SMFInterfaces {

type enumeration {

enum N4 ;

enum N7 ;

enum N10 ;

enum N11 ;

enum S5-C ;

enum N38 ;

enum N16 ;

enum N16a ;

}

}

leaf-list SMSFInterfaces {

type enumeration {

enum N20 ;

enum N21 ;

}

}

leaf-list UDMInterfaces {

type enumeration {

enum N8 ;

enum N10 ;

enum N13 ;

enum N21 ;

}

}

leaf-list UPFInterfaces {

type enumeration {

enum N4 ;

}

}

leaf-list ng-eNBInterfaces {

type enumeration {

enum NG-C ;

enum Xn-C ;

enum Uu ;

}

}

leaf-list gNB-CU-CPInterfaces {

type enumeration {

enum NG-C ;

enum Xn-C ;

enum Uu ;

enum F1-C ;

enum E1 ;

enum X2-C ;

}

}

leaf-list gNB-CU-UPInterfaces {

type enumeration {

enum E1 ;

}

}

leaf-list gNB-DUInterfaces {

type enumeration {

enum F1-C ;

}

}

}

leaf-list listOfNeTypes {

type enumeration {

enum MSC_SERVER;

enum SGSN;

enum MGW;

enum GGSN;

enum RNC;

enum BM_SC;

enum MME;

enum SGW;

enum PGW;

enum ENB;

enum EN_GNB;

enum GNB_CU_CP;

enum GNB_CU_UP;

enum GNB_DU;

}

description "Specifies the network element types where the trace should

be activated. The attribute is applicable only for Trace with

Signalling Based Trace activation.";

reference "Clause 5.4 of 3GPP TS 32.422. ";

}

leaf traceDepth {

when '../../jobType = "TRACE_ONLY"'

+  ' or ../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration  {

enum MINIMUM;

enum MEDIUM;

enum MAXIMUM;

enum VENDORMINIMUM;

enum VENDORMEDIUM;

enum VENDORMAXIMUM;

}

default MAXIMUM;

description "It specifies the trace depth. The attribute is applicable

only for Trace.";

reference "Clause 5.3 of 3GPP TS 32.422.";

}

list triggeringEvents {

key idx;

description "It specifies the triggering event parameter of the trace

session. The attribute is applicable only for Trace.";

reference "Clause 5.1 of 3GPP TS 32.422";

leaf idx { type uint32; }

max-elements 1;

leaf-list MSC_SERVER {

type enumeration {

enum MO_MT_CALLS;

enum MO_MT_SMS;

enum LU_IMSIattach_IMSIdetach;

enum SS;

}

}

leaf-list SGSN {

type enumeration {

enum PDPcontext;

enum MO_MT_SMS;

enum RAU_GPRSattach_GPRSdetach;

enum MBMScontext;

}

}

leaf-list MGW {

type enumeration {

enum CONTEXT;

}

}

leaf-list GGSN {

type enumeration {

enum PDPcontext;

enum MBMScontext;

}

}

leaf-list IMS {

type enumeration {

enum SIPsession_StandaloenTransaction;

}

}

leaf-list BM_SC {

type enumeration {

enum MBMSactivation;

}

}

leaf-list MME {

type enumeration {

enum UEinitiatedPDNconnectivityRequest;

enum ServiceRequest;

enum InitialAttach_TAU_Detach;

enum UEInitiatedPDNdisconnection;

enum BearerActivationModificationDeletion;

enum Handover;

}

}

leaf-list SGW {

type enumeration {

enum PDNconnectionCreation;

enum PDNconnectionTermination;

enum BearerActivationModificationDeletion;

}

}

leaf-list PGW {

type enumeration {

enum PDNconnectionCreation;

enum PDNconnectionTermination;

enum BearerActivationModificationDeletion;

}

}

leaf-list AMF {

type enumeration {

enum Registration;

enum ServiceRequest;

enum Handover;

enum UEderegistration;

enum NetworkDeregistration;

enum UEMobilityFromEPC;

enum UEMobilityToEPC;

enum CovergedChargingService;

}

}

leaf-list SMF {

type enumeration {

enum PDUsessionEstablishment;

enum PDUsessionModification;

enum PDUsessionRelease;

enum PDUsessionUPactivationDeactivation;

enum MobilityBtw3gppAndN3gppTo5GC;

enum MobilityFromEpc;

enum MobilityBtwISMForVSMF;

enum CovergedChargingService;

enum OfflineOnlyCharging;

}

}

leaf-list PCF {

type enumeration {

enum AMpolicy;

enum SMpolicy;

enum Authorization;

enum BDTpolicy;

enum SpendingLimitControl;

}

}

leaf-list UPF {

type enumeration {

enum N4Session;

}

}

leaf-list AUSF {

type enumeration {

enum UEauthentication;

}

}

leaf-list NEF {

type enumeration {

enum EventExposure;

enum PFDmanagement;

enum ParameterProvisiong;

enum Trigger;

}

}

leaf-list NRF {

type enumeration {

enum NFmanagement;

enum NFdiscovery;

}

}

leaf-list NSSF {

type enumeration {

enum NSSelection;

enum NSSAI;

}

}

leaf-list SMSF {

type enumeration {

enum SMService;

}

}

leaf-list UDM {

type enumeration {

enum UEcontext;

enum SubscriberData;

enum UEauthentication;

enum EventExposure;

}

}

}

}

grouping ImmediateMdtConfigGrp {

description "Represents the ImmediateMdtConfig dataType.

This <<dataType>> defines the configuration parameters of

IOC TraceJob which are specific for Immediate MDT or combine

Trace and Immediate MDT.

The optional attribute positioningMethod allows to specify

the positioning methods to use.

The following attributes are conditional available based on the

measurements configured in listOfMeasurements:

-reportInterval (conditional for M1 in LTE or NR and M1/M2 in UMTS),

-reportAmount (conditional for M1/M2 in UMTS),

-reportAmountM1LTE (conditional for M1 in LTE),

-reportAmountM4LTE (conditional for M4 in LTE),

-reportAmountM5LTE (conditional for M5 in LTE),

-reportAmountM6LTE (conditional for M6 in LTE),

-reportAmountM7LTE (conditional for M7 in LTE),

-reportAmountM1NR (conditional for M1 in NR),

-reportAmountM4NR (conditional for M4 in NR),

-reportAmountM5NR (conditional for M5 in NR),

-reportAmountM6NR (conditional for M6 in NR),

-reportAmountM7NR (conditional for M7 in NR),

-reportingTrigger (conditional for M1 in LTE or NR and M1/M2 in UMTS),

-eventThreshold (conditional for A2 event reporting or A2 event

triggered periodic reporting),

-collectionPeriodRRMNR (conditional for M4 and M5 in NR),

-collectionPeriodM6NR (conditional for M6 in NR),

-collectionPeriodM7NR (conditional for M7 in NR),

-collectionPeriodRRMLTE (conditional for M3 in LTE),

-measurementPeriodLTE (conditional for M4 and M5 in LTE),

-collectionPeriodM6LTE (conditional for M6 in LTE),

-collectionPeriodM7LTE (conditional for M7 in LTE),

-collectionPeriodRRMUMTS (conditional for M4 and M5 in UMTS),

-measurementPeriodUMTS (conditional for M6 and M7 in UMTS),

-measurementQuantity (conditional for 1F event reporting),

-beamLevelMeasurement (conditional for M1 in NR),

-excessPacketDelayThresholds (conditional for M6 UL measurement in NR).

For immediate MDT, the measurement reporting is dependent on the

configured measurements:

- For measurement M1 in LTE or NR, it is possible to select between

periodical, event triggered, event triggered periodic reporting or

reporting according to all configured RRM event triggers. For M1 and M2

measurement in UMTS, it is possible to select between periodical, event

triggered reporting or reporting according to all configured RRM event

triggers. Parameter reportingTrigger determines which of the reporting

methods is selected and in case of event triggered or event-triggered

periodic, which is the decisive event type. For periodical reporting,

parameters reportInterval and reportAmount determine the interval between

two successive reports and the number of reports. This means the

periodical reporting terminates after reportAmount reports have been

sent as long as reportAmount is configured with a value different from

infinity. For event-triggered periodic reporting, these two parameters

apply in addition to parameter eventThreshold which determines the

threshold of the event. In this case up to reportAmount reports are

sent with a periodicity of reportInterval after the entering condition

is fulfilled. The reporting is stopped, if the leaving condition is

fulfulled and is restarted if the configured event reoccurs. For event

based reporting, there is only one report sent after the event occurs.

The parameters to configure are reportingTrigger and eventThreshold.

In case of UMTS and 1f event reporting, additionally parameter

measurementQuantity is necessary in order to determine for which

measurement(s) the event threshold is applicable.  Parameter

beamLevelMeasurement determines whether beam level measurements shall

be included in case of NR.

- For measurement M2 in LTE or NR, reporting is according to RRM

configuration, see TS 38.321, TS 36.321 and TS 38.331, TS 36.331.

- For measurement M4 in UMTS, reporting is either according to RRM

configuration, see TS 25.321 and TS 25.331 or periodic or event

triggered periodic using parameter collectionPeriodRRMUMTS and

eventThresholdUphUMTS.

- For measurement M3 in UMTS, the reporting is done upon

availability, see TS 37.320.

- For measurements M4, M5, M6 and M7 in NR, for measurements

M3, M4, M5, M6 and M7 in LTE and for measurements M5, M6 and M7

in UMTS periodical reporting is applied. The configurable parameter

is the interval between two measurements (collectionPeriodRRMNR,

collectionPeriodM6NR, collectionPeriodM7NR, collectionPeriodRRMLTE,

measurementPeriodLte, collectionPeriodM6LTE, collectionPeriodM7LTE,

collectionPeriodRRMUMTS, measurementPeriodUMTS) and the number of

reports (reportAmountM4NR, reportAmountM5NR, reportAmountM6NR,

reportAmountM7NR, reportAmountM4LTE, reportAmountM5LTE,

reportAmountM6LTE,reportAmountM7LTE).  If no collection period

is configured for M5 in UMTS, all available measurements are

logged according to RRM configuration.

- Measurements M8 and M9 in NR or LTE are reported according to

configured M1 and/or M6 related UE measurement reporting.

";

leaf listOfMeasurements {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum M1;

enum M2;

enum M3;

enum M4;

enum M5;

enum M6_DL;

enum M6_UL;

enum M7_DL;

enum M7_UL;

enum M1_EVENT_TRIGGERED;

enum M6;

enum M7;

enum M8;

enum M9;

}

description "It specifies the UE measurements that shall be

collected in an Immediate MDT job. The attribute is

applicable only for Immediate MDT.";

reference "Clause 5.10.3 of 3GPP TS 32.422.";

}

leaf reportingTrigger {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum PERIODICAL;

enum A2_FOR_LTE;

enum 1F_FOR_UMTS;

enum 1I_FOR_UMTS_MCPS_TDD;

enum A2_TRIGGERED_PERIODIC_FOR_LTE;

enum ALL_CONFIGURED_RRM_FOR_LTE;

enum ALL_CONFIGURED_RRM_FOR_UMTS;

}

description "It specifies whether periodic or event based measurements

should be collected.

The attribute is applicable only for Immediate MDT and when the

listOfMeasurements is configured for M1 (for both UMTS and LTE)

or M2 (only for UMTS).";

reference "Clause 5.10.4 of 3GPP TS 32.422.";

}

leaf reportInterval {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' and ../reportingTrigger = "PERIODICAL"';

type uint32 {

range "120|240|250|480|500|640|1000|1024|2000|2048|3000|4000|"

+"5120|6000|8000|10240|12000|16000|20000|"

+"20480|24000|28000|32000|40960|60000|64000|"

+"360000|720000|1800000|3600000";

}

units milliseconds;

description "It specifies the interval between the periodical measurements

that shall be taken when the UE is in connected mode.

The attribute is applicable only for Immediate MDT and when

reportingTrigger is configured for periodical measurements.";

reference "5.10.5 of 3GPP TS 32.422.";

}

leaf reportAmount {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' and ../reportingTrigger = "PERIODICAL"';

type union {

type uint32 {

range "1|4|8|16|32|64" ;

}

type enumeration {

enum INFINITY;

}

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected.

The attribute is applicable only for Immediate MDT and when

reportingTrigger is configured for periodical measurements.";

reference "Clause 5.10.6 of 3GPP TS 32.422";

}

leaf eventThreshold {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"';

type int64;

description "Specifies the threshold which should trigger the reporting

in case A2 event reporting in LTE or 1F/1l event in UMTS. The attribute

is applicable only for Immediate MDT and when reportingTrigger is

configured for A2 event in LTE or 1F event or 1l event in UMTS.";

reference "Clauses 5.10.7 and 5.10.7a of 3GPP TS 32.422";

}

leaf collectionPeriodRRMNR {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1024|2048|5120|10240|60000";

}

units milliseconds;

description "Specifies the collection period for collecting RRM

configured measurement samples for M4, M5 in NR. The attribute is

applicable only for Immediate MDT.";

reference "Clause 5.10.30 of 3GPP TS 32.422";

}

leaf collectionPeriodM6NR {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 120ms;

enum 240ms;

enum 480ms;

enum 640ms;

enum 1024ms;

enum 2048ms;

enum 5120ms;

enum 10240ms;

enum 20480ms;

enum 40960ms;

enum 1min;

enum 6min;

enum 12min;

enum 30min;

}

description "It specifies the collection period for the Packet Delay

measurement (M6) for NR MDT taken by the gNB. The attribute is

applicable only for Immediate MDT. ";

reference "clause 5.10.34 of  TS 32.422";

}

leaf collectionPeriodM7NR {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1..60";

}

description "It specifies the collection period for the Packet Loss Rate

measurement (M7) for NR MDT taken by the gNB. The attribute is

applicable only for Immediate MDT.";

reference "clause 5.10.35 of  TS 32.422";

}

leaf collectionPeriodRRMLTE {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 100ms;

enum 1000ms;

enum 1024ms;

enum 1280ms;

enum 2048ms;

enum 2560ms;

enum 5120ms;

enum 10000ms;

enum 10240ms;

enum 1min;

}

description "Specifies the collection period for collecting RRM configured

measurement samples for M3 in LTE. The attribute is applicable only

for Immediate MDT.";

reference "Clause 5.10.20 of 3GPP TS 32.422.";

}

leaf measurementPeriodLTE {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 1024ms;

enum 2048ms;

enum 5120ms;

enum 10240ms;

enum 1min;

}

mandatory true;

description "It specifies the measurement period for the

Data Volume (M4) and Scheduled IP throughput

measurements (M5) for LTE MDT taken by the eNB.

The attribute is applicable only for Immediate MDT.";

reference "Clause 5.10.23 of 3GPP TS 32.422.";

}

leaf collectionPeriodM6LTE {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1024|2048|5120|10240";

}

units milliseconds;

description "Specifies the collection period for the Packet Delay

measurement (M6) for MDT taken by the eNB. The attribute is applicable

only for Immediate MDT. In case this attribute is not used,

it carries a null semantic.";

reference "Clause 5.10.32 of  TS 32.422. ";

}

leaf collectionPeriodM7LTE {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint16 {

range 1..60 ;

}

description "It specifies the collection period for the Packet Loss Rate

measurement (M7) for LTE MDT taken by the eNB. The attribute is

applicable only for Immediate MDT.";

reference "Clause 5.10.33 of  TS 32.422.";

}

leaf eventThresholdUphUMTS {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint16 {

range 0..31 ;

}

description "It specifies the threshold which should trigger

the reporting in case of event-triggered periodic reporting for M4

(UE power headroom measurement) in UMTS.";

reference "Clause 5.10.39 of TS 32.422.";

}

leaf collectionPeriodRRMUMTS {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "100|250|500|1000|2000|"

+"3000|4000|6000";

}

units milliseconds;

description "Specifies the collection period for collecting RRM configured

measurement samples for M3, M4, M5 in UMTS. The attribute is applicable

only for Immediate MDT. ";

reference "Clause 5.10.21 of 3GPP TS 32.422.";

}

leaf measurementPeriodUMTS {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  'or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1000|2000|3000|4000|6000|8000|12000|16000|20000|"

+"24000|28000|32000|64000";

}

units milliseconds;

description "It specifies the measurement period for the Data Volume and

Throughput measurements for MDT taken by RNC.

The attribute is applicable only for Immediate MDT. In case this

attribute is not used, it carries a null semantic.";

reference "Clause 5.10.22 of 3GPP TS 32.422";

}

leaf measurementQuantity {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum CPICH_ECNO;

enum CPICH_RSCP;

enum PATHLOSS;

}

description "It specifies the measurements that are collected in an MDT

job for a UMTS MDT configured for event triggered reporting.";

reference "Clause 5.10.15 of 3GPP TS 32.422.";

}

leaf beamLevelMeasurement {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type boolean;

default false;

description "Indicates whether the NR M1 beam level measurements shall

be included or not.";

reference "Clause 5.10.40 of 3GPP TS 32.422.";

}

leaf positioningMethod {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or ../../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum GNSS;

enum E_CELL_ID;

}

description "It specifies what positioning method should be used in the

MDT job.";

reference "Clause 5.10.19 of 3GPP TS 32.422.";

}

list excessPacketDelayThresholds {

description "Excess packet delay thresholds info for M6 UL measurement.";

min-elements 1;

key idx;

leaf idx { type string; }

uses ExcessPacketDelayThresholdsGrp;

}

leaf reportAmountM1LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.19 of 3GPP TS 32.422.";

}

leaf reportAmountM4LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM5LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM6LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM7LTE {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for LTE.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM1NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM4NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM5NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM6NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

leaf reportAmountM7NR {

type enumeration {

enum 1;

enum 2;

enum 4;

enum 8;

enum 16;

enum 32;

enum 64;

enum INFINITY;

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected mode.

The attribute is applicable only for Immediate MDT and combined Trace

and Immediate MDT and when reportingTrigger is configured for periodical

measurements and applicable only for NR.";

reference "Clause 5.10.6 of TS 32.422.";

}

}

grouping LoggedMdtConfigGrp {

description "This <<dataType>> defines the configuration parameters of

IOC TraceJob which are specific for Logged MDT or Logged MBSFN MDT.

Based on the value configured for attribute jobType in IOC TraceJob,

different attributes are available. In case of LOGGED_MDT_ONLY, the

attributes reportType, eventListForEventTriggeredMeasurement,

eventThresholdL1, hysteresisL1, timeToTriggerL1,

areaConfigurationForNeighCells and npnIdentityList are applicable.

In case of LOGGED_MBSFN_MDT,

the attribute mbsfnAreaList is applicable. The optional attribute

plmnList allows to specify the PLMNs where

measurement collection, status indication and log reporting is

allowed, the optional attribute areaConfigurationForNeighCell

allows to specify the area for which UE is requested to perform

measurements logging for neighbour cells which have list of

frequencies.

For logged MDT in UMTS and LTE, the reporting is periodical. Parameter

loggingInterval determines the interval between the reports and parameter

loggingDuration determines how long the configuration is valid meaning

after this duration has passed no further reports are sent. In NR, the

reporting can be periodical or event based, determined by parameter

reportType. For periodical reporting the same parameters as in LTE and

UMTS apply. For event based reporting,

parameter eventListForEventTriggeredMeasurement configures the event type,

namely 'out of coverage' or 'L1 event'. In case 'L1 event' is selected as

event type, the logging is performed according to parameter loggingInterval

at regular intervals only when the conditions indicated by

eventThresholdL1, hysteresisL1, timeToTriggerL1 (defining the thresholds,

hysteresis and time to trigger) are met and if UE is 'camped normally'

state (TS 38.331,

TS 38.304).  In case 'out of coverage' is selected as event type, the

logging is performed according to parameter loggingInterval at regular

intervals only when the UE is in 'any cell selection' state.

Furthermore, logging is performed immediately upon transition from the

'any cell selection' state to the 'camped normally' state (TS 38.331,

TS 38.304).";

leaf traceCollectionEntityId {

type int64;

description "It specifies the TCE Id which is sent to the UE in

Logged MDT.";

reference "Clause 5.10.11 of 3GPP TS 32.422.";

}

leaf loggingDuration {

when '../../../jobType = "LOGGED_MDT_ONLY" or'

+ ' ../../../jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "600|1200|2400|3600|5400|7200";

}

units seconds;

description "Specifies how long the MDT configuration is valid at the

UE in case of Logged MDT. The attribute is applicable only for

Logged MDT and Logged MBSFN MDT.";

reference "Clause 5.10.9 of 3GPP TS 32.422";

}

leaf loggingInterval {

when '../../../jobType = "LOGGED_MDT_ONLY" or'

+ ' ../../../jobType = "LOGGED_MBSFN_MDT"';

type enumeration {

enum 320ms;

enum 640ms;

enum 1280ms;

enum 2560ms;

enum 5120ms;

enum 10240ms;

enum 20480ms;

enum 30720ms;

enum 40960ms;

enum 61440ms;

enum INFINITY;

}

description "Specifies the periodicty for Logged MDT. The attribute is

applicable only for Logged MDT and Logged MBSFN MDT.";

reference "Clause 5.10.8 of 3GPP TS 32.422";

}

leaf reportType {

when '../../../jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum PERIODICAL;

enum EVENT_TRIGGERED;

}

description "It specifies report type for logged NR MDT";

reference "Clause 5.10.27 of 3GPP TS 32.422.";

}

leaf eventListForEventTriggeredMeasurement {

when '../../../jobType = "LOGGED_MDT_ONLY"';

type enumeration {

enum OUT_OF_COVERAGE ;

enum A2_EVENT ;

}

description "Specifies event types for event triggered measurement in the

case of logged NR MDT.  Each trace session may configure at most one

event. The UE shall perform logging of measurements only upon certain

condition being fulfilled:

- Out of coverage.

- A2 event.";

reference "Clause 5.10.28 of 3GPP TS 32.422.";

}

leaf eventThresholdL1 {

when '../../../jobType = "LOGGED_MDT_ONLY" or'

+ ' ../../../jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "0..127";

}

description "It specifies the threshold which should trigger

the reporting in case of event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT and when reportType

is configured for event triggered reporting and when

eventListForEventTriggeredMeasurement is configured for L1 event.";

reference "clause 5.10.36 of TS 32.422";

}

leaf hysteresisL1 {

when '../../../jobType = "LOGGED_MDT_ONLY" or '

+ '../../../jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "0..30";

}

description "It specifies the hysteresis used within the entry and leave

condition of the L1 event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT, when reportType

is configured for event triggered reporting and when

eventListForEventTriggeredMeasurement is configured for L1 event.";

reference "clause 5.10.37 of TS 32.422";

}

leaf timeToTriggerL1 {

when '../../../jobType = "LOGGED_MDT_ONLY" or '

+ '../../../jobType = "LOGGED_MBSFN_MDT"';

type int32 {

range 0|40|64|80|100|128|160|256|320|480|512|640|1024|1280|2560|5120;

}

units milliseconds;

description "It specifies the threshold which should trigger

the reporting in case of event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT, when reportType

is configured for event triggered reporting and when

eventListForEventTriggeredMeasurement is configured for L1 event.";

reference "clause 5.10.38 of TS 32.422";

}

list areaConfigurationForNeighCell {

when '../../../jobType = "LOGGED_MDT_ONLY"';

key "idx";

leaf idx { type uint32 ; }

description "It specifies the area for which UE is requested to perform

measurement logging for neighbour cells which have list of frequencies.

If it is not configured, the UE shall perform measurement logging for

all the neighbour cells.

Applicable only to NR Logged MDT.";

reference "Clause 5.10.26 of 3GPP TS 32.422.";

uses AreaConfigGrp;

}

list mbsfnAreaList {

when '../../../jobType = "LOGGED_MBSFN_MDT"';

key "mbsfnAreaId earfcn";

max-elements 8;

description "The MBSFN Area consists of a MBSFN Area ID and Carrier

Frequency (EARFCN). The target MBSFN area List can have up to 8 entries.

This parameter is applicable only if the job type is Logged MBSFN MDT.";

reference "Clause 5.10.25 of 3GPP TS 32.422.";

uses MbsfnAreaGrp;

}

list nPNIdentityList {

description "It defines which NPNs that can be served by the NR cell,

and which CAG IDs or NIDs can be supported by the NR cell for

corresponding PNI-NPN or SNPN in case of the cell is NPN-only cell.

(NPN-Identity referring to TS 38.331)";

key idx;

max-elements 1;

uses types3gpp:NpnIdGrp;

leaf idx { type string;}

}

}

grouping MdtConfigGrp {

description "Defines the configuration parameters of IOC

TraceJob which are specific for MDT. The attribute

anonymizationOfMdtData specifies the level of anonymization

of MDT data.  The attribute areaScope defines the area scope

of MDT, which is specified in clause 5.10.2 of TS 32.422.

The optional attribute sensorInformation allows to specify

the sensor information to include.  Based on the value

configured for attribute jobType in IOC TraceJob, the attributes

immediateMdtConfig or loggedMdtConfig are available: In case of

IMMEDIATE_MDT_ONLY or IMMEDIATE_MDT_AND_TRACE the attribute

immediateMdtConfig is applicable. In case of LOGGED_MDT_ONLY or

LOGGED_MBSFN_MDT the attribute loggedMdtConfig is applicable.";

leaf anonymizationOfMdtData {

when ../areaScope ;

type enumeration {

enum NO_IDENTITY;

enum TAC_OF_IMEI;

}

default NO_IDENTITY;

description "Specifies level of anonymization of MDT data.

This attribute is only applicable for management based activation.";

reference "Clause 5.10.12 of 3GPP TS 32.422";

}

list areaScope {

key "idx";

leaf idx { type uint32 ; }

description "It specifies the area where data shall be collected. ";

max-elements 1;

uses AreaScopeGrp;

}

leaf-list sensorInformation {

type enumeration {

enum BAROMETRIC_PRESSURE;

enum UE_SPEED;

enum UE_ORIENTATION;

}

description "It specifies which sensor information shall be included in

logged NR MDT and immediate NR MDT measurement if they are available.";

reference "Clause 5.10.29 of 3GPP TS 32.422.";

}

list immediateMdtConfig {

description "The set of parameters specific for Immediate MDT

configuration.";

key idx;

max-elements 1;

leaf idx { type string; }

uses ImmediateMdtConfigGrp;

}

list loggedMdtConfig {

description "The set of parameters specific for Logged MDT and Logged

MBSFN MDT configuration.";

key idx;

max-elements 1;

leaf idx { type string; }

uses LoggedMdtConfigGrp;

}

leaf mNOnly {

type boolean;

default false;

description "Specifies whether the MDT configuration is for MN

only or not. The value FALSE means the MDT configuration is for

both MN and SN. The value TRUE means the MDT configuration is

for MN only.";

}

list plmnList {

key "mcc mnc";

uses types3gpp:PLMNId;

max-elements 16;

description "It indicates the PLMNs where measurement collection, status

indication and log reporting is allowed.";

reference "Clause 5.10.24 of 3GPP TS 32.422.";

}

}

grouping UECoreMeasConfigGrp {

description "Represents the UECoreMeasConfig dataType.

This <<dataType>> defines the aconfiguration parameters of IOC TraceJob

which are specific for UE level measurements collection.";

leaf-list ueCoreMeasurements {

type string;

min-elements 1;

description "List of 5GC UE level measurements identified by name.

The list may include 5GC UE level measurements defined in TS 28.558,

or vendor specific measurements.

For non-3GPP specified 5GC UE level measurements the name is defined

elsewhere.";

}

leaf ueCoreMeasGranularityPeriod {

type uint32;

mandatory true;

units milliseconds;

description "Granularity period used to produce 5GC UE

level measurements. The period is defined in milliseconds (ms).";

}

leaf nfTypeToMeasure {

type string;

mandatory true;

description "It indicates the type of NE to produce the 5GC UE level

measurements.

allowedValues: The NF types represented by the measured object

classes as defined by f) of the 5GC UE level measurements specified

in TS 28.558.";

}

leaf-list objectInstances {

type string;

description "List of object instances.

Each object instance is identified by its DN.";

}

leaf-list rootObjectInstances {

type string;

description "List of root object instances.

Each object instance is identified by its DN and designates

the root of a substree that contains the root object and all

descendant objects.";

}

}

grouping TraceJobGrp {

leaf jobType {

type enumeration {

enum IMMEDIATE_MDT_ONLY;

enum LOGGED_MDT_ONLY;

enum TRACE_ONLY;

enum IMMEDIATE_MDT_AND_TRACE;

enum RLF_REPORT_ONLY;

enum RCEF_REPORT_ONLY;

enum LOGGED_MBSFN_MDT;

enum 5GC_UE_LEVEL_MEASUREMENTS_ONLY;

enum TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS;

enum IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS;

enum TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS;

enum RRC_REPORT;

}

default TRACE_ONLY;

description "It specifies whether the

TraceJob represents only MDT, Trace, RLF, RCEF, RRC or 5GC UE

level measurements job, or a combined job. It also defines the

MDT mode.";

reference "Clause 5.9a of 3GPP TS 32.422.";

}

leaf-list rrcReportType {

type enumeration {

enum RLF_REPORT;

enum RCEF_REPORT;

enum SHR;

enum SPR;

enum MHI;

enum RA_REPORT;

}

description "Specifies the RRC reports requested. ";

reference "3GPP TS 38.331";

}

list pLMNTarget {

key "mcc mnc";

description "It specifies which PLMN that the subscriber of the session to

be recorded uses as selected PLMN.";

reference "Clause 5.9b of 3GPP TS 32.422";

uses types3gpp:PLMNId;

}

leaf-list listOfTraceMetrics {

when '../jobType = "TRACE_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type string;

description "List of trace metrics identified by name.

Includes trace messages, MDT measurements (Immediate MDT,

Logged MDT, Logged MBSFN MDT), RLF, RCEF and RRC reports,

see TS 32.422 Trace messages are identified with their message

identifier. Trace metric identifier is constructed as defined

in clause 10 of TS 32.422.

For non-3GPP specified trace metrics the name is defined elsewhere.

";

reference "Clause 10 of 3GPP TS 32.422";

}

leaf traceCollectionEntityIPAddress {

type inet:ip-address;

description "It specifies the address of the Trace Collection Entity

when the attribute traceReportingFormat is configured for the

file-based reporting. The attribute is applicable for both Trace and

MDT.";

reference "Clause 5.9 of 3GPP TS 32.422.";

}

leaf traceReportingConsumerUri {

when '../traceReportingFormat  = "STREAMING"';

type inet:uri;

description "URI of the Streaming Trace data reporting MnS consumer

(a.k.a. streaming target).";

reference "Clause 5.9 of 3GPP TS 32.422.";

}

list traceReference {

key "idx";

min-elements 1;

max-elements 1;

description "A globally unique identifier, which uniquely identifies the

Trace Session that is created by the TraceJob.

In case of shared network, it is the MCC and MNC of the Participating

Operator that request the trace session that shall be provided.

The attribute is applicable for both Trace and MDT.";

reference "Clause 5.6 of 3GPP TS 32.422.";

leaf idx { type uint32 ; }

uses trace3gpp:TraceReferenceGrp ;

}

leaf jobId {

type string;

yext3gpp:inVariant;

description "Identifier of a TraceJob";

}

leaf traceReportingFormat {

type enumeration {

enum FILE_BASED;

enum STREAMING;

}

default FILE_BASED;

description "Specifies the trace reporting format - streaming trace

reporting or file-based trace reporting";

reference "3GPP TS 32.422 clause 5.11";

}

list traceTarget {

key "traceTargetType";

max-elements 1;

description "It specifies the target object of the Trace, MDT and

5GC UE level measurements collection. The attribute is applicable for

Trace, MDT, and 5GC UE level measurements collection.

In case of management based Immediate MDT, RLF reporting, RCEF

reporting or RRC reposring, the traceTarget attribute shall be null

value.";

leaf traceTargetType {

type enumeration {

enum IMSI;

enum IMEI;

enum IMEISV;

enum PUBLIC_ID;

enum UTRAN_CELL;

enum E_UTRAN_CELL;

enum NG_RAN_CELL;

enum ENB;

enum RNC;

enum GNB;

enum SUPI;

enum N4_SESSION_ID;

}

description "It specifies the target object of the Trace, MDT and

5GC UE level measurements collection. The attribute is applicable for

Trace, MDT, and 5GC UE level measurements collection.

The targetIdType shall be PUBLIC_ID in case of a Management Based

Activation is done to an SCSCFFunction (Serving Call Session Control

Function) or PCSCFFunction (Proxy Call Session Control Function)

The targetIdType shall be UTRAN_CELL only in case of

UTRAN cell traffic trace function.

The targetIdType shall be E-UTRAN_CELL only in case of E-UTRAN cell

traffic trace function.

The targetIdType shall be NG-RAN_CELL only in case of NR cell

traffic trace function.

The targetIdType shall be either IMSI, IMEI or

IMEISV if the Trace Session is activated to any of the following

ManagedEntity(ies):

- HssFunction

- MscServerFunction

- SgsnFunction

- GgsnFunction

- BmscFunction

- RncFunction

- MmeFunction

- ServingGWFunction

- PGWFunction

The targetIdType shall be either SUPI or IMEISV if the Trace Session

is activated to any of the following ManagedEntity(ies):

- AFFunction

- AMFFunction

- AUSFunction

- NEFFunction

- NRFFunction

- NSSFFunction

- PCFFunction

- SMFFunction

- UPFFunction

- UDMFunction

In case of signalling based MDT, the targetIdType shall be

able to carry PUBLIC_ID, IMSI, IMEI, IMEISV or SUPI.

In case of management based Logged MDT, the targetIdType

shall carry an eNB or a gNB or an RNC.

The Logged MDT should be initiated on the specified eNB/gNB/RNC in

targetIdValueList.

In case of signalling based 5GC UE level measurements collection,

the targetIdTypee shall be able to carry IMEISV or SUPI.

In case of management based 5GC UE level measurements collection,

the targetIdType shall be able to carry the corresponding

Measured UE Identifier as defined by the bullet g) of the 5GC UE

level measurements (see TS 28.558) when the TraceJob is created at

the subject ManagedEntity.";

}

leaf-list traceTargetValueList {

type string;

description "It specifies the ID value(s) of the target object defined

by traceTargetType";

}

reference "3GPP TS 32.422";

}

list traceConfig {

when '../jobType = "TRACE_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"'

+  ' or ../jobType = "TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS"'

+  ' or ../jobType =

"TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"';

key idx;

description "Trace config";

max-elements 1;

uses TraceConfigGrp;

leaf idx { type string; }

}

list mdtConfig {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"'

+ ' or ../jobType = "RLF_REPORT_ONLY"'

+ ' or ../jobType = "RCEF_REPORT_ONLY"'

+ ' or ../jobType = "LOGGED_MBSFN_MDT"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"'

+ ' or ../jobType =

"TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"';

key idx;

description "MDT config";

max-elements 1;

uses MdtConfigGrp;

leaf idx { type string; }

}

list ueCoreMeasConfig {

when '../jobType = "5GC_UE_LEVEL_MEASUREMENTS_ONLY"'

+ ' or ../jobType = "TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"'

+ ' or ../jobType =

"TRACE_AND_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS"';

key idx;

description "5GC UE level measurements config";

max-elements 1;

uses UECoreMeasConfigGrp;

leaf idx { type string; }

}

list nPNTarget {

description "applicable only for NR and shall be present in case of NPN

either a PNI-NPN or a SNPN) and for management-based activation when

several NPNs are supported in the RAN.";

key idx;

max-elements 1;

uses types3gpp:NpnIdGrp;

leaf idx { type string;}

}

}

grouping TraceSubtree {

description "Contains classes that manage Tracing.

Should be used in all  classes (or classes inheriting from)

- SubNetwork

- ManagedElement

- ManagedFunction

If a YANG module wants to augment these classes/list/groupings they must

augment all user classes!";

list TraceJob {

description "A TraceJob instance represents the Trace Control and

Configuration parameters of a particular Trace Job (see TS 32.421 and

TS 32.422 for details). It can be name-contained by SubNetwork,

ManagedElement, ManagedFunction.

To activate Trace Jobs, a MnS consumer has to create TraceJob object

instances on the MnS producer. A MnS consumer can activate a Trace Job

for another MnS consumer since it is not required the value of

traceCollectionEntityIPAddress or traceReportingConsumerUri to be

his own.

For the details of Trace Job activation see clauses 4.1.1.1.2 and

4.1.2.1.2 of TS 32.422.

When a MnS consumer wishes to deactivate a Trace Job, the MnS consumer

shall delete the corresponding TraceJob instance.

For details of management Trace Job activation/deactivation see clause

4.1.1.1.2 of TS 32.422.

The attribute traceReference specifies a globally unique ID and

identifies a Trace session. One Trace Session may be activated to

multiple Network Elements. The traceReference is populated by the

consumer that makes the request for a Trace Session.

The jobId attribute presents the job identifier of a TraceJob instance.

The jobId can be used to associate  multiple TraceJob instances.

For example, it is possible to configure the same jobId value for

multiple TraceJob instances required to produce the data (e.g. RSRP

values of M1 and RLF reports) for a specific network analysis.

The attribute traceReportingFormat defines the method for reporting

the produced measurements. The selectable options are file-based or

stream-based reporting. In case of file-based reporting the attribute

traceCollectionEntityIPAddress is used to specify the IP address to

which the trace records shall be transferred, while in case of

stream-based reporting the attribute traceReportingConsumerUri

specifies the streaming target.

The mandatory attribute traceTarget determines the target object of

the TraceJob. Dependent on the network element to which the Trace

Session is activated different types of the target object are possible.

The attribute pLMNTarget defines the PLMN for which sessions shall be

selected in the Trace Session in case of management based activation

when several PLMNs are supported in the RAN.The MDT configuration may

include area scope defined by network slice, in which case

the attribute pLMNTarget is not applicable.

The attribute listOfTraceMetrics allows configuraton of which metrics

shall be recorded.

The attribute jobType specifies the kind of data to collect. In case of

TRACE_ONLY, the configuration parameters of attribute traceConfig

shall be applied. In case of IMMEDIATE_MDT_ONLY, LOGGED_MDT_ONLY,

RLF_REPORT_ONLY, RCEF_REPORT_ONLY and LOGGED_MBSFN_MDT the

configuration parameters of

attribute mdtConfig or a subset of these shall be applied. In case of

5GC_UE_LEVEL_MEASUREMENTS_ONLY, the configuration parameters

of attribute ueCoreMeasConfig shall be applied. In case of any

combination of Trace, Immediate MDT, and 5GC UE level measurements,

the configuration parameters

of the corresponding attributes, traceConfig, mdtConfig and

ueCoreMeasConfig are applicable.

If jobType has the value RRC Report, the attribute rrcReportType shall

bepresent. The rrcReportType allows the tracing of RRC reports.

Creation and deletion of TraceJob instances by MnS consumers is

optional; when not supported, the TraceJob instances may be created

and deleted by the system or be pre-installed.";

key id;

uses top3gpp:Top_Grp ;

container attributes {

uses TraceJobGrp ;

}

uses files3gpp:FilesSubtree {

if-feature FilesUnderTraceJob;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 32 ***

*** START OF CHANGE 33 ***

*** yang-models/_3gpp-common-yang-types.yang ***

<CODE BEGINS>

module _3gpp-common-yang-types {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-yang-types";

prefix "types3gpp";

import ietf-inet-types { prefix inet; }

import ietf-yang-types { prefix yang; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "The model defines a YANG mapping of the top level

information classes used for management of 5G networks and

network slicing.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623";

revision 2025-08-13 { reference CR-0551 ; }

revision 2025-02-19 { reference CR-0512; }

revision 2025-02-07 { reference CR-0492; }

revision 2024-11-25 { reference CR-1442; }

revision 2024-11-25 { reference CR-1412; }

revision 2024-11-01 { reference CR-0463; }

revision 2024-08-13 { reference CR-0387; }

revision 2024-05-18 { reference CR-0359; }

revision 2024-04-12 { reference CR-0346; }

revision 2023-11-06 { reference CR-0305; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2023-08-09 { reference CR-0266; }

revision 2023-05-10 { reference CR-0250; }

revision 2023-02-14 { reference CR-0234; }

revision 2022-11-04 { reference "CR-0194"; }

revision 2022-10-24 { reference CR-0196;  }

revision 2022-07-26 { reference "CR-0180" ; }

revision 2022-02-09 { reference "CR-0144"; }

revision 2021-11-01 { reference "CR-0141"; }

revision 2021-09-30 {

description "Added Longitude, Latitude, TenthOfDegrees, OnOff.";

reference "CR-0138";

}

revision 2020-11-06 {

description "Removed incorrect S-NSSAI definitions.";

reference "CR-0118";

}

revision 2020-03-10 {

description "Removed faulty when statements.";

reference "SP-200229";

}

revision 2019-10-25 {

description "Added ManagedNFProfile.";

reference "S5-194457";

}

revision 2019-10-16 {

description "Added SAP and usageState.";

reference "S5-193518";

}

revision 2019-06-23 {

reference  "Initial version.";

}

typedef EnabledDisabled {

type enumeration {

enum DISABLED ;

enum ENABLED ;

}

}

grouping nameValuePair {

leaf name { type string; }

leaf value { type string; }

}

typedef DayOfWeekT {

type enumeration {

enum Monday;

enum Tuesday;

enum Wednesday;

enum Thursday;

enum Friday;

enum Saturday;

enum Sunday;

}

}

typedef DayOfWeek {

type enumeration {

enum MONDAY;

enum TUESDAY;

enum WEDNESDAY;

enum THURSADY;

enum FRIDAY;

enum SATURDAY;

enum SUNDAY;

}

}

typedef DateMonth {

type uint8 {

range 0..12;

}

}

typedef DateMonthDay {

type uint8 {

range 0..31;

}

}

typedef FullTime {

type yang:time-with-zone-offset;

}

grouping DayInYearGrp {

description "This <<dataType>> represents a day in a year.";

leaf month {

description "It indicates the month in a year.";

type DateMonth;

mandatory true;

}

leaf monthDay {

description "It indicates the day in a month.";

type DateMonthDay;

mandatory true;

}

}

grouping TimeWindowGrp {

leaf startTime {

type yang:date-and-time;

yext3gpp:inVariant;

}

leaf endTime {

type yang:date-and-time;

yext3gpp:inVariant;

}

}

grouping ProcessMonitorGrp {

description "Provides attributes to monitor the progress of processes

with specific purpose and limited lifetime running on MnS producers.

It may be used as data type for dedicated progress monitor attributes

when specifying the management representation of these processes.

The attributes in this clause are defined in a generic way.

For some attributes specialisations may be provided when specifying a

concrete process representation.

If a management operation on some IOCs triggers an associated

asynchronous process (whose progress shall be monitored), this should

also result in creating an attribute named 'processMonitor' (of type

'ProcessMonitor') in these IOC(s). The processMonitor attribute may be

accompanied by use-case specific additional data items.

The progress of the process is described by the 'status' and

'progressPercentage' attributes. Additional textual qualifications for

the 'status' attribute may be provided by the 'progressStateInfo' and

'resultStateInfo' attributes.

When the process is instantiated, the 'status' is set to 'NOT_RUNNING'

and the 'progressPercentage' to '0'. The MnS producer decides when to

start executing the process and to transition into the 'RUNNING' state.

This time is captured in the 'startTime' attribute. Alternatively, the

process may start to execute directly upon its instantiation. One

alternative must be selected when using this data type.

During the 'RUNNING' state the 'progressPercentage' attribute may be

repeatedly updated. The exact semantic of this attribute is subject to

further specialisation. The 'progressInfo' attribute may be used to

provide additional textual information in the 'NOT_RUNNING', 'CANCELLING'

and 'RUNNING' states. Further specialisation of

'progressStateInfo' may be provided where this data type is

used.

Upon successful completion of the process, the 'status' attribute is set

to 'FINISHED', the 'progressPercentage' to 100%. The time is captured in

the 'endTime' attribute. Additional textual information may be provided

in the 'resultStateInfo' attribute. The type of

'resultStateInfo' in this data type definition is 'String'.

Further specialisation of 'resultStateInfo' may be provided

where this data type is used.

In case the process fails to complete successfully, the 'status'

attribute is set to 'FAILED' or 'PARTIALLY_FAILED', the current value of

'progressPercentage' is frozen, and the time captured in 'endTime'. The

'resultStateInfo' specifies the reason for the failure.

Specific failure reasons may be specified where the data type defined in

this clause is used. The exact semantic of failure may be subject for

further specialisation as well.

In case the process is cancelled, the 'status' attribute is first set to

'CANCELLING' and when the process is really cancelled then to 'CANCELLED'.

The transition to 'CANCELLED' is captured in the 'endTime' attribute.

The value of 'progressPercentage' is frozen. Additional textual

information may be provided in the 'resultStateInfo' attribute.

The 'resultStateInfo' attribute is provided only for additional textual

qualification of the states 'FINISHED', 'FAILED', 'PARTIALLY_FAILED' or

'CANCELLED'. It shall not be used for making the outcome, that the

process may produce in case of success, available.

The process may have to be completed within a certain time after its

creation, for example because required data may not be available any

more after a certain time, or the process outcome is needed until a

certain time and when not provided by this time is not needed any more.

The time until the MnS producer automatically cancels the process is

indicated by the 'timer' attribute.";

leaf id {

type string;

mandatory true;

description "Id of the process. It is unique within a single

multivalue attribute of type ProcessMonitor.";

}

leaf status {

type enumeration {

enum NOT_STARTED ;

enum RUNNING ;

enum CANCELLING ;

enum FINISHED ;

enum FAILED ;

enum PARTIALLY_FAILED ;

enum CANCELLED ;

}

config false;

default  RUNNING;

description "Represents the status of the associated process,

whether it fails, succeeds etc.

It does not represent the returned values of a successfully finished

process. ";

}

leaf progressPercentage {

type uint8 {

range 0..100;

}

config false;

description "Progress of the associated process as percentage";

}

leaf-list progressStateInfo {

type string;

config false;

description "Additional textual qualification of the states

'NOT_STARTED', 'CANCELLING' and 'RUNNING'.

For specific processes, specific well-defined strings (e.g. string

patterns or enums) may be defined as a specialisation.";

}

leaf resultStateInfo {

type string;

config false;

description "Additional textual qualification of the states

'FINISHED', 'FAILED', 'PARTIALLY_FAILED and 'CANCELLED'.

For example, in the 'FAILED' or 'PARTIALLY_FAILED' state this

attribute may be used to provide error reasons.

This attribute shall not be used to make the outcome of the process

available for retrieval, if any. For this purpose, dedicated

attributes shall be specified when specifying the representation of

a specific process.

For specific processes, specific well-defined strings (e.g. string

patterns or enums) may be defined as a specialisation.";

}

leaf startTime {

type yang:date-and-time;

config false;

description "Start time of the associated process, i.e. the time when the

status changed from 'NOT_STARTED' to 'RUNNING'.";

}

leaf endTime {

type yang:date-and-time;

config false;

description "Date and time when status changed to 'SUCCESS', 'CANCELLED',

'FAILED' or 'PARTIALLY_FAILED'.

If the time is in the future, it is the estimated time

the process will end.";

}

leaf timer {

type uint32;

units minutes;

description "Time until the associated process is automatically cancelled.

If set, the system decreases the timer with time. When it reaches zero

the cancellation of the associated process is initiated by the

MnS_Producer.

If not set, there is no time limit for the process.

Once the timer is set, the consumer can not change it anymore.

If the consumer has not set the timer the MnS Producer may set it.";

yext3gpp:notNotifyable;

}

}

grouping NpnIdGrp {

description "Represents the NPN supported by the <<IOC>> using this

<<dataType>> as one of its attributes in case of the cell is a

NPN-only cell.";

list plmnId {

key "mcc mnc";

min-elements 1;

description "It specifies the PLMN Id of the NPN network.";

uses types3gpp:PLMNId;

}

choice npnidChoice {

leaf-list cAGIdList {

type string;

max-elements 256;

description "It identifies a CAG list containing up to 12

CAG-identifiers per Ue or up to 12 CAG-identifiers per cell,

see TS 38.331.

CAG ID is used to combine with PLMN ID to identify a PNI-NPN.CAG ID is

a hexadecimal range with size 32 bit";

}

leaf-list nIDList {

type string;

max-elements 16;

description "It identifies a list of NIDs containing up to 16 NIDs, see

TS 38.331. NID is used to combine with PLMN ID to identify an SNPN.

NID is a hexadecimal range with size 44 bit.";

}

}

}

typedef TenthOfDegrees {

type uint16 {

range 0..3600;

}

units "0.1 degrees";

description "A single integral value corresponding to an angle in degrees

between 0 and 360 with a resolution of 0.1 degrees.";

}

typedef Latitude {

type decimal64 {

fraction-digits 4;

range "-90.0000..+90.0000";

}

description "Latitude values";

}

typedef Longitude {

type decimal64 {

fraction-digits 4;

range "-180.0000..+180.0000";

}

description "Longitude values";

}

typedef Altitude  {

type decimal64 {

fraction-digits 6;

}

units "meters";

description

"Height from a reference 0 value.";

}

grouping GeographicalCoordinates {

description "This datatype represents the geographical coordinates";

reference "3GPP TS 28.538 clause 6.3.8";

leaf latitude {

type Latitude;

mandatory true;

}

leaf longitude {

type Longitude;

mandatory true;

}

}

typedef OnOff {

type enumeration {

enum ON;

enum OFF;

}

}

typedef usageState {

type enumeration {

enum IDLE;

enum ACTIVE;

enum BUSY;

}

description "It describes whether or not the resource is actively in

use at a specific instant, and if so, whether or not it has spare

capacity for additional users at that instant. The value is READ-ONLY.";

reference "ITU T Recommendation X.731";

}

grouping SAPGrp {

leaf host {

type inet:host;

mandatory true;

}

leaf port {

type inet:port-number;

mandatory true;

}

description "Service access point.";

reference "TS 28.622";

}

typedef Mcc {

description "The mobile country code consists of three decimal digits,

The first digit of the mobile country code identifies the geographic

region (the digits 1 and 8 are not used):";

type string {

pattern '[02-79][0-9][0-9]';

}

reference "3GPP TS 23.003 subclause 2.2 and 12.1";

}

typedef Mnc {

description "The mobile network code consists of two or three

decimal digits (for example: MNC of 001 is not the same as MNC of 01)";

type string {

pattern '[0-9][0-9][0-9]|[0-9][0-9]';

}

reference "3GPP TS 23.003 subclause 2.2 and 12.1";

}

grouping PLMNId {

leaf mcc {

mandatory true;

type Mcc;

}

leaf mnc {

mandatory true;

type Mnc;

}

reference "TS 38.413 clause 9.3.3.5";

}

typedef Nci {

description "NR Cell Identity. The NCI shall be of fixed length of 36 bits

and shall be coded using full hexadecimal representation.

The exact coding of the NCI is the responsibility of each PLMN operator";

reference "TS 23.003";

type union {

type string {

length 36;

pattern '[01]+';

}

type string {

length 9;

pattern '[a-fA-F0-9]*';

}

}

}

typedef OperationalState {

reference "3GPP TS 28.625 and ITU-T X.731";

type enumeration {

enum DISABLED {

value 0;

description "The resource is totally inoperable.";

}

enum ENABLED {

value 1;

description "The resource is partially or fully operable.";

}

}

}

typedef BasicAdministrativeState {

reference "3GPP TS 28.625 and ITU-T X.731";

type enumeration {

enum LOCKED {

value 0;

description "The resource is administratively prohibited from performing

services for its users.";

}

enum UNLOCKED {

value 1;

description "The resource is administratively permitted to perform

services for its users. This is independent of its inherent

operability.";

}

}

}

typedef AdministrativeState {

reference "3GPP TS 28.625 and ITU-T X.731";

type enumeration {

enum LOCKED {

value 0;

description "The resource is administratively prohibited from performing

services for its users.";

}

enum UNLOCKED {

value 1;

description "The resource is administratively permitted to perform

services for its users. This is independent of its inherent

operability.";

}

enum SHUTTINGDOWN {

value 2;

description "Use of the resource is administratively permitted to

existing instances of use only. While the system remains in

the shutting down state the manager or the managed element

may at any time cause the resource to transition to the

locked state.";

}

}

}

typedef AvailabilityStatus {

type enumeration {

enum IN_TEST;

enum FAILED;

enum POWER_OFF;

enum OFF_LINE;

enum OFF_DUTY;

enum DEPENDENCY;

enum DEGRADED;

enum NOT_INSTALLED;

enum LOG_FULL;

}

}

typedef CellState {

type enumeration {

enum IDLE;

enum INACTIVE;

enum ACTIVE;

}

}

typedef Nrpci {

type uint32;

description "Physical Cell Identity (PCI) of the NR cell.";

reference "TS 36.211 subclause 6.11";

}

typedef Tac {

type string {

pattern "([A-Fa-f0-9]{4})|([A-Fa-f0-9]{6})" ;

}

description "Tracking Area Code

2 or 3-octet string identifying a tracking area code as specified in

clause 9.3.3.10 of 3GPP TS 38.413 [34], in hexadecimal representation.

Each character in the string shall take a value of

'0' to '9', 'a' to 'f' or 'A' to 'F' and shall represent 4 bits.

The most significant character representing the 4 most significant

bits of the TAC shall appear first in the string, and the character

representing the 4 least significant bit of the TAC shall appear last

in the string.

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

Examples:

A legacy TAC 0x4305 shall be encoded as '4305'.

An extended TAC 0x63F84B shall be encoded as '63F84B'";

reference "TS 23.003 clause 19.4.2.3";

}

grouping TaiGrp {

description "This <<dataType>> defines a Tracking Area Identity (TAI)

as specified in clause 28.6 of TS 23.003, clause 8.2 of TS 38.300

and clause 9.3.3.11 of TS 38.413. It is composed of the PLMN

identifier (PLMN-Id, which is composed of the MCC and MNC) and

the Tracking Area Code (TAC).";

list plmnId {

description "PLMN Identity.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf tac {

type Tac;

mandatory true;

description "Tracking Area Code";

}

}

grouping GeoCoordinateGrp {

description "Geographical location on earth";

leaf latitude {

type decimal64 {

fraction-digits 4;

range -90..90 ;

}

mandatory true;

description "Latitude based on World Geodetic System (1984 version)

global reference frame (WGS 84). Positive values correspond to the

northern hemisphere.";

}

leaf longitude {

type decimal64 {

fraction-digits 4;

range -180..180 ;

}

mandatory true;

description "Longitude based on World Geodetic System (1984 version)

global reference frame (WGS 84). Positive values correspond to

degrees east of 0 degrees longitude.";

}

leaf altitude {

type Altitude;

}

}

grouping GeoAreaGrp {

description "This data type defines a geographical area.

The geo-area is defined using a convex polygon in the attribute

'geoPolygon'.";

list geoPolygon {

description "Specifies the geographical area with a convex polygon.

The convex polygon is specified by its corners.";

key "latitude longitude";

min-elements 3;

ordered-by user;

uses GeoCoordinateGrp;

}

}

typedef AmfRegionId {

type union {

type uint8 ;

type string {

length 8;

pattern '[01]*';

}

}

reference "clause 2.10.1 of 3GPP TS 23.003";

}

typedef AmfSetId {

type union {

type uint16 {

range '0..1023';

}

type string {

length 8;

pattern '[01]*';

}

}

reference "clause 2.10.1 of 3GPP TS 23.003";

}

typedef AmfPointer {

type union {

type uint8 {

range '0..63';

}

type string {

length 6;

pattern '[01]*';

}

}

reference "clause 2.10.1 of 3GPP TS 23.003";

}

grouping AmfIdentifier {

leaf amfRegionId {

type AmfRegionId;

}

leaf amfSetId {

type AmfSetId;

}

leaf amfPointer {

type AmfPointer;

}

description "The AMFI is constructed from an AMF Region ID,

an AMF Set ID and an AMF Pointer.

The AMF Region ID identifies the region,

the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and

the AMF Pointer uniquely identifies the AMF within the AMF Set. ";

}

// type definitions especially for core NFs

typedef NfType {

type enumeration {

enum NRF;

enum UDM;

enum AMF;

enum SMF;

enum AUSF;

enum NEF;

enum PCF;

enum SMSF;

enum NSSF;

enum UDR;

enum LMF;

enum GMLC;

enum 5G_EIR;

enum SEPP;

enum UPF;

enum N3IWF;

enum AF;

enum UDSF;

enum BSF;

enum CHF;

}

}

typedef NotificationType {

type enumeration {

enum N1_MESSAGES;

enum N2_INFORMATION;

enum LOCATION_NOTIFICATION;

}

}

typedef Load {

description "Latest known load information of the NF, percentage ";

type uint8 {

range 0..100;

}

}

typedef N1MessageClass {

type enumeration {

enum 5GMM;

enum SM;

enum LPP;

enum SMS;

}

}

typedef N2InformationClass {

type enumeration {

enum SM;

enum NRPPA;

enum PWS;

enum PWS_BCAL;

enum PWS_RF;

}

}

grouping DefaultNotificationSubscription {

leaf notificationType {

type NotificationType;

}

leaf callbackUri {

type inet:uri;

}

leaf interPlmnCallbackUri {

description "It indicates the callback URI to be used by NF Service

Producers located in PLMNs that are different from the PLMN of

the NF consumer.";

type inet:uri;

}

leaf n1MessageClass {

type N1MessageClass;

}

leaf n2InformationClass {

type N2InformationClass;

}

leaf versions {

description "This attribute identifies the API versions

(e.g. v1) supported for the default notification type";

//optional

type N2InformationClass;

}

leaf binding {

description "This attribute shall contain the value of the Binding

Indication for the default subscription notification ";

reference " i.e. the value part of 3gpp-Sbi-Binding header, as

specified in

clause 6.12.4 of 3GPP TS 29.500 76. ";

//optional

type string;

config false;

}

leaf acceptedEncoding {

description "It indicates the content encodings that are accepted by a

NF Service Consumer when receiving a notification related to a default

notification subscription";

reference "IETF RFC 9110 clause 12.5.3 ";

//optional

type string;

config false;

}

leaf supportedFeatures {

description "It is a string, which indicates the features of the service

corresponding to the subscribed default notification, which are supported

by the NF (Service) instance acting as NF service consumer. The string

shall contain a bitmask indicating supported features in hexadecimal

representation:Each character in the string shall take a value

of 0 to 9, a to f or A to F ";

reference "shall represent the support of 4 features as described in

table 5.2.2-3 of TS 29.571 [61].";

//optional

type string;

config false;

}

leaf serviceInfoList {

description "It indicates a list of service specific information. It may

be present when the notification request of the notification type may be

generated by multiple services, i.e. notifications from different

services may be received by the subscription.";

//optional

type string;

config false;

}

leaf callbackUriPrefix {

description "It indicates the optional path segment(s) used to construct

the prefix of the Callback URIs during the reselection of an NF service

consumer, as described in 3GPP TS 29.501";

//optional

type inet:uri;

}

}

grouping Ipv4AddressRange {

leaf start {

type inet:ipv4-address;

}

leaf end {

type inet:ipv4-address;

}

}

grouping Ipv6PrefixRange {

leaf start {

type inet:ipv6-prefix;

}

leaf end {

type inet:ipv6-prefix;

}

}

typedef NsiId {

type string;

}

typedef UeMobilityLevel {

type enumeration {

enum STATIONARY;

enum NOMADIC;

enum RESTRICTED_MOBILITY;

enum FULLY_MOBILITY;

}

}

typedef ResourceSharingLevel {

type enumeration {

enum SHARED;

enum NOT_SHARED;

}

}

typedef TxDirection {

type enumeration {

enum DL;

enum UL;

enum DL_AND_UL;

}

}

grouping AddressWithVlan {

leaf ipAddress {

type inet:ip-address;

}

leaf vlanId {

type uint16;

}

}

/* DistinguishedName pattern is built up based on the

EBNF in 32.300 clause 7.3  EBNF of DN String Representation

leaf DN { type string {   //  Same pattern as LocalDN

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?(,[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)*';

} }

leaf fullLocalDN { type string {   // LocalRDN , { RDNSeparator , LocalRDN }    RDNSeparator is a single , no space or \R allowed   Me.mykey=1 allowed

//  (fullLocalRDN)(,(fullLocalRDN))*

pattern '(([A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*))=(([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?))(,(([A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*))=(([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)))*';

} }

leaf LocalDN { type string {   // LocalRDN , { RDNSeparator , LocalRDN }    RDNSeparator is a single , no space or \R allowed

//  LocalRDN(,LocalRDN)*

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?(,[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)*';

} }

leaf fullLocalRDN { type string {   // same as fullLocalDNAttributeTypeAndValue

pattern '([A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*))=(([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)';

} }

leaf LocalRDN { type string {   // same as LocalDNAttributeTypeAndValue

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?';

} }

leaf fullLocalDNAttributeTypeAndValue { type string { // LocalDNAttributeType , AttributeTypeAndValueSeparator , RegularAttributeValue

// pattern LocalDNAttributeType=RegularAttributeValue

pattern '([A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*))=(([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)';

} }

// limitation: NamesOfClassAndNamingAttributenot supported Me.mykey=1

leaf LocalDNAttributeTypeAndValue { type string {

// ebnf1          LocalDNAttributeType , AttributeTypeAndValueSeparator , RegularAttributeValue

// ebnf2-limited  NameOfClassWithIdAttribute , AttributeTypeAndValueSeparator , RegularAttributeValue

// pattern        NameOfClassWithIdAttribute=RegularAttributeValue

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?';

} }

leaf LocalDNAttributeType { type string {   // NameOfClassWithIdAttribute | NamesOfClassAndNamingAttribute  RDNSeparator is a single , no space or \R allowed

//  NameOfClassWithIdAttribute|NamesOfClassAndNamingAttribute

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*|([A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*)';

} }

leaf RegularAttributeValue { type string {       // ( AttributeValueChar - SpaceChar ) , [ { AttributeValueChar } , ( AttributeValueChar - SpaceChar ) ]

pattern '([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?' ;

} }

leaf NamesOfClassAndNamingAttribute  { type string {  // ClassName , ClassNamingAttributeSeparator , NamingAttributeName

// pattern: ClassName\.NamingAttributeName

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*\.[a-z][^,=+<>#;\\"\r\n*.]*' ;

} }

leaf restrictiveClassName { type string {     //

pattern '[a-zA-Z][a-zA-Z0-9-_]*' ;

} }

leaf ClassName { type string {     // CapitalLetterChar , { LocalDNAttributeTypeChar }

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*' ;

} }

leaf NamingAttributeName { type string {   // SmallLetterChar , { LocalDNAttributeTypeChar }

pattern '[a-z][^,=+<>#;\\"\r\n*.]*' ;

} }

*/

typedef DistinguishedName {

type string {

pattern '[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|'

+ '(\\[a-fA-F0-9]{2}))(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*'

+ '([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?'

+ '(,[A-Z][^,=+<>#;\\"\r\n*.]*=([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2}))'

+ '(([^,=+<>#;\\"\r\n*]|(\\[a-fA-F0-9]{2}))*'

+ '([^,=+<>#;\\"\r\n* ]|(\\[a-fA-F0-9]{2})))?)*';

}

description "Represents the 3GPP standard for DistinguishedName.

Limitations:

- RDNSeparator: don't allow SpaceChar or CarriageReturnChar

- NullDN: Disallow nullDN that is the same as not providing a DN

- NamesOfClassAndNamingAttribute format not allowed

(eg. ManagedElement.mykey=345436)";

reference  "3GPP TS 32.300";

}

typedef QOffsetRange  {

type int8 {

range "-24 | -22 | -20 | -18 | -16 | -14 | -12 | -10 | -8 | -6 | " +

" -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 8 | 10 | " +

" 12 | 14 | 16 | 18 | 20 | 22 | 24";

}

units dB;

}

grouping ReportingCtrl {

choice reportingCtrl {

mandatory true;

description "

This choice defines the method for reporting collected performance

metrics to MnS consumers as well as the parameters for configuring the

reporting function. It is a choice between the control parameter

required for the reporting methods, whose presence selects the

reporting method as follows:

- When only the fileReportingPeriod attribute is present, the MnS

producer shall store files on the MnS producer at a location selected

by the MnS producer and, on condition that an appropriate subscription

is in place, inform the MnS consumer about the availability of new

files and the file location using the notifyFileReady notification.

In case the preparation of a file fails, 'notifyFilePreparationError'

shall be sent instead.

- When the 'fileReportingPeriod' and 'notificationRecipientAddress'

attributes are present, then the MnS producer shall behave like

described for the case that only the 'fileReportingPeriod' is present.

In addition, the MnS producer shall create on behalf of the MnS

consumer a subscription, using 'NtfSubscriptionControl', for the

notification types 'notifyMOICreation' and 'notifyMOIDeletion' related

to the 'File' instances that will be produced later. In case an

existing subscription does already include the 'File' instances to be

produced, no new subscription shall be created. The

'notificationRecipientAddress' attribute in the created

'NtfSubscriptionControl' instance shall be set to the value of the

'notificationRecipientAddress' in the related 'PerfMetricJob'. This

feature is called implicit notification subscription, as opposed to the

case where the MnS consumer creates the subscription (explicit

notification subscription). When the related 'PerfMetricJob' is

deleted, the 'NtfSubscriptionControl' instance created due to the

request for implicit subscription shall be deleted as well.

- When only the 'fileReportingPeriod' and 'fileLocation' attributes

are present (CHOICE_3), the MnS producer shall store the files at the

location specified by 'fileLocation'. The file location may identify

any entity such as a file server or a MnS consumer. The identified

MnS consumer may or may not be identical to the MnS consumer creating

the 'PerfMetricJob'. As for CHOICE_1 the MnS producer may emit

'notifyFileReady' and 'notifyFilePreparationError' notifications to

inform subscribers that a file has been made available at the location

specified by 'fileLocation'.

- When only the streamTarget attribute is present, the MnS producer

shall stream the data to the location specified by streamTarget.

For the file-based reporting methods the fileReportingPeriod attribute

specifies the time window during which collected measurements are

stored into the same file before the file is closed and a new file is

opened.";

case file-based-reporting {

leaf fileReportingPeriod {

type uint32 {

range 1..max;

}

units minutes;

mandatory true;

description "For the file-based reporting method this is the time

window during which collected measurements are stored into the same

file before the file is closed and a new file is opened.

The time-period must be a multiple of the granularityPeriod.

Applicable when the file-based reporting method is supported.";

}

choice reporting-target {

case file-target {

leaf fileLocation {

type string ;

description "Applicable and must be present when the file-based

reporting method is supported, and the files are stored on the

MnS consumer.";

}

}

case notification-target {

leaf notificationRecipientAddress {

type string;

description "Must be present when the notification-based reporting

method is supported, and the the files are available as

notifications for the MnS consumer to subscribe to.";

}

}

description "When netiher fileLocation or notificationRecipientAddress

are present, the files are stored and available to the MnS consumer

if the MnS subscribes to the notifyFileReady notification.";

}

}

case stream-based-reporting {

leaf streamTarget {

type string;

mandatory true;

description "Applicable when stream-based reporting method is

supported.";

}

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 33 ***