3GPP TSG-SA5 Meeting #162	S5-253284

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.111 | CR | 0052 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.111 Remove notifyChangedAlarm |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | notifyChangedAlarm has already been deprecated in release-18. According to the lifecycle process it should be removed in the next release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove notifyChangedAlarm |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unneeded duplication in specification and implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 7.5.1, 8.4.2, 8.6.1, 8.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1840 at commit 736b4b690d522e75199a108dc5bc08fa18d836c1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 6.1	Solution components

The solution consists of the basic solution and the following optional solution components:

-	Dedicated perceived severity change notification

-	Acknowledging alarms by MnS consumers

-	Commenting alarms by MnS consumers

-	Alarm correlation

-	Reliability of alarm lists

Table 6.1-1: FM solution components

| Usage | Operations and notifications | NRM |
| --- | --- | --- |
| FM basic | notifyNewAlarm notifyChangedAlarmGeneral notifyClearedAlarm | AlarmList |
| Acknowledging alarms by MnS consumers | notifyAckStateChanged | alarmRecord.ackTime alarmRecord.ackUserId alarmRecord.ackSystemId alarmRecord.ackState |
| Commenting alarms by MnS consumers | notifyComments | alarmRecord:comments, datatype:alarmComment |
| Alarm correlation | notifyCorrelatedNotificationChanged | alarmRecord:correlatedNotifications alarmRecord:rootCauseIndicator |
| Reliability of alarm lists | notifyPotentialFaultyAlarmList notifyAlarmListRebuilt | AlarmList.unreliableAlarmScope |

Next change

### 7.5.1	Alarm notifications

This clause presents a list of notifications, defined in clause 8, that a MnS consumer can receive. The notification header attribute objectClass/objectInstance captures the DN of an instance of an IOC defined in the present document.

| Name | Notes |
| --- | --- |
| notifyNewAlarm |  |
| notifyClearedAlarm |  |
| notifyAlarmListRebuilt |  |
| notifyChangedAlarmGeneral |  |
| notifyCorrelatedNotificationChanged |  |
| notifyAckStateChanged |  |
| notifyComments |  |
| notifyPotentialFaultyAlarmList |  |

Next change

### 8.4.2	Input parameters

Table 8.4.2-1: Input parameters for notifyChangedAlarmGeneral

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See TS 28.532 [2] clause 11.0.2 |  |
| objectInstance | M | alarmRecord.objectInstance DN of the MonitoredEntity that is the source of the alarm |  |
| notificationId | M | See TS 28.532 [2] clause 11.0.2 |  |
| notificationType | M | "notifyChangedAlarmGeneral" |  |
| eventTime | M | alarmRecord.alarmChangedTime |  |
| systemDN | M | See TS 28.532 [2] clause 11.0.2 |  |
| sequenceNo | CM | See TS 28.532 [2] clause 11.0.2 |  |
| subscriptionId | CM | See TS 28.532 [2] clause 11.0.2 |  |
| alarmId | M | alarmRecord.alarmId |  |
| alarmType | M | alarmRecord.alarmType |  |
| probableCause | M | alarmRecord.probableCause |  |
| specificProblem | O | alarmRecord.specificProblem |  |
| perceivedSeverity | M | alarmRecord.perceivedSeverity | Value shall not be "CLEARED" |
| backedUpStatus | CO | alarmRecord.backedUpStatus | Used only in non-security notifications. |
| backUpObject | CO | alarmRecord.backUpObject | Used only in non-security notifications. |
| trendIndication | CO | alarmRecord.trendIndication | Used only in non-security notifications. |
| thresholdInfo | CO | alarmRecord.thresholdInfo | Used only in non-security notifications. |
| correlatedNotifications | O | alarmRecord.correlatedNotifications |  |
| stateChangeDefinition | CO | alarmRecord.stateChange | Used only in non-security notifications. |
| monitoredAttributes | CO | alarmRecord.monitoredAttributes | Used only in non-security notifications. |
| proposedRepairActions | CO | alarmRecord.proposedRepairActions | Used only in non-security notifications. |
| additionalText | O | alarmRecord.additionalText |  |
| additionalInformation | O | alarmRecord.additionalInformation |  |
| rootCauseIndicator | O | alarmRecord.rootCauseIndicator |  |
| serviceUser | CM | alarmRecord.securityServiceUser | Available if security alarms are supported. Used only in security notifications. This may contain no information if the identify of the service-user (requesting the service) is not known. |
| serviceProvider | CM | alarmRecord.securityServiceProvider | Available if security alarms are supported. Used only in security notifications. This shall always identify the service-provider receiving a service request, from serviceUser, that provokes the security alarm. |
| securityAlarmDetector | CM | alarmRecord.securityAlarmDetector | Available if security alarms are supported. Used only in security notifications. This may contain no information if the detector of the security alarm is the serviceProvider. |
| changedAlarmAttributes | O | LIST OF SEQUENCE <AttributeName, OldAttributeValue> | The changed alarm attributes (name/value pairs) (with old values). |

Next change

### 8.6.1	Definition

The notification was removed. Its functionality is replaced by notifyChangedAlarmGeneral.

### 8.6.2	Void

End of  changes

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1840 at commit 736b4b690d522e75199a108dc5bc08fa18d836c1

*** START OF CHANGE 1 ***

*** OpenAPI/TS28111_FaultNotifications.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Fault Management Notifications

version: 18.1.0

description: >-

OAS 3.0.1 definition of the Fault Supervision MnS

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.111; Fault Management

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.111/

servers:

- url: '{notificationRecipientAddress}'

variables:

notificationRecipientAddress:

description: Address on the MnS consumer side defined by NtfSubscriptionControl.notificationRecipientAddress. See 3GPP TS 28.622 clause 4.3.22

default: http://exampleConsumer.com/notificationReceiver

MnSversion:

description: Version number of the OpenAPI definition

default: XXX

paths:

/:

post:

summary: Receive one of a set of alarm related notifications

description: >-

The MnS producer sends an alarm to the consumer.

This interface shall be implemented by the notification-receiver and

used by the notification-sender.

requestBody:

required: true

content:

application/json:

schema:

oneOf:

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/NotifyNewAlarm'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/NotifyNewSecAlarm'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/NotifyClearedAlarm'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/NotifyChangedAlarmGeneral'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/NotifyChangedSecAlarmGeneral'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/NotifyCorrelatedNotificationChanged'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/NotifyAckStateChanged'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/NotifyComments'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/NotifyPotentialFaultyAlarmList'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/NotifyAlarmListRebuilt'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

<CODE ENDS>

*** END OF CHANGE 1 ***

*** START OF CHANGE 2 ***

*** OpenAPI/TS28111_FaultNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Fault Management NRM

version: 19.2.0

description: >-

OAS 3.0.1 definition of the Fault Supervision MnS

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.111; Fault Management

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.111/

servers:

- url: '{MnSRoot}/FaultSupervisionMnS/{MnSversion}'

variables:

MnSRoot:

description: See subclause 4.4.3 of TS 32.158

default: http://example.com/3GPPManagement

MnSversion:

description: Version number of the OpenAPI definition

default: XXX

paths: {}

components:

schemas:

#---- Definition of AlarmRecord ----------------------------------------------------#

AlarmId:

type: string

AlarmType:

type: string

enum:

- COMMUNICATIONS_ALARM

- QUALITY_OF_SERVICE_ALARM

- PROCESSING_ERROR_ALARM

- EQUIPMENT_ALARM

- ENVIRONMENTAL_ALARM

- INTEGRITY_VIOLATION

- OPERATIONAL_VIOLATION

- PHYSICAL_VIOLATION

- SECURITY_SERVICE_OR_MECHANISM_VIOLATION

- TIME_DOMAIN_VIOLATION

- OTHER

readOnly: true

ProbableCause:

description: >-

The value of the probable cause may be a specific standardized string, or any

vendor provided string. Probable cause strings are not standardized in the

present document. They may be added in a future version. Up to then the

mapping of the generic probable cause strings "PROBABLE_CAUSE_001" to

"PROBABLE_CAUSE_005" is vendor specific.

The value of the probable cause may also be an integer. The mapping of integer

values to probable causes is vendor specific.

oneOf:

- anyOf:

- type: string

enum:

- PROBABLE_CAUSE_001

- PROBABLE_CAUSE_002

- PROBABLE_CAUSE_003

- PROBABLE_CAUSE_004

- PROBABLE_CAUSE_005

readOnly: true

- type: string

readOnly: true

- type: integer

readOnly: true

SpecificProblem:

oneOf:

- type: string

readOnly: true

- type: integer

readOnly: true

PerceivedSeverity:

type: string

enum:

- INDETERMINATE

- CRITICAL

- MAJOR

- MINOR

- WARNING

- CLEARED

TrendIndication:

type: string

readOnly: true

enum:

- MORE_SEVERE

- NO_CHANGE

- LESS_SEVERE

ThresholdHysteresis:

type: object

required:

- high

properties:

high:

oneOf:

- type: integer

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/FloatRo'

low:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/FloatRo'

ThresholdLevelInd:

oneOf:

- type: object

properties:

up:

$ref: '#/components/schemas/ThresholdHysteresis'

- type: object

properties:

down:

$ref: '#/components/schemas/ThresholdHysteresis'

ThresholdCrossing:

type: object

properties:

observedMeasurement:

type: string

readOnly: true

observedValue:

type: number

readOnly: true

thresholdLevel:

$ref: '#/components/schemas/ThresholdLevelInd'

armTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

required:

- observedMeasurement

- observedValue

CorrelatedNotification:

type: object

properties:

sourceObjectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

notificationIds:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

required:

- sourceObjectInstance

- notificationIds

CorrelatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

AckState:

type: string

enum:

- ACKNOWLEDGED

- UNACKNOWLEDGED

AlarmRecord:

description: >-

The alarmId is not a property of an alarm record. It is used as key

in the map of alarm records instead.

type: object

properties:

# alarmId:

#  $ref: '#/components/schemas/AlarmId'

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

notificationId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

alarmRaisedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

alarmChangedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

alarmClearedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdinfo:

$ref: '#/components/schemas/ThresholdCrossing'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

stateChangeDefinition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

readOnly: true

additionalText:

type: string

readOnly: true

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

readOnly: true

ackTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

ackUserId:

type: string

ackSystemId:

type: string

ackState:

$ref: '#/components/schemas/AckState'

clearUserId:

type: string

clearSystemId:

type: string

serviceUser:

type: string

readOnly: true

serviceProvider:

type: string

readOnly: true

securityAlarmDetector:

type: string

readOnly: true

AlarmList-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

numOfAlarmRecords:

type: integer

readOnly: true

lastModification:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

alarmRecords:

description: >-

This resource represents a map of alarm records.

The alarmIds are used as keys in the map.

type: object

additionalProperties:

$ref: '#/components/schemas/AlarmRecord'

unreliableAlarmScope:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

#---- Definition of alarm notifications --------------------------------------------#

AlarmNotificationTypes:

type: string

enum:

- notifyNewAlarm

- notifyChangedAlarmGeneral

- notifyAckStateChanged

- notifyCorrelatedNotificationChanged

- notifyComments

- notifyClearedAlarm

- notifyAlarmListRebuilt

- notifyPotentialFaultyAlarmList

AlarmListAlignmentRequirement:

type: string

enum:

- ALIGNMENT_REQUIRED

- ALIGNMENT_NOT_REQUIRED

NotifyNewAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdInfo:

$ref: '#/components/schemas/ThresholdCrossing'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

stateChangeDefinition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

NotifyNewSecAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- serviceUser

- serviceProvider

- securityAlarmDetector

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

NotifyClearedAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

clearUserId:

type: string

clearSystemId:

type: string

NotifyChangedAlarmGeneral:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdInfo:

$ref: '#/components/schemas/ThresholdCrossing'

stateChangeDefinition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

changedAlarmAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyChangedSecAlarmGeneral:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- serviceUser

- serviceProvider

- securityAlarmDetector

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

changedAlarmAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyCorrelatedNotificationChanged:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- correlatedNotifications

- alarmType

- probableCause

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

rootCauseIndicator:

type: boolean

NotifyAckStateChanged:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- ackState

- ackUserId

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

ackState:

$ref: '#/components/schemas/AckState'

ackUserId:

type: string

ackSystemId:

type: string

NotifyComments:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- comments

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

comments:

$ref: '#/components/schemas/Comments'

NotifyPotentialFaultyAlarmList:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- reason

properties:

reason:

type: string

NotifyAlarmListRebuilt:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- reason

properties:

reason:

type: string

alarmListAlignmentRequirement:

$ref: '#/components/schemas/AlarmListAlignmentRequirement'

#---- Definition of resources ------------------------------------------------------#

Comment:

type: object

properties:

commentTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

commentUserId:

type: string

commentSystemId:

type: string

commentText:

type: string

Comments:

description: >-

Collection of comments. The comment identifiers are allocated by the

MnS producer and used as key in the map.

type: object

additionalProperties:

$ref: '#/components/schemas/Comment'

#----- Definitions in TS 28.111 for TS 28.532 --------------------------#

resources-faultNrm:

oneOf:

- $ref: '#/components/schemas/AlarmList-Single'

#----- Definitions in TS 28.111 for TS 28.532 --------------------------#

<CODE ENDS>

*** END OF CHANGE 2 ***