3GPP TSG SA WG5 #162							S5-253296

Goteborg, Sweden, August 25-29, 2025

3GPP TSG RAN WG3 Meeting #128	R3-253958

Malta, MT, 19-23 May 2025

Title:	Reply LS on Continuous MDT

Response to:	R3-253024/ S5-252114

Release:	Rel-19

Work Item:	NR_AIML_NGRAN_enh-Core

Source:	RAN3

To:	SA5

Cc:

Contact Person:

#### Name:	Angelo Centonza

E-mail Address:	angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

# Overall Description

RAN3 would like to thank SA5 for the reply LS on Continuous MDT. In the Reply LS, SA5 provided responses to the set of questions posed by RAN3 on guidance on possible solutions for Continuous Management Based MDT. Based on the replies from SA5, RAN3 had a discussion and agreed to the following principles:

- Information from OAM to the NG-RAN nodes participating to Continuous MDT, allowing for the identification of a continuous MDT process, are provided e.g. in the form of specific Trace Reference(s).

- Solutions for Continuous MDT shall not have an impact on the 5GC in Rel19 from a RAN3 perspective.

- RAN3 agrees to use TR(s) and TRSR(s) to resolve the MDT measurement correlation problem

Concerning the clarifications requested by SA5 on the Measurement Continuity Problem across different RRC states, RAN3 would like to clarify that the Measurement Continuity Problem across different RRC states refers to how to enable correlation of management-based MDT measurements collected while a UE was in RRC_Connected, RRC_Inactive and RRC_Idle.

In one of their replies SA5 stated:

Answer: SA5 agrees that at lease TR and TRSR (which does not revealing UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

In response to the question from SA5 to elaborate the details of how to avoid overlapping, RAN3 has discussed based on available solutions and provides the following details:

- In some solutions, a dedicated TRSR range is assigned to each NG-RAN node participating in Continuous MDT. When an NG-RAN node selects a UE for Continuous MDT (not configured so far with Continuous MDT), the NG-RAN node assigns a TRSR from such range to the UE. The same TRSR will be used for the whole duration of the Continuous MDT trace of a UE for correlation purposes. Namely, the TRSR is kept for the UE even if the UE moves to a different participating node via handover (the TR and TRSR are signalled from source NG-RAN node to target NG-RAN node) or if the UE is configured with Logged MDT and it moves to RRC_Idle/RRC_Inactive. Trace Correlation is achieved by the assigned TR/TRSR which can identify that the trace records belong to the same selected UE. Overlapping is avoided because the same TRSR from the TRSR range assigned to the NG-RAN node that initiated the Continuous MDT process will be used for the whole duration of the Continuous management-based MDT trace of a UE. Or

- In some other solutions, RAN3 assumes that the problem of overlapping does not occur because OAM configures gNBs in an area with same immediate MDT configuration (TR1) and same logged MDT configuration (TR2). A gNB chooses a UE for Continuous MDT and configures the UE for Immediate MDT and Logged MDT separately (with separate TR and may be also different TRSRs, i.e. TR1/TRSR1 and TR2/TRSR2). At connected-to-idle transition the serving gNB sends the Immediate MDT report (TR1/TRSR1) to TCE by also including the TR2/TRSR2 assigned for the Logged MDT. At idle-to-connected transition the (new) serving gNB sends the Logged MDT report (TR2/TRSR2) to TCE by also including the TR1/TRSR3 assigned for the next Immediate MDT. Or,

- In some other solutions, RAN3 assumes that the problem of overlapping does not occur because OAM configures gNBs in an area with same configuration (single TR) encompassing both immediate MDT and logged MDT. A gNB chooses a UE for Continuous MDT and configures the UE for Immediate MDT and Logged MDT with the same TR and TRSR (TRSR1). If at idle-to-connected, or at handover the new serving gNB cannot re-assign TRSR1 to the UE, it will assign a new TRSR (TRSR2) and include both the previously assigned TRSR (TRSR1) and the new TRSR (TRSR2) in the Logged MDT (TR & TRSR1/TRSR2) or Immediate MDT report (TR & TRSR1/TRSR2) to TCE.

RAN3 would like to ask SA5 to take the above agreements and descriptions from RAN3 into account and to specify a solution for continuous MDT accordingly. RAN3 asks SA5 to check the security aspects of the Continuous MDT solution derived.

# 2. Actions

To SA5

ACTION: RAN3 kindly asks SA5 to check the feasibility of the principles agreed by RAN3, to specify a solution that reflects such principles (if feasible) and to provide feedback to RAN3.

# 3. Date of Next TSG-RAN3 Meetings

TSG-RAN WG3 Meeting RAN3#129			25 - 29 August 2025		India (TBC), IN

TSG-RAN WG3 Meeting RAN3#129-bis			13 - 17 October 2025		Prague, CZ