3GPP TSG-SA5 Meeting #161	S5-253xx2

Fukuoka, Japan, 19 - 23 May 2025

Source:	Nokia

Title:	Pseudo-CR on NRM for Collaboration between NDTs

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS28.561

Version:	0.2.0

Work Item:	Management aspects of Network Digital Twins

Comments

This pCR is to add NRM for the NDT  following agreements in the NDT study.

Proposed Changes

* * * First Change * * * *

# 6	Management service for NDT (stage 2)

## 6.1	Management operation for NDT (MnS component type A)

The operations (e.g., createMOI operations) and notifications (e.g., notifyMOIcreation) of generic provisioning MnS defined in 3GPP TS 28.532 [8] can be used for NDT management, including NDT function management, NDT job management and NDT report management. The NDTFunction, NDTJob, NDTReport instances can be treated as Managed Object instances.

Following is the IS to support NDT lifecycle management:

Table 6.1-1: IS to support NDT function management

| NDT function management | IS operation |
| --- | --- |
| Query an NDT function | getMOIAttributes operation |

Following is the IS to support NDT job management:

Table 6.1-2: IS to support NDT job management

| NDT job management | IS operation |
| --- | --- |
| Create an NDT job | createMOI operation |
| Delete an NDT job | deleteMOI operation |
| Modify an NDT job | modifyMOIAttributes operation |
| Query an NDT job | getMOIAttributes operation |

Following is the IS to support NDT report management:

Table 6.1-3: IS to support NDT report management

| NDT lifecycle management | IS operation |
| --- | --- |
| Query an NDT report | getMOIAttributes operation |
| Subscribe an NDT report | createMOI operation |
| Notify an NDT report | notifyMOIAttributeValueChanges notification |
| Unsubscribe an NDT report | deleteMOI operation |
| Query an NDT report subscription | getMOIAttributes operation |

## 6.2	Information model for NDT (MnS component typeB)

### 6.2.1	Information model definition

#### 6.2.1.1	Imported information entities and local labels

| 3GPP TS 28.622 [7], DataType, GeoArea | GeoArea |
| --- | --- |
| 3GPP TS 28.622 [7], DataType, TimeWindow | TimeWindow |
| 3GPP TS 28.622 [7], DataType, ThresholdInfo | ThresholdInfo |
| 3GPP TS 28.622 [7], ConditionMonitor | ConditionMonitor |

#### 6.2.1.2	Class diagram

##### 6.2.1.2.1	Relationship

Figure 6.2.1.2.1-1: Relationship UML diagram for NDT management

##### 6.2.1.2.2	Inheritance

Figure 6.2.1.2.2-1: Inheritance UML diagram for NDT management

#### 6.2.1.3	Class definition

##### 6.2.1.3.1	NDTFunction <<InformationObjectClass>>

###### 6.2.1.3.1.1	Definition

This IOC represents the properties of an NDT Function.

The NDTFunction includes a reference to one or more NDTFunction instances which act as component NDTs contributing to the functionality of the NDTFunction.

###### 6.2.1.3.1.2	Attributes

The NDTFunction IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

The NDTFunction has a relationship indicating that an NDT Function may be associated with one or more other NDT Functions. For example, one or more NDT Functions with small scope might support the operation of an NDT Function with a wider scope.

Table 6.2.1.3.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedNDTCapabilities | M | T | F | T | T |
| nDTFunctionScope | M | T | F | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtRef | M | T | T | F | T |

###### 6.2.1.3.1.3	Attribute constraints

None.

###### 6.2.1.3.1.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

##### 6.2.1.3.2	NDTJob <<InformationObjectClass>>

###### 6.2.1.3.2.1	Definition

This IOC represents the properties of an NDT job demand created by an MnS consumer.

The attribute "nDTJobSynchScope” indicates the scope of the network that should be synchronized into and modelled by the NDT for the specific NDT job. If the NDTjob is not meant to synchronize with the network, no cnten tis provided in the nDTJobSynchScope attribute.

The attribute "ndtJobScenario" indicates the input that is defined by MnS consumer for the characteristics of network objects that should be simulated/emulated by NDT. If the NDT is able to synchronize with an actual network, the ndtJobScenario indicates the delta between the actual network and twin network that is simulated/emulated. Otherwise, it indicates the critical features that should be modelled, allowing the NDT to use defaults for all other features. The ndtJobScenario can be network configurations or automation functionality configurations, network events, issues that are defined by MnS consumer and will be injected to NDT.

The ndtJobScenario also captures requirements to be simulated to see the network’s response to specific network events or issues. The network issue or events that ned to be simulated/emulated (including configuration, performance and fault characteristics may result in a particular network issue) are added into the networkResponseTask attribute.

The attribute "ndtJobExecutionRequirements" represents the execution-related requirements for an NDT job, e.g., maximum run time for each simulation/emulation task, precision, etc which are used to select the model parameters (e.g., simulation/emulation step and number of simulation/emulation times) for building NDT models. It is up to implementation how the NDT model is built and used to execute the simulation/emulation task.

NOTE: the model for the specific tasks can be extended as needed.

###### 6.2.1.3.2.2	Attributes

The NDTJob IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTCapability | M | T | T | F | T |
| nDTJobSynchScope | M | T | T | F | T |
| ndtJobScenario | M | T | T | F | T |
| ndtJobExecustionRequirements | O | T | T | F | T |
| collaboratingNDT | O | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtReportRefList | M | T | T | F | T |

###### 6.2.1.3.2.3	Attribute constraints

None.

###### 6.2.1.3.2.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

##### 6.2.1.3.3	NDTFunctionScope << dataType >>

###### 6.2.1.3.3.1	Definition

This IOC represents the properties of a scope that can be modelled by an NDT

###### 6.2.1.3.3.2	Attributes

The NDTFunctionScope IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTRANScope | M | T | T | F | T |
| nDTCNScope | M | T | T | F | T |

###### 6.2.1.3.3.3	Attribute constraints

None.

###### 6.2.1.3.3.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

##### 6.2.1.3.4	NDTInputDescription<<dataType>>

###### 6.2.1.3.4.1	Definition

This dataType represents a description of the network. It may be used to describe any of the following:

-	aspects of the network that should be modelled in the NDT,

-	configurations that should be applied or have been applied by the NDT

The objects to be considered may be described in terms of the attribute networkObjectType or by specific network object instances represented by objectInstance. If a list of specific managed objects are to be modelled, they are listed in the attribute objectInstance. Otherwise, their type if listed in the attribute networkObjectType and their specific characteristics listed in objectAttributeList.

###### 6.2.1.3.4.2	Attributes

The NDTInputDescription  <<datatype>> includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.4.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTInputDescriptionId | M | T | T | F | T |
| simulationData | O | T | T | F | T |
| networkEventInfo | O | T | T | F | T |
| condition | M | T | T | F | T |

###### 6.2.1.3.4.3	Attribute constraints

None

###### 6.2.1.3.4.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

##### 6.2.1.3.5	NDTOutputDescription <<dataType>>

###### 6.2.1.3.5.1	Definition

This dataType represents a description of the individual outputs of the NDT modelled twin. It may be used to describe any of the following:

-	states of the network that have been modelled and are being reported by the NDT,

-	characteristics of the network that are being reported by the NDT.

The objects that have been modelled are described by specific network object instances represented by objectInstance and their specific characteristics (PMs, alarms, etc.) listed in objectAttributeList.

###### 6.2.1.3.5.2	Attributes

The NDTInputDescription  <<datatype>> includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.5.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTInputDescriptionId | M | T | T | F | T |
| objectInstance | M | T | T | F | F |
| objectAttributeList | M | T | T | F | T |

###### 6.2.1.3.5.3	Attribute constraints

None

###### 6.2.1.3.5.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

##### 6.2.1.3.6	ScopeDefinition <<choice>>

###### 6.2.1.3.6.1	Definition

This <<choice>> represents a definition of a scope.

The managedEntitiesScope attribute identifies the scope by the DNs of the managed entities.

The areaScope attribute identify the scope by the geographical area information.

The managedEntitiesScope attribute and areaScope attribute shall not be present at the same time.

###### 6.2.1.3.6.2	Attributes

Table 6.2.3.6.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Choice_1 managedEntitiesScope | CM | T | T | F | T |
| Choice_2 areaScope | CM | T | T | F | T |

###### 6.2.1.3.6.3	Attribute constraints

Table 6.2.1.3.6.3-1

| Name | Definition |
| --- | --- |
| _1 managedEntitiesScope | the MnS producer supports identifying the scope by managed entities. |
| _2 areaScope | the MnS producer supports identifying the scope by geographical area information. |

###### 6.2.1.3.6.4	Notifications

None.

##### 6.2.1.3.7	NdtJobExecutionReqts <<dataType>>

###### 6.2.1.3.7.1	Definition

The NdtJobExecutionReqs <<dataType>> represents the execution requirements for NDT job.

An NDT may for a defined scenario evaluate the secnarion several times. The attribute iterations indicates the number of times of times a defined scenario may be executed.

###### 6.2.1.3.7.2	Attributes

The NdtJobExecutionReqs includes the following attributes.

Table 6.2.1.3.7.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| maxRuntime | O | T | T | F | F |
| iterations | O | T | T | F | F |

###### 6.2.1.3.7.3	Attribute constraints

None

###### 6.2.1.3.7.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

##### 6.2.1.3.8	NDTReport <<InformationObjectClass>>

###### 6.2.1.3.8.1	Definition

This IOC represents the properties of an NDT report corresponding to an NDT job.

An NDT job may run more than one task at the same time, e.g.,  a network configuration task and a network response task. The NDTReport contains an output for each task that is executed by the NDT job.

The attribute "NDTJobOutputData"  specified performance metrics and/or alarm types that are collected and reported by NDT after the behaviour is modelled in NDT is put in a list for which each entry is an NDTOutputDataPoint.

###### 6.2.1.3.8.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.8.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTJobOutputData | M | T | F | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtJobRef | M | T | F | F | T |
| ndtFunctionRef | M | T | F | F | T |

###### 6.2.1.3.8.3	Attribute constraints

None.

###### 6.2.1.3.8.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

##### 6.2.1.3.9	NDTOutputDataPoint <<dataType>>

###### 6.2.1.3.9.1	Definition

This dataType represents a single output of the NDT.

###### 6.2.1.3.9.2	Attributes

The nDTOutputDataPoint <<datatype>> includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

The nDTOutputDataPoint indicates three different types of information about the network. The networkState indicates the state of the network at which a specific networkConfiguration (i.e. a delta of the network state or new state of the netwrok) is applied. The observations shows the outcome on the network when the stated networkConfiguration is applied in the stated networkState.

Table 6.2.1.3.9.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| networkState | M | T | F | T | T |
| networkConfiguration | M | T | F | T | T |
| observations | M | T | F | T | T |

###### 6.2.1.3.9.3	Attribute constraints

None.

###### 6.2.1.3.9.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType .

## 6.3	Attribute definitions

6.3.1	Attribute properties

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ndtJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtFunctionRef | It indicates an DN of a NDTFunction Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtReportRefList | It indicates a list of DN for NDTReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedNDTCapabilities | It indicates the different types of NDT application use cases which the NDT is capable of undertaking. allowedValues: "RISKY-ACTIONS_PREDICTION", "EVENTS-IMPACTS_VERIFICATION", "FAULT_INJECTION", "NETWORK_EVENTS_VERIFICATION" "NETWORK_CONFIGURATIONS_VERIFICATION", "AUTOMATION_CONFIGURATION_VERIFICATION" "ML-TRAINING_DATA_GENERATION", "USER_EXPERIENCE_DATA_GENERATION" New values can be added to this list in future releases to support new use cases. The meaning of these values is as follows: "RISKY-ACTIONS_PREDICTION" means NDTFunction supports the use case described in 5.2.2.2. "EVENTS-IMPACTS_VERIFICATION" means NDTFunction supports the use case described in 5.2.2.3. "FAULT_INJECTION" means NDTFunction supports the use case described in 5.2.2.4. "NETWORK_EVENTS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.2. "NETWORK_CONFIGURATIONS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.3. "AUTOMATION_CONFIGURATION_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.4. "ML-TRAINING_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.2. "USER_EXPERIENCE_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.3. | type: ENUM multiplicity:1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCapability | It indicates the type of application use cases that is desired to be executed. allowedValues: "RISKY-ACTIONS_PREDICTION", "EVENTS-IMPACTS_VERIFICATION", "FAULT_INJECTION", "NETWORK_EVENTS_VERIFICATION" "NETWORK_CONFIGURATIONS_VERIFICATION", "AUTOMATION_CONFIGURATION_VERIFICATION" "ML-TRAINING_DATA_GENERATION", "USER_EXPERIENCE_DATA_GENERATION" New values can be added to this list in future releases to support new use cases. The meaning of these values is as follows: "RISKY-ACTIONS_PREDICTION" means NDTFunction supports the use case described in 5.2.2.2. "EVENTS-IMPACTS_VERIFICATION" means NDTFunction supports the use case described in 5.2.2.3. "FAULT_INJECTION" means NDTFunction supports the use case described in 5.2.2.4. "NETWORK_EVENTS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.2. "NETWORK_CONFIGURATIONS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.3. "AUTOMATION_CONFIGURATION_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.4. "ML-TRAINING_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.2. "USER_EXPERIENCE_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.3. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTRANScope | It indicates the scope of the RAN that can be modelled by the NDT function. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCNScope | It indicates the scope of the CN that can be modelled by the NDT function. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTJobSynchScope | It indicates the scope of the network that should be synchronized into and modelled by the NDT for the specific NDT job. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ndtJobScenario | It indicates a network scenario that should be modelled in the NDTJob as an extra beyond what is synchronized from the network. The ndtJobScenario can be used for - Verification of network response to one or more events - evaluation of the impact of one or more failure events, e.g. a signalling storm - Evaluating one or more network issues, e.g. a coverage issue. The network issues involve one or more network events. - Evaluation of high-risk network operations which are listed within the planned configuration - Verification of network configurations which are listed within the planned configuration - Using NDT to generate ML training data - Using NDT to generate user experience data | type: NDTInputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NDTInputDescriptionId | It indicates the identifier for a specific input to be modelled in the NDTJob | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NDTInputDescriptionId | It indicates the identifier for a specific output provided as outcomes of the NDTJob | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkEveFntInfo | This defines the information related with a network event (a provisioning, performance measurement, KPI or fault/ alarm event) that can be introduced into the NDT. The NetworkEventData can be used for - Verification of network response to one or more events - evaluation of the impact of one or more failure events, e.g. a signalling storm - Evaluating one or more network issues, e.g. a coverage issue. The network issues involve one or more network events. Editor’s note: The definition and modelling of networkEventInfo is to be clarified | Type: TBD multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| simulationData | This will define which management data is to be updated artificially in order to induce a particular network issue. The management data includes: - Performance data: The name of the performance measurement or the KPI as defined in 3GPP TS 28.552 and TS 28.554 - MDT/Trace data: The name of MDT measurements as defined in 3GPP TS 32.422 - Configuration data: The name of the attribute from any of the available MOIs. Editor’s note: The definition and modelling of simulationData is to be clarified | type: TBD multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| condition | This will define the condition that has to be satisfied in order to update the simulation data for the task that is executed by the NDT. This can be defined in terms of location and time. This will be the DN of ConditionMonitor[7]. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtJobExecustionRequirements | It describes the performance requirements for NDT modelling, e.g., maximum run time for each simulation/emulation job, precision, etc | type: NdtJobExecutionReqts multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| NDTJobOutputData | It indicates the list of NDTOutput(s) that are provided by the NDT function as the output for any task executed in an instantiated NDT job. | type: NDTOutputDataPoint multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| maxRuntime | Maximum run time for each simulation task executed in NDT Editor’s note: The unit of this attribute is to be added | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| iterations | It indicates the number of times of times a defined scenario may be executed. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| networkState | It indicates a state of the twin network (the modelled network in the NDT) for which a configuration or reconfiguration is applied. The networkState is the desvription of what exists in the network at the time when the networkConfiguration is made | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkConfiguration | It indicates a network configuration that is executed by the NDT and being reported in the NDT report | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| observations | It indicates an impact on the network. It shows the list of network objects that are affected and the effects on the specific objects | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| collaboratingNDT | It indicates the related NDT Job contributing as a collaborator to the executed NDT Job. It describes a relationship to an NDT job, i.e. it indicates the DN of a component NDT which provides input to the NDT job | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * End of Changes * * * *