3GPP TSG-SA5 Meeting #162	S5-253410

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0229 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.104 Correction on AIML descriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 28.105 removed the terminology ML training by using ML model training in Rel-19. This CR aims to align those changes with TS 28.104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To use term ML model training instead of ML training. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in terminology for ML model training. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.9, 8.4.10, 8.5.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

7.2.9	Correlation analytics of Management data

7.2.9.1	Measurement data correlation analytics for ML model training

7.2.9.1.1	Description

This MDA capability is for correlation analysis of Measurement data.

7.2.9.1.2	Use case

For ML model training, collecting a large volume of measurement data instances does not necessarily enhance training performance. The measurement data collected for ML model training may exhibit high correlation (linear or non-linear), resulting in significant redundancy. Consequently, using the entire dataset for model training can lead to unnecessary consumption of computational resources and energy.

Optimizing training data preparation based on correlation analysis and redundancy information can be very helpful. Correlation analysis can identify redundancy patterns within the measurement data for ML model training, enabling more efficient model training. This may be achieved in the following ways:

-	For a given task (e.g. analytics, model training), correlation analysis can identify relationships among the data, resulting in a reduced dataset that can be used for training the ML model. This approach may improve training efficiency while managing the impact on model performance compared to using the full dataset. The analysis may also provide recommendations, such as optimizing data collection for training purposes.

-	Regularly updating the correlation analytics may be required, as correlation relationships may evolve over time. This is very useful when there is a recurring need to re-train the ML model.

7.2.9.1.3	Requirements

Table 7.2.9.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-DATA-CORRELATION-1 | MDA capability for data correlation analytics for ML model training should include a capability to provide the measurement data redundancy analysis including which measurement data correlate to which measurement data, the indication of redundancy, and recommendation to optimize training data collection. | Measurement data correlation analytics for ML model training (clause 7.2.9.1) |

Next change

8.4.10	Correlation analytics

8.4.10.1	Measurement data correlation analytics for ML model training

8.4.10.1.1	MDA type

The MDA type for ML model training data Correlation analysis is: CorrelationAnalytics.TrainingDataAnalysis.

8.4.10.1.2	Enabling data

The enabling data for CorrelationAnalytics.TrainingDataAnalysis MDA type are provided in table 8.4.10.1.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.10.1.2-1: Enabling data for ML model training data Correlation analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | The performance metric or KPI data used for ML model training. | The performance metric data defined in TS 28.552 [4], and KPI data defined in TS 28.554 [5] |

8.4.10.1.3	Analytics output

The specific information elements of the analytics output for management data correlation analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.10.1.3-1.

Table 8.4.10.1.3-1: Analytics output for correlation analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| measurementDataCorrelationRecommendation | The attribute indicates measurement data correlation analytics recommendation. This attribute may carry null value, which may indicate no recommendation. allowedValues: N/A | M | type: MeasurementDataCorrelationRecommendation multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

Next change

### 8.5.24	MeasurementDataCorrelationRecommendation <<dataType>>

#### 8.5.24.1	Definition

This data type specifies the measurement data correlation analytics recommendation. The data type can be the contents of the analytics report representing the recommendations from MDA for the measurement data correlation analytics for ML model training.

#### 8.5.24.2	Information elements

Table 8.5.24.2-1

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| recommendedMeasurementDataToCollect | The attribute indicates the measurement data which is recommended to be collected. allowedValues: The list may include metrics or set of metrics defined in TS 28.552 [4], TS 28.554 [5] and TS 32.422 [6]. For performance measurements defined in TS 28.552 [4] the name is constructed as the bullet e) of measurement definition with allowed measurement type. For trace metrics (including trace messages, MDT measurements (Immediate MDT, Logged MDT, Logged MBSFN MDT), RRC, RLF and RCEF reports) defined in TS 32.422 [6], the name (metric identifier) is defined in clause 10 of TS 32.422 [6]. For non-3GPP specified managment data the name is defined elsewhere. | M | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedMeasurementDataNotToCollect | The attribute indicates the measurement data which is recommended not to be collected. allowedValues: refer to allowed values in attribute recommendedMeasurementDataToCollect | M | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| modelPerformanceImpact | The attribute indicates the model performance impact. It is a percentage indicates the loss the model performance from trained MLModel with generated measurement data comparison to the performance trained with full measurement data. E.g., 3% means the model performance for the MLModel trained with generated measurement data is 3% worse than the performance trained with full measurement data. The consumer may use the value of this attribute to help decide on whether to accept or not accept the recommendation. allowedValues: 0..100 | O | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of change