3GPP TSG-SA6 Meeting #68	S5-253455

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

Source:	Jio Platforms

Title:	Use Case on outlier detection for ML model training optimization using NDT

Spec:	3GPP TR 28.883

Agenda item:	6.20.3

Document for:	Approval

Contact:	Raghvendra Bhushan <Raghvendra.Bhushan@ril.com>

1. Introduction

The FS_NDT_Ph2 on management aspects of network digital twins mentions multiple work items. This proposal is based the scenario:

- WT-4: Investigate new use cases and requirements that require enhancement of NDT in 3GPP management system

The MLT MnS producer server aggregates training data of multiple AIMLE clients. This collection can be in two modes i.e. synchronous and asynchronous. These training modes allows AIMLE server or MLT MnS producer to merge the training data into a model being trained. However, the data coming from AIMLE clients may degrade the training if the client is low performing or malicious. This may cause retraining of model. This proposal introduces management capabilities on MLT MnS producer to create a model instances in NDT environment, train them with AIMLE client output data and then check the performance/accuracy of model instance in NDT environment. This will help in detecting which output of AIMLE client may result in low performing models.

The focus of this pCR lies on introducing management capabilities for outlier detection by comparing training output and model performance to optimize ML training. The following scenarios will be covered

- MLT MnS producer is capable to compare training data of actual AIMLE client with NDT generated training data (as per clause 5.6 of TR 28.561)

- The MLT MnS producer is capable to train simulated model in NDT, check their accuracy and accordingly merge the training output to final model in any type of training mode i.e. synchronous or asynchronous.

2. Reason for Change

To enhance management capabilies of MLT MnS Producer for outlier detection in training data and ensuring robust training using NDT based model simulation.

3. Conclusions

This paper proposes a use case on ML model outlier detection using Model and AIMLE client simulations in NDT environment.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 28.883 v0.0.0.

* * * First Change * * * *

## 7.XX	Outlier detection and model training optimization using NDT

### 7.XX.1	Solution Description

#### 7.XX.1.1	General

This use case describes how network digital twin can be used to enhance AIML model training by detecting outliers in training data and comparison of model accuracies in NDT environment before training actual model. The synchronous and asynchronous training approaches can also be simulated in NDT environment before training actual model. This use case leverages NDT to compare training data generated by AIMLE clients and training data generated by NDT (asp per clause 5.6 of TR 28.915.), train multiple model instances and dynamically adjust training data contributions based on model performance, thereby improving robustness and accuracy of AI/ML models used in network management.

Figure7.XX.1.1-1: Training optimization and outlier detection with NDT

The following scenarios are included in this use case:

- Outlier Detection: AIMLE clients sends training data to MLT MnS producer and NDT generates simulated training data based on configurations mentioned in clause 5.6 of TR 28.915. The MLT MnS produced compares the output of AIMLE client with simulated data to detect outliers to ensure robust model training.

- Model performance comparison: The MLT MnS producer may train model or group of models in NDT environment on data generated by AIMLE clients or group of AIMLE clients. The comparison on model performance can be done among multiple model instances being trained in NDT environment over same type of dataset in case of synchronous training or it can be comparisons among model’s performance before and after merging training data in NDT environment in case of asynchronous training.

#### 7.XX.1.2	Potential Requirements

REQ-NDT-OUTLIER-01: The NDT MnS producer should have the capability to allow an authorized MnS consumer to request outlier detection using AIMLE client-to-NDT simulated training data comparison.

REQ-NDT-OUTLIER-02: The NDT MnS producer should have the capability to allow an authorized MnS consumer to send training data towards different instances of simulated models.

REQ-NDT-OUTLIER-03: The NDT MnS producer should have the capability to evaluate performance of AI/ML models trained in NDT environment and report performance metrics to MnS consumer as per configuration.

#### 7.XX.1.3	Potential Solutions

#### 7.XX.1.3.1	Solution: Procedure for outlier detection and model training optimization using NDT

1.	MnS Consumer request for outlier detection and training:

- The MnS consumer requests NDT MnS Producer to generate simulated training data as described in clause 5.6. the request parameter shall contain all data types as per section 5.6.3.1.-1.

- The NDT generates the simulated training data. The NDT may also synchronize with live network or MnS Producer to ensure data relevance.

- The NDT shall report the generated data to MnS consumer or AIMLE server. The AIMLE server shall use the generated data by NDT to compare the data generated by AIMLE clients to detect outliers. Optionally, the NDT may include the outlier training data in its report.

2.	Model comparisons for synchronous training.

- The MnS consumer requests multiple NDTs to simulate ML model and train the model with provided different AIMLE clients data or data from different AIMLE client groups.

- The NDT MnS Producer shall train ML models in different NDT environments with data from different AIMLE clients or different AIMLE client groups.

- Every NDT MnS producer reports the performance of ML model instance post training with the AIMLE client data to MnS consumer or AIMLE server.

- The AIMLE server may compare the performance data of every model instance trained by group of different clients in multiple NDT environments. The data which was used to train low performing model instance will be treated as outlier and the AIMLE server may discard it or merge to main model with low weights.

3.	Model comparisons for asynchronous training.

- The MnS consumer requests to train a single model with AIMLE client data and provide performance report of model before and after training using a validation dataset.

- The NDT MnS producer trains a simulated model with data of AIMLE client and reports performance metrics before and after training to MnS consumer or AIMLE server. Optionally, it may provide the optimum weight that shall be added to AIMLE client training data before training actual model.

- The AIMLE server compares the performance data and decides if the AIMLE client training data shall be discarded or merged to main model with a calculated weight. Or it may directly use the weight provided by NDT to add to training data and train the final model.

# *** End of Changes ***