3GPP TSG-SA5 Meeting #162	S5-253528

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0721 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Add average DL PDU set delay for XR measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support XR services (AR/VR applications) and interactive media services that require high data rate and low latency communication, a PDU set is introduced, which is comprised of one or more PDUs carrying an application layer payload such as a video frame or video slice. The XR service requires the transmission delay of the PDU Set to be within the PDU Set Delay Budget (PSDB). Therefore, it is necessary to add the measurement of average DL PDU set delay for XR service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add XR service related measurements： - average DL PDU set delay between NG-RAN and PSA UPF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot monitor average DL PDU set delay for XR measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.4.X	PDU set delay

5.4.X.1	average DL PDU set delay between NG-RAN and PSA UPF

a)	This measurement provides the average DL PDU set delay between PSA UPF and NG-RAN.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

For each PDU set (PDU Set Sequence Number PSSNi, see 38.415 [31]), the gNB records the following time stamps and information (see 29.281 [42], 23.501 [4]):

-	T1 received the first PDU of a PDU set (PSSNi) by the PSA UPF at the N6 termination point that was sent by the DN;

-	T2 that all DL PDUs of a PDU set (PSSNi) have been successfully received by NG-RAN that was sent by the UPF;

The gNB counts the successfully received number (N) of PDU sets and takes the following calculation:

i=1N(T2i-T1i)N

d)	Each measurement is a real representing the average delay in microseconds.

e)	The measurement name has the form PDUSet.DelayDlPsaUpfNgranMean

f)	EP_N3 (contained by UPFFunction); 
EP_N6 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

next change

A.X Use case of monitoring of PDU sets management for XR

eXtended Reality (XR) service is considered as one of the key candidate services in 5GA and 6G networks. To support XR services, PDU Set is defined (see TS 26.522 clause 3.1), i.e. one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. frame(s), video slice(s), metadata, etc.).

The PDU Set based QoS handling by the 5G-AN is determined by PDU Set QoS Parameters, such as PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER), etc., which can be used by the 5G-AN to support the configuration of scheduling and link layer functions.

The DL PSDB applies to the DL PDU Set received by the PSA UPF. To enable support for PSDB, it is required that a maximum inter arrival time between the first received PDU and the last received PDU of a PDU Set complies with SLA. It is therefore necessary to monitor whether the DL PDU set delay exceeds the PSDB is necessary to support the network with frame scheduling and guarantees.

End of changes