# 3GPP TSG SA WG5 Meeting #162	S5-253634

# Goteborg, SE 25 - 29 August 2025

# Source:	Vodafone

# Title:	Discussion on unified management interface for multi-RAT support (4G/5G/6G)

# Document for:	Discussion

Agenda Item:	5.5

# 1	Decision/action requested

To analyse the management interface status in 3GPP specifications.

# 2	References

[1]	O-RAN.WG1.OAD-R003-v10.00. October 2023

# 3	Rationale

3.1	Purpose of this Discussion Paper

Vodafone has analysed the current 3GPP specifications and identified significant challenges, particularly in relation to 4G support for implementations. In 5G, network functions are defined using YANG-based information models, whereas this approach is not applied in 4G. This difference results in inconsistencies between generations and reduces the overall coherence of the management framework.

The analysis has been conducted in relation to the workshop held in April 2025 between 3GPP and O-RAN Alliance on 6G coordination, specifically regarding O1 interface [1], to ensure unified management, services, and NRM. The session highlighted the need to define a coherent, unified framework that can support 4G, 5G, and future 6G network deployments in a consistent and interoperable manner.

Vodafone does not intend to propose any modifications to the existing network architecture, and the 4G eNodeB should remain as currently defined. However, it is necessary to help to clarify how the O1 interface operates in 4G and provide clear guidelines in this regard for O-RAN Alliance, as summarised in the workshop conclusions.

This discussion paper proposes initiating a focused study within SA5 to thoroughly analyse the existing gaps across specifications and to actively coordinate with the O-RAN Alliance to ensure alignment from the early stages. Special attention will be given to 4G support, which remains essential in many current network configurations but lacks adequate coverage in existing specifications.

To support this effort, a Study Item Description (SID) proposal is attached, outlining the scope and objectives of the proposed work. In parallel, a Liaison Statement (LS-out) is also provided and is expected to be sent to the O-RAN Alliance based on the outcome of the current SA5 meeting. Early coordination is considered critical to avoid duplication of work and to ensure alignment and consistency of the management frameworks defined by 3GPP and the O-RAN Alliance.

3.2	Background

The O1 interface facilitates essential network management functions such as configuration, performance monitoring and fault handling. Figure 1 shows that the O1 interface is defined by O-RAN Alliance and its impact on the Service Management and Orchestration (SMO) framework and O-eNB, which is the node that maps to 3GPP eNB. Note that O1 also facilitates the interaction with O-DU/O-CU entities.

In 3GPP, specifications have been developed following a generation-based approach, with separate definitions for 4G and 5G. While this structure was suitable during the independent deployment of each technology, it has become a source of fragmentation as networks increasingly adopt multi-technology architectures.

Unlike 5G, where CU/DU functions are clearly defined and modelled in 3GPP specifications using YANG, the equivalent level of definition is missing for 4G, leading to inconsistencies in O1 implementations.

For clarity, this initiative does not aim to modify the 4G architecture or the eNB functional model, but rather to guarantee that its management representation allows the O1 interface to be consistent and interoperable, to ensure a unified management, services and NRM for 4G, 5G and the upcoming 6G.

Figure 1: Logical Architecture (figure 5.1-2 in [1]).

# 4	Detailed proposal

The study could be structured around three main work tasks (WTs) aimed at addressing the challenges and gaps identified to help O1 interface to support multi-RAT deployments consistently and efficiently.

WT-1: Analyse and identify inconsistencies, redundant and inexistent model definitions across Radio accesses, documenting them.

This work task includes:

- Redundant and inconsistent definitions across Radio accesses: Similar functionalities are described differently for 4G and 5G, resulting in fragmented models that increase complexity and hinder unified management.

- Incomplete support for 4G (eNB): Particular emphasis on 4G eNB management, where current specifications lack sufficient detail and consistency.

- Documentation: All identified gaps, inconsistencies, and redundancies will be clearly documented in a structured manner.

- Alignment with 6G studies: Although 6G specifications are not yet defined, potential future evolution aspects will be considered to ensure that the resulting guidelines and models are forward-compatible.

- Coordination with O-RAN Alliance: Minimize duplication of work and inconsistent interpretations by addressing areas of alignment between 3GPP and O-RAN related to the O1 interface.

WT-2: Define requirements and transition considerations for a unified multi-RAT management interface.

This work task includes:

- Define requirements for multi-RAT management: Develop a clear guideline for accessing and managing information across 4G, 5G, and future 6G networks, ensuring backward compatibility, interoperability, and clarity for operators and vendors.

- Alignment with 6G studies: Although 6G specifications are not yet defined, potential future evolution aspects will be considered to ensure that the resulting guidelines and models are forward-compatible.

- Coordination with O-RAN Alliance: Minimize duplication of work and inconsistent interpretations by addressing areas of alignment between 3GPP and O-RAN related to the O1 interface.

# 5	Conclusion

Vodafone proposes to initiate a SID in SA5 to study to address gaps and inconsistencies across 4G, 5G and 6G potentially precluding a unified, multi-RAT management interface.

In addition, as coordination with the O-RAN Alliance will be maintained throughout the process to align efforts and minimize duplication, a LS will propose such collaboration by sharing relevant requirements and known issues, supporting a joint approach towards a consistent interface framework, specifically on O1 interface.

Vodafone kindly requests the discussion and approval of the accompanying documents S5-253635 (SID) and S5-253636 (LS) to proceed with this initiative.