3GPP TSG-SA5 Meeting #162	S5-253640

Goteborg, Sweden, 25 - 29 August 2025

Source:	Huawei, China Mobile

Title:	Rel-19 pCR TS 28.561 Add RESTful HTTP-based solution set for NDT

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	1.0.0

Work Item:	NDT

Comments

This contribution is to add stage 3 RESTful HTTP-based solution set for NDT.

Proposed Changes

* * * First Change * * * *

7	Management service for NDT (stage 3)

Editor's note: this clause will specify the solution set for NDT capabilities and services defined in clause 6.

7.X	RESTful HTTP-based solution set

he RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in 3GPP TS 28.532 [8]. Corresponding className is NDTFunction, NDTJob and NDTReport.

Following is the SS to support NDT function lifecycle management based on Table 12.1.1.1.1-1 in TS 28.532 [8].

Table 7.X-1: SS to support NDT lifecycle management

| NDT lifecycle management | IS operation | HTTP Method | Resource URI |
| --- | --- | --- | --- |
| Query an NDT function | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTFunction}={id} |

Following is the SS to support NDT job management based on Table 12.1.1.1.1-1 in TS 28.532 [8].

Table 7.X-2: SS to support NDT job management

| NDT job management | IS operation | HTTP Method | Resource URI |
| --- | --- | --- | --- |
| Create an NDT job | createMOI operation | POST | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part} |
| Delete an NDT job | deleteMOI operation | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTJob}={id} |
| Modify an NDT job | modifyMOIAttributes operation | PUT PATCH | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTJob}={id} |
| Query an NDT job | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTJob}={id} |

Following is the SS to support NDT report management based on Table 12.1.1.1.1-1 and Table 12.1.1.2.1-1 in TS 28.532 [8].

Table 7.X-3: SS to support NDT report management

| NDT report management | IS operation | HTTP Method | Resource URI |
| --- | --- | --- | --- |
| Query an NDT report | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTReport}={id} |
| Subscribe an NDT report | createMOI operation | POST | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part} |
| Notify an NDT report | notifyMOIAttributeValueChanges notification | POST | {notificationTarget} |
| Unsubscribe an NDT report | deleteMOI operation | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NtfSubscriptionControl}={id} |
| Query an NDT report subscription | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NtfSubscriptionControl}={id} |

NOTE:	The NtfSubscriptionControl is defined in TS 28.622 [7].

* * * End of Changes * * * *