3GPP TSG-SA5 Meeting #162	S5-253724

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.558 | CR | 0038 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS28.558 Add DL PDCP SDU Drop rate in gNB-CU-UP for UE level |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In S5-253295(LS from RAN3), for UL PDCP SDU Loss Rate – TS 28.552 clause 5.1.3.1.1 PDCP SDU Drop Rate in gNB-CU-UP – TS 28.552 clause 5.1.3.2.1 RAN3 asks SA5 to specify the above Packet Loss Rate metrics with a per-DRB per-UE granularity in the UL and DL at PDCP layer, which may also be aggregated to reflect a per-UE Packet Loss Rate metric. Since UL PDCP SDU Loss Rate for UE level is already defined, so this contribution is to add PDCP SDU Drop for UE level. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add DL PDCP SDU Drop rate in gNB-CU-UP for UE level |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot satisfy RAN3’s requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.3.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

##### 6.3.1.3.x	DL PDCP SDU Drop rate in gNB-CU-UP

a)	This measurement provides the fraction of PDCP SDU packets which are dropped on the downlink, due to high traffic load, traffic management etc in the gNB-CU-UP. Only user-plane traffic (DTCH) is considered. A dropped packet is one whose context is removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC), and subcounters per supported S-NSSAI.

NOTE:	this measurement may include packets that were supposed to be sent via the eUtran air interface if using NR split bearer option 3, 4 or 7.

b)	SI.

c)	This measurement is obtained as: 1000000*Number of dropped DL PDCP SDU packets whose contexts are removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface, of a data radio bearer, divided by Number of DL PDCP SDU packets for data radio bearers that have entered PDCP-SAP after being decoded from GTP-U packets. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the drop rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.PdcpPacketDropRateDlUe and optionally DRB.PdcpPacketDropRateDlUe.QOS 
where QOS identifies the target quality of service class, and DRB.PdcpPacketDropRateDlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction.

NRCellCU.

g)	N/A

h)	One usage of this measurement is to support ML training and performance evaluation.

*** END OF CHANGE 1 ***