3GPP TSG-SA5 Meeting #162	S5-253873

Goteborg, Sweden, 25 - 29 August 2025

Source:	Huawei

Title:	Pseudo-CR on TS 28.561 Improvements to NDTOutputDescription

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TR 28.561

Version:	1.0.0

Work Item:	NDT

Comments

It is proposed to enhance the description of NDTOutputDescription to include non-3GPP data. This is essential for use cases related to evaluation of high-risk network operations, verification of network response to events, verification of network configurations, and verification of automation-function configurations.

Proposed Changes

* * * First Change * * * *

##### 6.2.1.3.5	NDTOutputDescription <<dataType>>

###### 6.2.1.3.5.1	Definition

This dataType represents a description of the individual outputs of the NDT modelled twin. It may be used to describe any of the following:

-	states of the network that have been modelled and are being reported by the NDT,

-	characteristics of the network that are being reported by the NDT.

The objects that have been modelled are described by specific network object instances represented by objectInstance and their specific characteristics.

The characteristics of the network may include aspects in objectAttributeList that are modelled in the 3GPP NRM (e.g. configuration, performance metrics, alarms, etc.) or aspects in non3GPPOutputList which are not modelled in the 3GPP NRM (e.g. risk levels, congestion levels, energy consumption, etc.).

###### 6.2.1.3.5.2	Attributes

The NDTInputDescription  <<datatype>> includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.5.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTInputDescriptionId | M | T | T | F | T |
| objectInstance | M | T | T | F | F |
| objectAttributeList | O | T | T | F | T |
| non3GPPOutputList | O | T | F | F | T |

###### 6.2.1.3.5.3	Attribute constraints

None

###### 6.2.1.3.5.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

* * * Next Change * * * *

6.3.1	Attribute properties

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ndtJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtFunctionRef | It indicates an DN of a NDTFunction Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtReportRefList | It indicates a list of DN for NDTReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedNDTCapabilities | It indicates the different types of NDT application use cases which the NDT is capable of undertaking. allowedValues: "RISKY-ACTIONS_PREDICTION", "EVENTS-IMPACTS_VERIFICATION", "FAULT_INJECTION", "NETWORK_EVENTS_VERIFICATION" "NETWORK_CONFIGURATIONS_VERIFICATION", "AUTOMATION_CONFIGURATION_VERIFICATION" "ML-TRAINING_DATA_GENERATION", "USER_EXPERIENCE_DATA_GENERATION" New values can be added to this list in future releases to support new use cases. The meaning of these values is as follows: "RISKY-ACTIONS_PREDICTION" means NDTFunction supports the use case described in 5.2.2.2. "EVENTS-IMPACTS_VERIFICATION" means NDTFunction supports the use case described in 5.2.2.3. "FAULT_INJECTION" means NDTFunction supports the use case described in 5.2.2.4. "NETWORK_EVENTS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.2. "NETWORK_CONFIGURATIONS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.3. "AUTOMATION_CONFIGURATION_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.4. "ML-TRAINING_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.2. "USER_EXPERIENCE_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.3. | type: ENUM multiplicity:1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCapability | It indicates the type of application use cases that is desired to be executed. allowedValues: "RISKY-ACTIONS_PREDICTION", "EVENTS-IMPACTS_VERIFICATION", "FAULT_INJECTION", "NETWORK_EVENTS_VERIFICATION" "NETWORK_CONFIGURATIONS_VERIFICATION", "AUTOMATION_CONFIGURATION_VERIFICATION" "ML-TRAINING_DATA_GENERATION", "USER_EXPERIENCE_DATA_GENERATION" New values can be added to this list in future releases to support new use cases. The meaning of these values is as follows: "RISKY-ACTIONS_PREDICTION" means NDTFunction supports the use case described in 5.2.2.2. "EVENTS-IMPACTS_VERIFICATION" means NDTFunction supports the use case described in 5.2.2.3. "FAULT_INJECTION" means NDTFunction supports the use case described in 5.2.2.4. "NETWORK_EVENTS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.2. "NETWORK_CONFIGURATIONS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.3. "AUTOMATION_CONFIGURATION_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.4. "ML-TRAINING_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.2. "USER_EXPERIENCE_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.3. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTRANScope | It indicates the scope of the RAN that can be modelled by the NDT function. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCNScope | It indicates the scope of the CN that can be modelled by the NDT function. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTJobSynchScope | It indicates the scope of the network that should be synchronized into and modelled by the NDT for the specific NDT job. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ndtJobScenario | It indicates a network scenario that should be modelled in the NDTJob as an extra beyond what is synchronized from the network. The ndtJobScenario can be used for - Verification of network response to one or more events - evaluation of the impact of one or more failure events, e.g. a signalling storm - Evaluating one or more network issues, e.g. a coverage issue. The network issues involve one or more network events. - Evaluation of high-risk network operations which are listed within the planned configuration - Verification of network configurations which are listed within the planned configuration - Using NDT to generate ML training data - Using NDT to generate user experience data | type: NDTInputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NDTInputDescriptionId | It indicates the identifier for a specific input to be modelled in the NDTJob | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NDTInputDescriptionId | It indicates the identifier for a specific output provided as outcomes of the NDTJob | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| non3GPPOutputList | It indicates network characteristics which are not modelled in the 3GPP NRM | type: string multiplicity: * isOrdered: False isUnique: False defaultValue: None isNullable: False |
| networkEveFntInfo | This defines the information related with a network event (a provisioning, performance measurement, KPI or fault/ alarm event) that can be introduced into the NDT. The NetworkEventData can be used for - Verification of network response to one or more events - evaluation of the impact of one or more failure events, e.g. a signalling storm - Evaluating one or more network issues, e.g. a coverage issue. The network issues involve one or more network events. Editor’s note: The definition and modelling of networkEventInfo is to be clarified | Type: TBD multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| simulationData | This will define which management data is to be updated artificially in order to induce a particular network issue. The management data includes: - Performance data: The name of the performance measurement or the KPI as defined in 3GPP TS 28.552 and TS 28.554 - MDT/Trace data: The name of MDT measurements as defined in 3GPP TS 32.422 - Configuration data: The name of the attribute from any of the available MOIs. Editor’s note: The definition and modelling of simulationData is to be clarified | type: TBD multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| condition | This will define the condition that has to be satisfied in order to update the simulation data for the task that is executed by the NDT. This can be defined in terms of location and time. This will be the DN of ConditionMonitor[7]. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtJobExecustionRequirements | It describes the performance requirements for NDT modelling, e.g., maximum run time for each simulation/emulation job, precision, etc | type: NdtJobExecutionReqts multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| NDTJobOutputData | It indicates the list of NDTOutput(s) that are provided by the NDT function as the output for any task executed in an instantiated NDT job. | type: NDTOutputDataPoint multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| maxRuntime | Maximum run time for each simulation task executed in NDT Editor’s note: The unit of this attribute is to be added | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| networkState | It indicates a state of the twin network (the modelled network in the NDT) for which a configuration or reconfiguration is applied. The networkState is the desvription of what exists in the network at the time when the networkConfiguration is made | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkConfiguration | It indicates a network configuration that is executed by the NDT and being reported in the NDT report | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| observations | It indicates an impact on the network. It shows the list of network objects that are affected and the effects on the specific objects | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| collaboratingNDT | It indicates the related NDT Job contributing as a collaborator to the executed NDT Job. It describes a relationship to an NDT job, i.e. it indicates the DN of a component NDT which provides input to the NDT job | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * Next Change * * * *

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1861 at commit 4ca047218914e1a942ac592827f8626b03375245

*** START OF CHANGE 1 ***

*** OpenAPI/TS28561_NdtNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: NDT NRM

version: 19.0.0

description: >-

OAS 3.0.1 definition of the NDT NRM

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.561; Management aspects of Network Digital Twins

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.561/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-NDTNrm:

type: object

properties:

NDTFunction:

$ref: '#/components/schemas/NDTFunction-Multiple'

#-------Definition of generic IOCs ----------#

NDTFunction-Single:

description: >-

This IOC represents the properties of an NDT Function

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

supportedNDTCapabilities:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NDTCapability'

description: >-

It indicates the different types of NDT application use cases which the NDT is capable of undertaking..

New values can be added to this list in future releases to support new use cases.

nDTFunctionScope:

$ref: '#/components/schemas/NDTFunctionScope'

NDTJob-Single:

description: >-

This IOC represents the properties of an NDT job demand created by an MnS consumer. An NDT job represents the requirements for a network simulation/emulation task

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

nDTCapability:

$ref: '#/components/schemas/NDTCapability'

description: >-

It indicates the type of application use cases that is desired to be executed.

New values can be added to this list in future releases to support new use cases.

nDTJobSynchScope:

$ref: '#/components/schemas/ScopeDefinition'

nDTJobScenario:

$ref: '#/components/schemas/NDTInputDescription'

nDTJobExecutionRequirements:

$ref: '#/components/schemas/NdtJobExecutionReqts'

collaboratingNDT:

description: >-

It indicates the related NDT Job contributing as a collaborator to the executed NDT Job. It describes the DN of the collaborated NDT Job

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

ndtReportRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

NDTReport-Single:

description: >-

This IOC represents the properties of an NDT report corresponding to an NDT job

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

nDTJobOutputData:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NDTOutputDataPoint'

description: It indicates the list of NDTOutput(s) that are provided by the NDT function as the output for any task executed in an instantiated NDT job.

ndtJobRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

ndtFunctionRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

#-------Definition of the dataType ----------#

NDTCapability:

type: string

enum:

- RISKY_ACTIONS_PREDICTION

- EVENTS_IMPACTS_VERIFICATION

- FAULT_INJECTION

- NETWORK_EVENTS_VERIFICATION

- NETWORK_CONFIGURATIONS_VERIFICATION

- AUTOMATION_CONFIGURATION_VERIFICATION

- ML_TRAINING_DATA_GENERATION

- USER_EXPERIENCE_DATA_GENERATION

NDTFunctionScope:

type: object

properties:

nDTRANScope:

$ref: '#/components/schemas/ScopeDefinition'

nDTCNScope:

$ref: '#/components/schemas/ScopeDefinition'

NDTInputDescription:

type: object

properties:

nDTInputDescriptionId:

type: string

simulationData:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

networkEventInfo:

type: string

condition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

NDTOutputDescription:

type: object

properties:

nDTOutputDescriptionId:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

objectAttributeList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

non3GPPOutputList:

type: array

items:

type: string

ScopeDefinition:

type: object

oneOf:

- required: [ managedEntitiesScope ]

- required: [ areaScope ]

properties:

managedEntitiesScope:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

areaScope:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

NdtJobExecutionReqts:

type: object

properties:

maxRuntime:

type: integer

NDTOutputDataPoint:

type: object

properties:

networkState:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NDTOutputDescription'

networkConfiguration:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NDTOutputDescription'

observations:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NDTOutputDescription'

#------Definition of JSON arrays for name-contained IOCs ---------------#

NDTJob-Multiple:

type: array

items:

$ref: '#/components/schemas/NDTJob-Single'

NDTReport-Multiple:

type: array

items:

$ref: '#/components/schemas/NDTReport-Single'

NDTFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/NDTFunction-Single'

#------Definition of JSON arrays for name-contained IOCs ---------------#

#----- Definitions in TS 28.561 for TS 28.532 --------------------------#

resources-ndtNrm:

oneOf:

- $ref: '#/components/schemas/NDTFunction-Single'

- $ref: '#/components/schemas/NDTJob-Single'

- $ref: '#/components/schemas/NDTReport-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***

* * * End of Changes * * * *