| 3GPP TS 28.561 V1.1.0 (2025-08) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Management aspects of Network Digital Twins (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	9

4	Concepts and overview	9

4.1	Introduction and Overview	9

4.1.1	Digital Twins and Network Digital Twins	9

4.1.2	Utilizing emulation and/or simulation	10

4.2	NDT Management Service	10

4.3	Relations of NDTs and network automation functions	10

4.4	NDT Life-cycle management	11

5	NDT Management capabilities	11

5.1	Control and life cycle management of NDTs	11

5.1.1	Description	11

5.1.2	Use cases	12

5.1.2.1	Life cycle management of NDT instances	12

5.1.2.2	Control of NDT instances	12

5.1.2.3	Synchronization with network	12

5.1.3	Requirements	12

5.2	NDT support for network automation	13

5.2.1	Description	13

5.2.2	Use cases	13

5.2.2.1	General capabilities on NDT support for network automation	13

5.2.2.2	Support for evaluation of high-risk network operations	13

5.2.2.3	Support for evaluation of failure events including signalling storm	14

5.2.2.4	Network issue inducement	14

5.2.3	Requirements	14

5.3	NDT support for verification	15

5.3.1	Description	15

5.3.2	Use cases	15

5.3.2.1	General verification	15

5.3.2.2	Verification of network response to events	15

5.3.2.3	Verification of network configurations	16

5.3.2.4	Verification of automation-function configurations	16

5.3.3	Requirements	16

5.4	NDT support for data generation	17

5.4.1	Description	17

5.4.2	Use cases	17

5.4.2.1	General use case on NDT support for data generation	17

5.4.2.2	Using NDT to generate ML training data	17

5.4.2.3	Using NDT to generate user experience data	17

5.4.3	Requirements	18

5.5	Advanced NDT capabilities	18

5.5.1	Description	18

5.5.2	Use Cases	18

5.5.2.1	Collaboration between NDTs	18

5.5.3	Requirements	18

6	Management service for NDT (stage 2)	19

6.1	Management operation for NDT (MnS component type A)	19

6.2	Information model for NDT (MnS component typeB)	19

6.2.1	Information model definition	19

6.2.1.1	Imported information entities and local labels	19

6.2.1.2	Class diagram	19

6.2.1.2.1	Relationship	19

6.2.1.2.2	Inheritance	20

6.2.1.3	Class definition	20

6.2.1.3.1	NDTFunction <<InformationObjectClass>>	20

6.2.1.3.1.1	Definition	20

6.2.1.3.1.2	Attributes	21

6.2.1.3.1.3	Attribute constraints	21

6.2.1.3.1.4	Notifications	21

6.2.1.3.2	NDTJob <<InformationObjectClass>>	21

6.2.1.3.2.1	Definition	21

6.2.1.3.2.2	Attributes	21

6.2.1.3.2.3	Attribute constraints	22

6.2.1.3.2.4	Notifications	22

6.2.1.3.3	NDTFunctionScope << dataType >>	22

6.2.1.3.3.1	Definition	22

6.2.1.3.3.2	Attributes	22

6.2.1.3.3.3	Attribute constraints	22

6.2.1.3.3.4	Notifications	22

6.2.1.3.4	NDTInputDescription<<dataType>>	22

6.2.1.3.4.1	Definition	22

6.2.1.3.4.2	Attributes	22

6.2.1.3.4.3	Attribute constraints	23

6.2.1.3.4.4	Notifications	23

6.2.1.3.5	NDTOutputDescription <<dataType>>	23

6.2.1.3.5.1	Definition	23

6.2.1.3.5.2	Attributes	23

6.2.1.3.5.3	Attribute constraints	23

6.2.1.3.5.4	Notifications	23

6.2.1.3.6	ScopeDefinition <<choice>>	23

6.2.1.3.6.1	Definition	23

6.2.1.3.6.2	Attributes	23

6.2.1.3.6.3	Attribute constraints	24

6.2.1.3.6.4	Notifications	24

6.2.1.3.7	NdtJobExecutionReqts <<dataType>>	24

6.2.1.3.7.1	Definition	24

6.2.1.3.7.2	Attributes	24

6.2.1.3.7.3	Attribute constraints	24

6.2.1.3.7.4	Notifications	24

6.2.1.3.8	NDTReport <<InformationObjectClass>>	24

6.2.1.3.8.1	Definition	24

6.2.1.3.8.2	Attributes	24

6.2.1.3.8.3	Attribute constraints	25

6.2.1.3.8.4	Notifications	25

6.2.1.3.9	NDTOutputDataPoint <<dataType>>	25

6.2.1.3.9.1	Definition	25

6.2.1.3.9.2	Attributes	25

6.2.1.3.9.3	Attribute constraints	25

6.2.1.3.9.4	Notifications	25

6.2.1.3.10	SimulationDataDescriptor <<dataType>>	25

6.2.1.3.10.1	Definition	25

6.2.1.3.10.2	Attributes	25

6.2.1.3.10.3	Attribute constraints	26

6.2.1.3.10.4	Notifications	26

6.2.1.3.11	SimulationData <<dataType>>	26

6.2.1.3.11.1	Definition	26

6.2.1.3.11.2	Attributes	26

6.2.1.3.11.3	Attribute constraints	26

6.2.1.3.11.4	Notifications	26

6.2.1.3.12	PerformanceData <<dataType>>	26

6.2.1.3.12.1	Definition	26

6.2.1.3.12.2	Attributes	26

6.3	Attribute definitions	27

6.4	Procedure for NDT management	34

6.4.1	Procedure for consuming NDT management service	34

6.4.2	Procedure for delete an NDT job	35

6.4.3	Procedure for query an NDT job	35

7	Management service for NDT (stage 3)	35

7.1	RESTful HTTP-based solution set	35

Annex A (informative): PlantUML source code	37

A.1	Information model definition for NDT	37

A.1.1	Relationship UML diagram for NDT management	37

A.1.2	Inheritance UML diagram for NDT management	37

A.2	Procedure for NDT management	37

A.2.1	Procedure for delete an NDT job	37

A.2.2	Procedure for query an NDT job	38

Annex B (informative):	38

B.1	NDT function in RAN domain	38

B.2	NDT function in cross domain	38

Annex C (informative): Change history	39

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

1	Scope

The present document specifies the terms and concepts of Network Digital Twin, as well as the use cases, requirements, and solutions for Network Digital Twins within the 3GPP management system.

The present document applies to the service based management architecture.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[3]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[4]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[5]	3GPP TS 28.111: "Management and orchestration; Fault Management (FM)".

[6]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[7]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[8]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[9] 	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Network Digital Twin (NDT): virtual replica of mobile network or part of one, that captures its attributes, behaviour and interactions

NOTE 1:	Mobile network includes both RAN and Core Network.

NOTE 2: 	An NDT can collaborate with other NDTs during simulation or emulation activities, and dynamically adapt its internal behaviour accordingly.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AMF	Access and Mobility Management Function

CSPs	Communication Service Providers

DT	Digital Twin

ES	Energy Saving

MDA	Management Data Analytics

MDAF	Management Data Analytics Function

ML	Machine Learning

MOI	Managed Object Instance

NDT	Network Digital Twin

NDT MnS	Network Digital Twin Management service

SMF	Session Management FunctionS-NSSAI	Single Network Slice Selection Assistance Information

SON	Self-Organizing Networks

UDM	Unified Data Management

UL/DL		Uplink / Downlink

4	Concepts and overview

4.1	Introduction and Overview

4.1.1	Digital Twins and Network Digital Twins

A digital twin is a software representation of an object or a set of objects for the purpose of modelling some of the characteristics and behaviours of such real-world object(s) or system(s). The digital twin provides support to network management and operations by creating a virtual representation of the corresponding physical network process(es). A digital twin can be created for any physical object, including any objects in communication networks. The digital twin may also be created for a group of objects, e.g. for the sets of network objects that form the RAN segment or the NFs in the Core network.

Accordingly, a digital twin modelling an object, or a set of objects, of a communication network is called Network Digital Twin.

Network Digital Twin (NDT) is used as a replica of a mobile network, in order to learn how an actual mobile network would behave in certain scenarios, without causing any changes to the actual mobile network. To provide meaningful results, an NDT needs to model the behaviour of the mobile network, so that the result of the operations on the virtual replica is a good approximation to the result of similar operations on the actual network. The created NDT can provide the capability of reporting the topology of the network, and the non-topology aspects of the real network which includes both network elements (e.g., 5GC NFs or gNB) information and infrastructure resource information. The implementation of an NDT can rely on simulation, emulation, AI-based modelling, or any other technique that enables the NDT to mimic the behaviour of the network. The present document uses the term “simulation/emulation” to refer to any combination of the techniques previously listed.

Thus, the NDT contributes to efficient management of mobile networks, helps to build resilient networks, enables the early deployment of new services, and enhances network quality. For example, with NDT, network operators can verify network behaviour before they apply changes to the real network to prevent unintended behaviour, which contributes to resilient networks and enhancement of network quality. Additionally, network automation function can use NDT to analyze network behaviour, which can contribute to reducing  operator’s manual operation and improve management efficiency. NDT may also utilize network automation functions to deliver NDT reports.

4.1.2	Utilizing emulation and/or simulation

The implementation of NDT modelling relies on simulation, emulation or other modelling technologies that enables the NDT to mimic the behaviour of the network.

Simulation implies that the NDT uses a mathematical model to characterize the behaviour of the system. This is used e.g. to model the traffic generated by users in the RAN to model the movement of users, or to model simplified behaviour of a NF. Emulation implies that the NDT uses a realizable object such as software that replaces the live object to characterize the behaviour of at least one aspect of the system. This is used e.g. to replicate or mimic the functionalities of core network functions which are implemented using the real software of NFs.

It is possible to combine emulation and simulation to create an integrated solution. The choice depends on implementation and deployment considerations, and is thus out of scope of this specification.

4.2	NDT Management Service

NDT MnS Producer enables an authorized consumer to request the modelling of a network scenario and to receive the output report. The simulation/emulation capabilities provided by the NDT MnS Producer can be invoked by  management functions, e.g., MDAF, AIML Inference Function, Intent Handling Function, when playing the role of an NDT MnS Consumer, to support their intelligence and automation functionality.

Figure 4.2-1

4.3	Relations of NDTs and network automation functions

Existing automation capabilities include analytics services and decision-making capabilities with the assumption that the knowledge about network behaviour is available within the automation functions. Accordingly, the unaddressed gap for network automation is the capabilities for modelling the behaviour of the network. The Network Resource Model (NRM) is used to describes the attributes of a mobile network and their relationships. The Network Digital Twin facilitates modelling the behaviour of a mobile network, and the analytic data from automation functions (e.g., MDA) may also be utilized when needed. Therefore, network digital twins could provide modelling capabilities that support network automation functions (e.g. MDA, SON, etc.) to accomplish their automation objectives. When the network digital twin provides modelling capabilities for the network, it can be integrated inside or outside of existing network automation functions. - see figure 4.3-1, and the options are implementation-specific.

Option 1	NDT internal/integrated into existing Network Automation Functions.

Option 2	NDT separate from existing Network Automation Functions

Figure 4.3-1: Relation of NDTs with network automation functions

NOTE: The double headed arrows indicate candidate flow of data or control while one headed arrow indicates only flow of data.

4.4	NDT Life-cycle management

An NDT may have multiple simulation/emulation jobs (here called NDT jobs) each considering a different network scenario and use case. An NDT function instance is a manageable object that provides simulation and emulation capabilities for specific network scenarios.

The NDT is an object that can be managed, the LCM includes two aspects: the LCM of the NDT function instance and the LCM of the NDT job instances. Accordingly, the following capabilities are included:

-	NDT function creation: The MnS producer who provides the NDT simulation/emulation capabilities creates an NDT function instance that is capable to model a specific scenario.

-	NDT job instantiation: NDT MnS producer receives the request to create a job. The NDT MnS producer instantiates and executes the simulation/emulation for this job, which is an NDT job instance. The NDT job instance can be configured by the MnS consumer at any time.

-	NDT job suspension: NDT MnS producer receives the request to pause or suspend a job.

- 	NDT job deletion: NDT MnS producer receives the request to delete a simulation/emulation job instance. The NDT MnS producer stops the execution of the simulation/emulation and deletes the NDT job instance.

-	NDT function deletion: the MnS producer may delete an NDT Function instance that is not active.

NOTE: All the life cycle operations may not be valid for all NDT Function instances. For example, a particular NDT function instance may not support suspending the NDT job execution.

5	NDT Management capabilities

5.1	Control and life cycle management of NDTs

5.1.1	Description

This describes uses on control and life cycle management of NDT instances.

5.1.2	Use cases

5.1.2.1	Life cycle management of NDT instances

NDTs can be used for many application use cases. In each case, the NDT needs to implement a defined network scenario, run a simulation/emulation of that scenario, and subsequently provide an output representing the statuses of different network for the simulated network scenario. The lifecycle management of NDT refers to the management processes that control and transition an NDT instance through the processes of creation, activation, operation, deactivation, deletion and any updates during these processes. At each step, it is critical to ensure that the NDT instance accurately reflects the status of physical entities and support efficient simulation/emulation.

-	For creation, the NDT MnS Consumer may provide requirements to request the NDT MnS producer to trigger the creation of an NDT instance based on its capabilities. NDT MnS producer may create a new NDT instance from scratch, updating existing NDT models or to combining existing NDT models to generate a new NDT.

-	At any point during the life of an NDT instance, the MnS consumer may provide physical network data to refine and adjust the NDT instance to accurately reflect the status of the physical entity and adapt it to specific application scenarios. For example, the consumer may trigger data collection from the network and synchronization of the state with the NDT.

5.1.2.2	Control of NDT instances

The 3GPP management system should support capabilities enabling consumers to define the network scenario, to control (start, suspend, resume or stop) the simulation/emulation of that network scenario and to provide outcomes representing measurements and counters from metrics in the simulated/emulated network scenario.

An NDT may have dependencies on other NDTs or DTs, each of which implements a specific network scope. The MnS consumer should also be enabled to provide input to NDT MnS Producer to create relationship among NDT instances, i.e., configure or modify the dependencies (e.g., to select which environment or radio propagation DT to be applied).

5.1.2.3	Synchronization with network

The NDT needs to obtain information from physical network including network functions and entities. The collected information includes the following:

-	Performance data: Performance measurement or the KPI as defined in 3GPP TS 28.552 [2] and 3GPP TS 28.554 [3].

-	MDT/Trace data: MDT measurements as defined in 3GPP TS 32.422 [4].

-	Fault management data: Fault management data as defined in 3GPP TS 28.111[5]

-	Configuration data: Configuration data defined in 3GPP TS 28.541 [6] and 3GPP TS 28.622 [7], any of the available MOIs;

Mobile networks are generally composed of numerous network elements. The data about these numerous network elements can potentially be vast. When collecting this vast amount of data to NDT, the collection might cause congestion in the network bandwidth used for data collection. NDT MnS Producer may create an NDT instance according to data provided by the NDT MnS Consumer or collect the data from the physical network for synchronization. The MnS producer should enable the MnS consumer to control the frequency of data to be collected by the NDT to minimize unnecessary collection of data and minimal congestion.

5.1.3	Requirements

Table 5.1.3-1: Control and life cycle management of NDTs

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-DTLCM-01: | The 3GPP management system should support a capability enabling an authorized MnS consumer to define the network scenario that should be modelled and simulated/emulated. | Life cycle management of NDT instances (See clause 5.1.2.1) |
| REQ-DTLCM-02: | The 3GPP management system should support a capability enabling an authorized MnS consumer to control the simulation/emulation of a network scenario, e.g. to start, or stop the simulation/emulation. | Life cycle management of NDT instances (See clause 5.1.2.1) |
| REQ-DTLCM-03: | The 3GPP management system should support a capability to provide to an authorized MnS consumer an output of a modelling or simulation process, the output representing the status of the network for the simulated scenario. | Life cycle management of NDT instances (See clause 5.1.2.1) |
| REQ-DTLCM-04: | The 3GPP management system should support a capability to allow the MnS consumer to provide input to NDT MnS Producer to create relationship among NDT instances | Control of NDT instances (See clause 5.1.2.2) |
| REQ-DTLCM-05: | The 3GPP management system should have the capability enabling an authorized MnS consumer to control the frequency of network data collection. | Synchronization with network (See clause 5.1.2.3) |

5.2	NDT support for network automation

5.2.1	Description

NDTs can be used to support automation use cases (e.g. MDA, SON, etc.). An NDT may be integrated into a network automation function, or it may be external to the network automation function. In the case where the NDT is external to the network automation function, it could be possible for the network automation function to configure the NDT and the scenario that could be modelled by the NDT. NDTs do not execute any actions in form of configuration of the live network but support decision-making.

5.2.2	Use cases

5.2.2.1	General capabilities on NDT support for network automation

An NDT, depending upon the network or service management use case and scenario to be modelled, might need data originating from various sources (network data, environment data, analytic, UEs data) and suitable hardware/software resources to function properly. MnS consumers would need to specify needed NDT characteristics or configurations to the NDT tailored to fulfil consumer specific needs i.e., to define the consumer preference for the specific NDT. For example, consumer preferences may be related to environment data sources e.g., weather, synthetic data etc, data characteristics (e.g., robustness, data granularity, maximum tolerable latency), required NDT output (e.g., latency), characteristics of the service to be twinned, resource constraints (HW/SW), etc. Furthermore, in the case that consumer's preference on NDT characteristics or configuration may change over time and MnS consumer may update the NDT with the needed changes.

The achieved performance by NDT may depend on NDT characteristics such as load, time or energy saving state. This means that the simulation results would change based on the network load, the target time of the day, or on whether the simulated node is running in energy saving mode. The NDT report should indicate the performance for the respective configured NDT characteristics. The network performance, such as latency, throughput, etc., contained in the NDT report can be used by the NDT MnS Consumer to evaluate the network performance of the modelled scenario.

The automation scenarios that the NDT might support are described in following sub-clauses.

5.2.2.2	Support for evaluation of high-risk network operations

Each operation for network optimization and maintenance on a mobile network may cause potential network failures, including network congestion leading to lack of ability to fulfil the expected requirements and network breakdown. High-risk operations are operations which have the potential of causing major service impact, for instance, invalid configuration or policy modification. Even other network activities such as software version upgrade, board switching, or optimization procedures on a live network may cause major network degradation and may be considered as high-risk operations. NDT can be utilized to evaluate both known high-risk network operations and those optimization and verification of network policies suspected to pose high-risk consequences to the network.

The results of NDT can give improved system awareness and insights to the consumer and can aid in finding the optimal policy or solution and in avoiding or minimizing risks.

5.2.2.3	Support for evaluation of failure events including signalling storm

The NDT can be used to support the evaluation of failure event refers to the situation where users in the network are completely unable to get service from the network. A typical example of a failure event is signalling storm, where a large number of signalling messages suddenly surge in the mobile communication network, overloading the network's processing capacity and consequently degrading the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signalling messages surge suddenly, causing signalling storm. When a signalling storm occurs on the network, network Functions (NFs) such as AMF, SMF, and UDM in the 5G network might all be potential impact points. Therefore, when received the request with network object information from MnS consumer for signalling storm analysis, the NDT can execute the modelling and validation based on network object related information (network performance, KPIs, S-NSSAIs, etc.), which synchronized from the network resources, and provide the report with the modelling results to MnS consumer.

An automation function may propose a solution to a signalling storm. The NDT can be used to evaluate the appropriateness of the proposed solution (e.g.,update the configuration of network flow control parameters) based on the analysing of failure events for resolving, predicting and preventing the signalling storm issue.

NDT models network behaviors and provides information on potential impacts of network failures, including surges in signalling requests. For instance, in case of the signalling storm, the information can enable identification of optimal flow control parameters for each signalling impact point.

5.2.2.4	Network issue inducement

This use case describes how a network issue can be induced using NDT. A resilient network requires that the behaviour and performance of the network are monitored during certain network failure issue e.g. node/functionality failure, service degradation etc. To plan for the optimal network configuration in case of such network failure issue, NDT can be used for issue inducements. For a particular issue that is induced, the NDT measures performance identifies degradation/faults/failures and the mitigation actions can be decided. The following are some of the examples of the issues that can be induced.

Scenario 1: Service degradation - The network slice performance degradation in terms of low PDU session establishment success rate or in terms of high latency can be induced, in a NDT, to see how the related network functions will behave when the PDU session establishment success rate is degraded. The remedial actions can be decided to mitigate the problem.

Scenario 2: Coverage issue - The coverage issue can be induced, in a NDT, to see how the related services are getting effected. The remedial actions can be decided to mitigate the problems arising due to the induced coverage issue.

Scenario 3: Fault injection - NDT can be used for fault injection experiments avoiding impact on the physical network while measuring and monitoring the impact of each injected fault in the NDT simulation. This could be leveraged to build a training data set for enhancing and enriching detection and diagnosing systems capabilities. In addition, NDT could be leveraged for improving root causes analysis.

5.2.3	Requirements

Table 5.2.3-1: NDT support for network automation

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTAUTO-01: | The 3GPP management system should support a capability enabling an authorized MnS consumer to specify characteristics or configurations of the network scenario to be modelled in the NDT. | General capabilities on NDT support for network automation (Clause 5.2.2.1) |
| REQ-NDTAUTO-02: | The 3GPP management system should support a capability to report to the authorized MnS consumer the simulation/ emulation output by NDT. | General capabilities on NDT support for network automation (Clause 5.2.2.1) |
| REQ-NDTAUTO-03: | The 3GPP management system should support a capability enabling an authorized MnS consumer to define and request simulation/ emulation of a network optimization operations. | Support for evaluation of high-risk network operations (Clause 5.2.2.2) |
| REQ-NDTAUTO-04: | The 3GPP management system should have a capability enabling an authorized MnS consumer to configure the network scenario to be modelled for the consumer to evaluate a network failure. | Support for evaluation of high-risk network operations (Clause 5.2.2.2) |
| REQ-NDTAUTO-05: | The 3GPP management system should support a capability to model the behaviour of the signalling storm in network. | Support for evaluation of failure events including Signaling storm (Clause 5.2.2.3) |
| REQ-NDTAUTO-06: | The 3GPP management system should support a capability enabling an authorized MnS consumer to define and request simulation/ emulation of the proposed solution for resolving a set of network failure events, including signalling storm. | Support for evaluation of failure events including Signaling storm (Clause 5.2.2.3) |
| REQ-NDTAUTO-07: | The 3GPP management system should support a capability enabling an authorized MnS consumer to request inducement of a network issue by an NDT. | Network issue inducement (Clause 5.2.2.4) |
| REQ-NDTAUTO-08: | The 3GPP management system should have a capability for NDT MnS producer to report the impact of the injected issue in NDT. | Network issue inducement (Clause 5.2.2.4) |

5.3	NDT support for verification

5.3.1	Description

This describes NDT use in support of verification.

5.3.2	Use cases

5.3.2.1	General verification

To evaluate a network scenario, the NDT needs to be configured with data defining a network scenario, and where applicable, the configurations and functionalities to be evaluated. The MnS consumer should be enabled to configure the NDT accordingly. Given the scenario, configurations and functionalities, the consumer can then control the simulation (e.g. to start or stop the simulation) according to the LCM capabilities as described in clause 5.1. The NDT simulates the defined scenario configurations and functionalities and collects data, which is then compiled into an NDT report. The MnS consumer should be enabled to configure the information that should be included in the simulation report. The NDT MnS producer should be enabled to send the simulation report to MnS consumer to indicate the network simulation results.

5.3.2.2	Verification of network response to events

The network can experience different kinds of events, including failure and non-failure events. A failure event refers to the situation where the network does not behave as expected, e.g., where users in the network are completely unable to get service from the network. A non-failure event refers to the situation where the network experiences an unusual occurrence, but users are still able to get service. The NDT can be used to evaluate how the network responds to any of these events. The NDT can model network behaviors and provide information on potential impact of network events. The NDT can be used to evaluate if a particular event will cause a network failure or whether the network can withstand the event.

The following is an example of events that can be supported:

- 	Signalling storms: A signalling storm is a failure events where a large number of signalling messages suddenly surge in the mobile communication network, resulting in the network processing capacity overload, thus affecting the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signalling messages surge suddenly, causing signalling storm. In this scenario, NDT is used to simulate/emulate the network behaviour of surges in signalling requests on the network. For example, NDT can simulate/emulate a sudden surge in the number of UE requests in the scenario of major holidays or network interruption recovery. Operators can also select the multiplication factor for the surge in signalling requests to be injected in NDT to evaluate the network resilience capability in response to different severity levels of events.

5.3.2.3	Verification of network configurations

Several configurations can be provided to the network, either as modifications to single parameters or as bulk configurations, e.g., as a configuration plan. There may be multiple configurations that are iteratively executed in NDT until the network performance, e.g. energy efficiency of NG-RAN, UE throughput in gNB, etc., meets the operators’ requirements. The MnS consumer should be enabled to configure the NDT with data defining a network scenario to be evaluated as well as the input information that should be applied to network in that scenario. The information may include configurations, simulated network scenarios or simulated traffic patterns.

The configurations can be evaluated for any network scenarios including the event listed in 5.3.2.1

5.3.2.4	Verification of automation-function configurations

Besides network configurations, network automation functionality (e.g., SON functions) may also be configured., e.g. configurations of functionality say for energy savings. The NDT can be used to evaluate the configurations of the automation functionality. The MnS consumer should be enabled to configure the NDT with data defining a network scenario to be evaluated for one or more automation functionality with the corresponding input information that should be applied to automation functionality to be evaluated. The information may include configurations, simulated network scenarios or simulated traffic patterns.

An example scenario of automation configuration verification is verification of Configuration of RAN energy saving management function (RAN ES MnF). A RAN ES MnF derives and executes network management operations via provisioning MnS to maximize energy saving for a group of cells e.g. based on the degree to which the cells overlap with each other. The NDT can be used to evaluate the configurations of the RAN ES MnF, i.e., the MnS consumer may instantiate an NDT simulation/emulation to verify the RAN ES configuration. The verification can be for configuration of behavior including which cells are grouped for ES and their degree of overlap. The verification can also be for configuration of performance requirements, which is MnS consumer’s requirements for network simulation performance, e.g., energy efficiency, RAN UE throughput, precision, maximum run time, etc. NDT performs network simulation/emulation, i.e., simulating/emulating the execution of RAN ES according to the configured behavior and performance requirements and collects its impact on the simulated network.

5.3.3	Requirements

Table 5.3.3-1: NDT support for verification

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTVER-01: | The management system should support a capability enabling an authorized MnS consumer to define the information that should be included in the report/output on the simulation/emulation of verification of network scenarios and configurations. | General verification (Clause 5.3.2.1) |
| REQ-NDTVER-02: | The management system should have a capability enabling an authorized MnS consumer to define one or more network events to be modelled and evaluated by the NDT. | Verification of network response to events (Clause 5.3.2.2) |
| REQ-NDTVER-03: | The management system should have the capability to report the simulation/emulation outcomes on the verification of one or more network events to be modelled and evaluated by the NDT. | Verification of network response to events (Clause 5.3.2.2) |
| REQ-NDTVER-04: | The management system should have a capability enabling an authorized MnS consumer to define one or more network configurations to be modelled and evaluated by the NDT. | Verification of network configurations (Clause 5.3.2.3) |
| REQ-NDTVER-05: | The management system should have the capability to report the simulation/emulation outcomes on the verification of network configurations modelled and evaluated by the NDT. | Verification of network configurations (Clause 5.3.2.3) |
| REQ-NDTVER-06: | The management system should have a capability enabling an authorized MnS consumer to define one or more automation functionality and configurations for the automation functionality to be modelled and evaluated by the NDT. | Verification of automation-function configurations (Clause 5.3.2.4) |
| REQ-NDTVER-07: | The management system should have the capability to report the outcomes on the verification of one or more automation. functionality and configurations modelled and evaluated by the NDT. | Verification of automation-function configurations (Clause 5.3.2.4) |

5.4	NDT support for data generation

5.4.1	Description

NDTs can be used to support the generation of data in different application use cases.

5.4.2	Use cases

5.4.2.1	General use case on NDT support for data generation

NDT can be utilized to provide a report with generated data and information related to network scenarios, configurations, policies, and performance outcomes. When receiving the request from the MnS consumer, the MnS Producer creates the NDT job and determines the network objects and data (e.g., PM data, CM data) according to data requirements. The simulated network objects include simulated network areas and managed objects in NDT. And then, according to the modelled network objects and data, the MnS Producer collects the data from the physical network used for NDT job simulation. Subsequently, based on the collected data, the MnS Producer creates/activates NDT instance, executes the network simulation and generates data corresponding to the request by using the NDT.

The data generation scenarios that the NDT might support include those in following sub-clauses.

5.4.2.2	Using NDT to generate ML training data

ML training usually requires reliable and data with a wide variety and range to guarantee the good performance of the ML models. Sufficient ML training data plays a key role for training the ML model. In general, the ML training data for network related use cases is obtained through historical network management data. For instance, for training an ML model for the MDA SLS analysis use case, described in TS 28.104 clause 7.2.2, the raw feature of training data could be the enabling data, such as UL/DL throughput, uplink/downlink delay, etc., as specified in clause 8.4.2 of TS 28.104 [9] can be used.

However, obtaining data from the network has the following limitations:

-	The quantity of issues that happened in the actual mobile network is limited.

-	The variety of issues that happened in actual mobile network is limited. There could be too many corner cases that rarely happen in a live network but can impact the performance of the ML model.

To overcome these challenges, the NDT MnS Consumer can request the NDT to generate data with an indication of data requirements, e.g. data type, required data period, data sampling periods, etc. When the request is sent by the NDT MnS Consumer, the NDT can execute the simulation/emulation and as an output generate data corresponding to the request. By using an appropriate amount of training data, the NDT MnS Consumer has a better opportunity to enhance the model accuracy.

5.4.2.3	Using NDT to generate user experience data

For operators, it is important to accurately measure user satisfaction with the network services from a network usage perspective. When the performance metrics related to customer satisfaction are low, operators are eager to identify the underlying causes and determine ways to boost performance.

Multiple factors influence user satisfaction, including user service quality, network usage experience etc. NDTs can be used to model and simulate end - user behaviors within the network, providing valuable insights for improving network services by generating simulation results, e.g. KPIs/PMs related to service experience of end user. The results may be provided to other network functionality, e.g. MDA, for further analysing the customer satisfaction level for network services.

The satisfaction of network service is affected by a combination of factors. NDT can be utilized to model network services by integrating multi-domain data sources, including network performance, user experience data, and fault predictions, and to report the user experience data.

This proactive approach allows CSPs (in this case, the NDT MnS Consumer) to identify potential detractors who are not satisfied with services across the network, monitor the end user journey, gain deeper insights into the end user's needs, and perform refined experience management based on user groups.

5.4.3	Requirements

Table 5.4.3-1: NDT support for data generation

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTDG-01: | The 3GPP management system should support a capability to provide an output on the simulation/emulation enabling generation of data. | General use case on NDT support for data generation (See clause 5.4.2.1) |
| REQ-NDTDG-02: | The 3GPP management system should have the capability to allow an authorized MnS consumer to request generation of simulated network data to be used for ML training | Using NDT to generate ML training data (See clause 5.4.2.2) |
| REQ-NDTDG-03: | The 3GPP management system should support a capability to report the generated data for ML training. | Using NDT to generate ML training data (See clause 5.4.2) |
| REQ-NDTDG-04: | The 3GPP management system should support a capability to allow an authorized MnS consumer to request generation of the user experience related data. | Using NDT to generate user experience (See clause 5.4.3) |
| REQ-NDTDG-05: | The 3GPP management system should support a capability to report the generated user experience related data. | Using NDT to generate user experience (See clause 5.4.3) |

5.5	Advanced NDT capabilities

5.5.1	Description

This clause describes advanced uses of Network digital twins.

5.5.2	Use Cases

5.5.2.1	Collaboration between NDTs

A single NDT Function might not be able to fulfil a task by itself and may depend on or need to use the service or outputs of another NDT Function during the simulation/emulation activity. This means that NDT Functions may have the capability to collaborate with each other, exchanging information and dynamically adapting their behavior according to the exchanged information. The collaborations between multiple NDT Functions can be within the same management domain or between different management domains.

The NDT MnS Consumer may express its preferences regarding relationships between NDT Functions to jointly fulfil the scenario to be modelled. Based on the NDT MnS Consumer's input, the NDT MnS Producer may evaluate whether such collaboration is necessary and determines how it can be realized..

The benefits of collaboration between NDT Functions are:

-	NDT Functions collaboration may lead to a broader system awareness and promotes flexibility

-	Complex scenarios can be fulfilled jointly by collaborating NDT Functions instances.

5.5.3	Requirements

Table 5.5.3-1: Advanced NDT capabilities

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- NDTAUT-01 | The 3GPP management system should support a capability enabling an authorized MnS consumer to configure relation between NDTs during simulation/emulation. | Collaboration between NDTs (Clause 5.5.2.1) |

6	Management service for NDT (stage 2)

6.1	Management operation for NDT (MnS component type A)

The operations (e.g., createMOI operations) and notifications (e.g., notifyMOIcreation) of generic provisioning MnS defined in 3GPP TS 28.532 [8] can be used for NDT management, including NDT function management, NDT job management and NDT report management. The NDTFunction, NDTJob, NDTReport instances can be treated as Managed Object instances.

Following is the IS to support NDT lifecycle management:

Table 6.1-1: IS to support NDT function management

| NDT function management | IS operation |
| --- | --- |
| Query an NDT function | getMOIAttributes operation |

Following is the IS to support NDT job management:

Table 6.1-2: IS to support NDT job management

| NDT job management | IS operation |
| --- | --- |
| Create an NDT job | createMOI operation |
| Delete an NDT job | deleteMOI operation |
| Modify an NDT job | modifyMOIAttributes operation |
| Query an NDT job | getMOIAttributes operation |

Following is the IS to support NDT report management:

Table 6.1-3: IS to support NDT report management

| NDT lifecycle management | IS operation |
| --- | --- |
| Query an NDT report | getMOIAttributes operation |
| Subscribe an NDT report | createMOI operation |
| Notify an NDT report | notifyMOIAttributeValueChanges notification |
| Unsubscribe an NDT report | deleteMOI operation |
| Query an NDT report subscription | getMOIAttributes operation |

6.2	Information model for NDT (MnS component typeB)

6.2.1	Information model definition

6.2.1.1	Imported information entities and local labels

| 3GPP TS 28.622 [7], DataType, GeoArea | GeoArea |
| --- | --- |
| 3GPP TS 28.622 [7], DataType, TimeWindow | TimeWindow |
| 3GPP TS 28.622 [7], DataType, ThresholdInfo | ThresholdInfo |
| 3GPP TS 28.622 [7], ConditionMonitor | ConditionMonitor |

6.2.1.2	Class diagram

6.2.1.2.1	Relationship

Figure 6.2.1.2.1-1: Relationship UML diagram for NDT management

6.2.1.2.2	Inheritance

Figure 6.2.1.2.2-1: Inheritance UML diagram for NDT management

6.2.1.3	Class definition

6.2.1.3.1	NDTFunction <<InformationObjectClass>>

6.2.1.3.1.1	Definition

This IOC represents the properties of a specific NDT Function of MnS Producer. NDTFunction instances are created by the MnS producer or are pre-installed, and also are modified, deleted by the MnS producer if needed. MnS consumers cannot request to create, modify or delete NDTFunction instances.

The NDTFunction includes a reference to one or more NDTFunction instances which act as component NDTs contributing to the functionality of the NDTFunction for NDT Collaboration.

To support collaboration among NDT Functions, the NDTFunction includes “nDTFunctionRef” attribute indicating that an NDT Function may be associated with one or more other NDT Functions. For example, one or more NDT Functions with small scope might support the operation of an NDT Function with a wider scope.

NOTE: The relationship among NDTReport of collaborating NDTFunctions is not defined.

6.2.1.3.1.2	Attributes

The NDTFunction IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedNDTCapabilities | M | T | F | T | T |
| nDTFunctionScope | M | T | F | F | T |
| Attribute related roles |  |  |  |  |  |
| nDTFunctionRef | O | T | T | F | T |

6.2.1.3.1.3	Attribute constraints

None.

6.2.1.3.1.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

6.2.1.3.2	NDTJob <<InformationObjectClass>>

6.2.1.3.2.1	Definition

This IOC represents the properties of an NDT job demand created by an MnS consumer. An NDT job represents the characteristics (e.g., scope, scenario, etc) for a network simulation/emulation task.

The attribute "nDTJobSynchScope” indicates the scope of the network that should be synchronized into and modelled by the NDT MnSProducer for the specific NDT job.

The attribute "ndtJobScenario" indicates the input that is defined by MnS consumer for the characteristics of network objects that should be simulated/emulated by NDT MnS Producer. If the NDT MnS Producer is able to synchronize with an actual network, the ndtJobScenario indicates the delta between the actual network and twin network that is simulated/emulated. Otherwise, it indicates the critical features that should be modelled, allowing the NDT MnS Producer to use defaults for all other features. The ndtJobScenario can be network configurations or automation functionality configurations, network events, issues that are defined by the MnS consumer and will be sent to the NDT MnS Producer.

The attribute "ndtJobExecutionRequirements" represents requirements related to the execution of network simulation/emulation task for an NDT job, e.g., maximum run time for each simulation/emulation task, precision, etc which are used to select the model parameters (e.g., simulation/emulation step and number of simulation/emulation times) for NDT modelling. It is up to implementation how the NDT modelling is built and used to execute the simulation/emulation task.

NOTE: the model for the specific tasks can be extended as needed.

To support collaboration among NDT Functions, the NDTJob includes the attribute “ndtJobRef” representing the NDT Job is associated with one or more other NDT Jobs that are contributing to the NDT Collaboration.

6.2.1.3.2.2	Attributes

The NDTJob IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTCapability | M | T | T | F | T |
| nDTJobSynchScope | M | T | T | F | T |
| ndtJobScenario | M | T | T | F | T |
| ndtJobExecutionRequirements | O | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtReportRefList | M | T | T | F | T |
| ndtJobRef | O | T | F | F | T |

6.2.1.3.2.3	Attribute constraints

None.

6.2.1.3.2.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

6.2.1.3.3	NDTFunctionScope << dataType >>

6.2.1.3.3.1	Definition

This dataType represents the properties of a scope that can be modelled by an NDT MnS Producer.

6.2.1.3.3.2	Attributes

The NDTFunctionScope dataType includes the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTRANScope | M | T | F | F | T |
| nDTCNScope | M | T | F | F | T |

6.2.1.3.3.3	Attribute constraints

None.

6.2.1.3.3.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

6.2.1.3.4	NDTInputDescription<<dataType>>

6.2.1.3.4.1	Definition

This dataType represents a description of the specific network scenario represented by the attribute ndtJobScenario. It may be used to describe any of the following:

-	aspects of the network that should be modelled in the NDT MnS Producer for a specific NDT Job,

-	configurations that should be applied or have been applied by the NDT MnS Producer for a specific NDT Job.

The attribute simulationDataDescriptor describes the simulation details for the NDT.

6.2.1.3.4.2	Attributes

The NDTInputDescription <<datatype>> includes the following attributes.

Table 6.2.1.3.4.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTInputDescriptionId | M | T | T | F | T |
| simulationDataDescriptor | O | T | T | F | T |
| networkEventInfo | O | T | T | F | T |

6.2.1.3.4.3	Attribute constraints

None

6.2.1.3.4.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

6.2.1.3.5	NDTOutputDescription <<dataType>>

6.2.1.3.5.1	Definition

This dataType represents a description of the individual outputs of the NDT Job. It may be used to describe any of the following:

-	states of the network that have been modelled and are being reported by the NDT MnS Producer,

-	characteristics of the network that are being reported by the NDT MnS Producer.

The objects that have been modelled are described by specific network object instances represented by objectInstance and their specific characteristics (PMs, alarms, etc.) listed in objectAttributeList.

6.2.1.3.5.2	Attributes

The NDTOutputDescription <<datatype>> includes the following attributes.

Table 6.2.1.3.5.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTOutputDescriptionId | M | T | T | F | T |
| objectInstance | M | T | T | F | F |
| objectAttributeList | M | T | T | F | T |

6.2.1.3.5.3	Attribute constraints

None

6.2.1.3.5.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

6.2.1.3.6	ScopeDefinition <<choice>>

6.2.1.3.6.1	Definition

This <<choice>> represents a definition of a scope.

The managedEntitiesScope attribute identifies the scope by the DNs of the managed entities.

The areaScope attribute identify the scope by the geographical area information.

The managedEntitiesScope attribute and areaScope attribute shall not be present at the same time.

6.2.1.3.6.2	Attributes

Table 6.2.3.6.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Choice_1 managedEntitiesScope | CM | T | T | F | T |
| Choice_2 areaScope | CM | T | T | F | T |

6.2.1.3.6.3	Attribute constraints

Table 6.2.1.3.6.3-1

| Name | Definition |
| --- | --- |
| _1 managedEntitiesScope | the MnS producer supports identifying the scope by managed entities. |
| _2 areaScope | the MnS producer supports identifying the scope by geographical area information. |

6.2.1.3.6.4	Notifications

None.

6.2.1.3.7	NdtJobExecutionReqts <<dataType>>

6.2.1.3.7.1	Definition

The NdtJobExecutionReqs <<dataType>> represents the execution requirements for NDT job.

6.2.1.3.7.2	Attributes

The NdtJobExecutionReqs includes the following attributes.

Table 6.2.1.3.7.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| maxRuntime | O | T | T | F | F |

6.2.1.3.7.3	Attribute constraints

None

6.2.1.3.7.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

6.2.1.3.8	NDTReport <<InformationObjectClass>>

6.2.1.3.8.1	Definition

This IOC represents the properties of an NDT report corresponding to an NDT job.

An NDT job may run more than one task at the same time, e.g., a network configuration task and a network response task. The NDTReport contains an output for each task that is executed by the NDT job.

The attribute "NDTJobOutputData" specified performance metrics and/or alarm types that are collected and reported by NDT MnS Producer after the behaviour is modelled in NDT MnS Producer is put in a list for which each entry is an NDTOutputDataPoint.

6.2.1.3.8.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.8.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTJobOutputData | M | T | F | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtJobRef | M | T | F | F | T |
| ndtFunctionRef | M | T | F | F | T |

6.2.1.3.8.3	Attribute constraints

None.

6.2.1.3.8.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

6.2.1.3.9	NDTOutputDataPoint <<dataType>>

6.2.1.3.9.1	Definition

This dataType represents a single output corresponding to the NDT Job.

6.2.1.3.9.2	Attributes

The nDTOutputDataPoint <<datatype>> includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

The nDTOutputDataPoint indicates three different types of information about the network. The networkState indicates the state of the network at which a specific networkConfiguration (i.e. a delta of the network state or new state of the network) is applied. The observations show the outcome on the network when the stated networkConfiguration is applied in the stated networkState.

Table 6.2.1.3.9.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| networkState | M | T | F | T | T |
| networkConfiguration | M | T | F | T | T |
| observations | M | T | F | T | T |

6.2.1.3.9.3	Attribute constraints

None.

6.2.1.3.9.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

6.2.1.3.10	SimulationDataDescriptor <<dataType>>

6.2.1.3.10.1	Definition

This dataType represents the simulation details for the NDT

6.2.1.3.10.2	Attributes

The attribute simulationData defines the management data is to be updated artificially in order to induce a particular network issue. It also defines the network management data that aims to be simulated/emulated by the NDT. The behaviour can be configurations for verification. The simulationData can be network configurations or automation functionality configurations, such as RAN ES functionality provided by SON.

The attribute Condition defines the condition that has to be satisfied in order to update the simulation data. This can be defined in terms of location and time.

Table 6.2.1.3.10.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| simulationData | M | T | F | T | T |
| condition | M | T | F | T | T |

6.2.1.3.10.3	Attribute constraints

None.

6.2.1.3.10.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

6.2.1.3.11	SimulationData <<dataType>>

6.2.1.3.11.1	Definition

This dataType defines the management data that is to be updated artificially in order to induce a particular network issue.

6.2.1.3.11.2	Attributes

The attribute performanceData defines the performance measurement or KPI name that is to be updated and with what value.

The attribute mDTData is defined as attribute/value pair representing the MDT data name that is to be updated and with what value.

The attribute configurationData defines the configuration updates for the network.

Table 6.2.1.3.11.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| performanceData | M | T | F | T | T |
| mDTData | M | T | F | T | T |
| configurationData | M | T | F | T | T |

6.2.1.3.11.3	Attribute constraints

None.

6.2.1.3.11.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

6.2.1.3.12	PerformanceData <<dataType>>

6.2.1.3.12.1	Definition

This dataType describes the performance data injected in NDT to represent network events.

The attribute performanceDataName identifies a performance measurement or KPI.

The attribute performanceDataValue is used to overwrite values which are synchronized from the real network. Alternatively, performanceDataScalingFactor is used to apply a multiplication factor to values which are synchronized from the real network. For example, in the scenario of using NDT to verify the network response to a signalling storm event, the consumer can request to inject the number of UE requests with multiplication factor of 300% in NDT, which represents three times of the number of UE requests in the real network.

6.2.1.3.12.2	Attributes

The PerformanceData <<datatype>> includes the following attributes.

Table 6.2.1.3.12.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| performanceDataName | M | T | T | F | F |
| performanceDataValue | O | T | T | F | F |
| performanceDataScalingFactor | O | T | T | F | F |

6.3	Attribute definitions

6.3.1	Attribute properties

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ndtJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtFunctionRef | It indicates a DN of NDTFunction Instance(s) that may be involved in the NDT collaboration. allowedValues: N/A | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtReportRefList | It indicates a list of DN for NDTReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedNDTCapabilities | It indicates the different types of scenario specific capability which the NDT MnS Producer is capable of undertaking. allowedValues: "RISKY_ACTIONS_PREDICTION", "EVENTS_IMPACTS_VERIFICATION", "NETWORK_ISSUE_INDUCEMENT", "NETWORK_EVENTS_VERIFICATION" "NETWORK_CONFIGURATIONS_VERIFICATION", "AUTOMATION_CONFIGURATION_VERIFICATION" "ML_TRAINING_DATA_GENERATION", "USER_EXPERIENCE_DATA_GENERATION" New values can be added to this list in future releases to support new use cases. The meaning of these values is as follows: "RISKY_ACTIONS_PREDICTION" means NDTFunction supports the use case described in 5.2.2.2. "EVENTS_IMPACTS_VERIFICATION" means NDTFunction supports the use case described in 5.2.2.3. "NETWORK_ISSUE_INDUCEMENT" means NDTFunction supports the use case described in 5.2.2.4. "NETWORK_EVENTS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.2. "NETWORK_CONFIGURATIONS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.3. "AUTOMATION_CONFIGURATION_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.4. "ML_TRAINING_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.2. "USER_EXPERIENCE_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.3. | type: ENUM multiplicity:1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTFunctionScope | It indicates a scope that can be modelled by an NDT. | type: NDTFunctionScope multiplicity:1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTCapability | It indicates the type of application use cases that is desired to be executed. allowedValues: "RISKY_ACTIONS_PREDICTION", "EVENTS_IMPACTS_VERIFICATION", "FAULT_INJECTION", "NETWORK_EVENTS_VERIFICATION" "NETWORK_CONFIGURATIONS_VERIFICATION", "AUTOMATION_CONFIGURATION_VERIFICATION" "ML_TRAINING_DATA_GENERATION", "USER_EXPERIENCE_DATA_GENERATION" New values can be added to this list in future releases to support new use cases. The meaning of these values is as follows: "RISKY_ACTIONS_PREDICTION" means NDTFunction supports the use case described in 5.2.2.2. "EVENTS_IMPACTS_VERIFICATION" means NDTFunction supports the use case described in 5.2.2.3. "FAULT_INJECTION" means NDTFunction supports the use case described in 5.2.2.4. "NETWORK_EVENTS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.2. "NETWORK_CONFIGURATIONS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.3. "AUTOMATION_CONFIGURATION_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.4. "ML_TRAINING_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.2. "USER_EXPERIENCE_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.3. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTRANScope | It indicates the scope of the RAN that can be modelled by the NDT function. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCNScope | It indicates the scope of the CN that can be modelled by the NDT function. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTJobSynchScope | It indicates the scope of the network that should be synchronized into and modelled by the NDT MnS Producer for the specific NDT job. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ndtJobScenario | It indicates a network scenario that should be modelled in the NDTJob as an extra beyond what is synchronized from the network. The ndtJobScenario can be used for - Verification of network response to one or more events - evaluation of the impact of one or more failure events, e.g. a signalling storm - Evaluating one or more network issues, e.g. a coverage issue. The network issues involve one or more network events. - Evaluation of high-risk network operations which are listed within the planned configuration - Verification of network configurations which are listed within the planned configuration - Generation of ML training data - Generation user experience data - Inducing particular issue in the network | type: NDTInputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NDTInputDescriptionId | It indicates the identifier for a specific input to be modelled in the NDTJob | type: string multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| NDTOutputDescriptionId | It indicates the identifier for a specific output provided as outcomes of the NDTJob. | type: string multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| networkEventInfo | This defines the information related with a network event (a provisioning, performance measurement, KPI or fault/ alarm event) that can be introduced by the NDT MnS Producer. The networkEventInfo can be used for - Verification of network response to one or more events - evaluation of the impact of one or more failure events, e.g. a signalling storm - Evaluating one or more network issues, e.g. a coverage issue. The network issues involve one or more network events. Editor’s note: The definition and modelling of networkEventInfo is to be clarified. | Type: TBD multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| simulationDataDescriptor | This defines the simulation details for the NDT. | type: SimulationDataDescriptorAttributeValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| simulationData | This described the management data that need to be updated artificially in order to induce a particular network issue. It also defines the network management data that aims to be simulated/emulated by the NDT. The behaviour can be configurations for verification. The simulationData can be network configurations or automation functionality configurations, such as RAN ES functionality provided by SON. | type: SimulationData multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| condition | This will define the condition that has to be satisfied in order to update the simulation data for the task that is executed by the NDT MnS Producer. This can be defined in terms of location and time. This will be the DN of ConditionMonitor[7]. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceData | This defines the performance data injected in NDT to represent network events. | type: PerformanceData multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mDTData | This defines attribute/value pair representing the MDT data name that is to be updated and with what value. | type: AttributeValuePair multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| configurationData | This defines the configuration updates for the network | type: AttributeValuePair multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceDataName | It indicates the name of performance measurement or the KPI as defined in 3GPP TS 28.552 [2] and 3GPP TS 28.554 [3]. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceDataValue | It indicates the value of performance data. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceDataScalingFactor | It indicates the percentage of scaling for performance data. A scaling factor less than 100% indicates a reduction in the performance data, a scaling factor greater than 100% indicates an increase in the performance data. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtJobExecutionRequirements | It describes the performance requirements for simulation/emulation by NDT MnS Producer, e.g., maximum run time for each simulation/emulation job, precision, etc. | type: NdtJobExecutionReqts multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| nDTJobOutputData | It indicates the list of NDTOutput(s) that are provided by the NDT function as the output for any task executed in an instantiated NDT job. | type: NDTOutputDataPoint multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| maxRuntime | Maximum run time for each simulation task executed by the NDT MnS Producer. The unit is second. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| networkState | It indicates a state of the twin network (the modelled network by the NDT MnS Producer) for which a configuration or reconfiguration is applied. The networkState is the description of what exists in the network at the time when the networkConfiguration is made. | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkConfiguration | It indicates a network configuration that is executed according to the NDT Job and being reported in the NDT report. | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| observations | It indicates an impact on the network. It shows the list of network objects that are affected and the effects on the specific objects. | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ndtJobRef | It indicates the related NDT Job contributing as a collaborator to the executed NDT Job. It describes a relationship to an NDT job, i.e. it indicates the DN of a component NDT which provides input to the NDT job. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| objectInstance | It indicates the MOI contained in the NDT report. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| objectAttributeList | It indicates the key-value pair of the characteristics of the MOI. | type: AttributeValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| managedEntitiesScope | A list of DN of managed entities which is the NDT modelling scope. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| areaScope | Geographical area which is the NDT modelling scope. | type: GeoArea multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

6.4 Procedure for NDT management

6. Notify the NDT reports

4. Send the request for NDT job execution

1. Request the creation of an NDT job instance

2a. Create and configure the NDT job instance

3. Response for the NDT job instance creation

MnS Consumer

NDT

5.Execute the NDT job and generate corresponding reports

2b. Synchronization

Managed entities

6.4.1	Procedure for consuming NDT management service

Figure 6.4.1-1: Procedure of consuming NDT management service

1. NDT MnS Producer receives a request from NDT MnS consumer to create an NDT job instance (see createMOI operation defined in TS 28.532 [8]) which represents the consumer’s requirements for the NDT job. In the request to create the NDT job instance, the NDTCapability is specified to trigger scenario specific NDT job, e.g., signalling storm analysis, network configuration verification, data generation, etc. The request further includes nDTJobSynchScope indicating the scope of NDT job (e.g., simulated network objects such as network functions, S-NSSAI, time).

2a. NDT MnS Producer creates and configures the NDT job instance according to the request from MnS consumer.

2b. NDT MnS Consumer synchronizes the simulated network objects related information from managed entities for NDT job modelling and validation.

3. NDT MnS Producer sends a response (see createMOI operation defined in TS 28.532[8]) to the NDT MnS Consumer.

4. NDT MnS Consumer sends the request to NDT MnS Producer to start the execution of the NDT job instance.

5. NDT MnS Producer executes the NDT job, creates an NDTReport instance (i.e., the instance of NDTReport IOC) and configures NDTReport MOI according to the modelling results.

6. NDT MnS Producer notifies the NDT report to MnS consumer to indicate the network simulation/emulation results.

6.4.2	Procedure for delete an NDT job

Figure 6.4.2-1: Procedure for delete an NDT job

1.	MnS Consumer sends a request to delete an NDT job instance (see deleteMOI operation defined in TS 28.532 [8]) to MnS Producer with 'objectInstance' of the NDT job MOI.

2.	Based on the request, MnS Producer deletes the NDT job MOI.

3.	MnS Producer sends response (see deleteMOI operation defined in TS 28.532 [8]) to the MnS consumer with status (OperationSucceeded or OperationFailed) and 'objectInstance' of the deleted NDT job MOI.

6.4.3	Procedure for query an NDT job

Figure 6.4.3-1: Procedure for query an NDT job

1.	MnS Consumer sends a request to query an NDT job instance (see getMOIAttributes operation defined in TS 28.532 [8]) to MnS Producer with 'objectInstance' of the existing NDT job MOI and list of attribute names of NDT job IOC. The list of attribute names identifies the attributes to be returned by this operation.

2.	Based on the request, the MnS Producer queries the concrete NDT job MOI.

3.	MnS Producer sends a response (see getMOIAttributes operation defined in TS 28.532 [8]) to the MnS consumer with 'objectClass', 'objectInstance', and list of [Attribute,Value] which is defined in clause 6.2.

7	Management service for NDT (stage 3)

7.1	RESTful HTTP-based solution set

The RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in 3GPP TS 28.532 [8]. Corresponding className is NDTFunction, NDTJob and NDTReport.

Following is the SS to support NDT function lifecycle management based on Table 12.1.1.1.1-1 in TS 28.532 [8].

Table 7.1-1: SS to support NDT lifecycle management

| NDT lifecycle management | IS operation | HTTP Method | Resource URI |
| --- | --- | --- | --- |
| Query an NDT function | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTFunction}={id} |

Following is the SS to support NDT job management based on Table 12.1.1.1.1-1 in TS 28.532 [8].

Table 7.1-2: SS to support NDT job management

| NDT job management | IS operation | HTTP Method | Resource URI |
| --- | --- | --- | --- |
| Create an NDT job | createMOI operation | POST | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part} |
| Delete an NDT job | deleteMOI operation | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTJob}={id} |
| Modify an NDT job | modifyMOIAttributes operation | PUT PATCH | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTJob}={id} |
| Query an NDT job | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTJob}={id} |

Following is the SS to support NDT report management based on Table 12.1.1.1.1-1 and Table 12.1.1.2.1-1 in TS 28.532 [8].

Table 7.1-3: SS to support NDT report management

| NDT report management | IS operation | HTTP Method | Resource URI |
| --- | --- | --- | --- |
| Query an NDT report | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTReport}={id} |
| Subscribe an NDT report | createMOI operation | POST | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part} |
| Notify an NDT report | notifyMOIAttributeValueChanges notification | POST | {notificationTarget} |
| Unsubscribe an NDT report | deleteMOI operation | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NtfSubscriptionControl}={id} |
| Query an NDT report subscription | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NtfSubscriptionControl}={id} |

NOTE:	The NtfSubscriptionControl is defined in TS 28.622 [7].

Annex A (informative):
PlantUML source code

A.1	Information model definition for NDT

A.1.1	Relationship UML diagram for NDT management

@startuml

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam linetype ortho

'skinparam BoxPadding 40

skinparam nodesep 2

class ManagedEntity <<ProxyClass>>

class NDTFunction <<InformationObjectClass>>

class NDTJob <<InformationObjectClass>>

class NDTReport <<InformationObjectClass>>

note right of ManagedEntity

Represents SubNetwork

end note

ManagedEntity "1" *-- "*" NDTFunction: <<names>>

NDTFunction "1" *-- "*" NDTJob: <<names>>

NDTFunction "1" *-- "*" NDTReport: <<names>>

NDTJob "1" <--> "*" NDTReport

@enduml

A.1.2	Inheritance UML diagram for NDT management

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class ManagedFunction << InformationObjectClass >>

class NDTFunction <<InformationObjectClass>>

class NDTJob <<InformationObjectClass>>

class NDTReport <<InformationObjectClass>>

ManagedFunction <|-- NDTFunction

Top <|-- NDTJob

Top <|-- NDTReport

@enduml

A.2	Procedure for NDT management

A.2.1	Procedure for delete an NDT job

@startuml

participant "MnS Consumer" as MNSC

participant “MnS Producer" as MNSP

MNSC -> MNSP: 1. Request to delete an NDT job instance

rnote over MNSP

2. Delete the NDT job MOI

endrnote

MNSP -> MNSC: 3. Response for delete an NDT job instance

@enduml

A.2.2	Procedure for query an NDT job

@startuml

participant "MnS Consumer" as MNSC

participant “MnS Producer" as MNSP

MNSC -> MNSP: 1. Request to query an NDT job instance

rnote over MNSP

2. Query the NDT job MOI

endrnote

MNSP -> MNSC: 3. Response for query an NDT job instance

@enduml

Annex B (informative):

NDT function in different domains

B.1	NDT function in RAN domain

The NDT function is provided by the RAN management domain to support RAN domain simulation/emulation capability. For instance, the NDT function in RAN domain is responsible for RAN energy saving configuration verification as introduced in clause 5.3.2.4. Figure B.1-1 depicts the scenario when NDT function is deployed in RAN domain.

Figure B.1-1: NDT function in RAN domain management

B.2	NDT function in cross domain

The NDT function is provided by the cross management domain to support cross-domain simulation/emulation capability. For instance, the NDT function in cross-domain is responsible for synthetic user experience data generation as introduced in clause 5.4.2.3. Figure B.2-1 depicts the scenario when NDT function is deployed in cross-domain. The cross management domain is introduced in TS 28.533.

Figure B.2-1: NDT function in cross-domain management

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-02 | SA5#159 | 1.S5-250858 2.S5-250859 3.S5-250861 4.S5-250862 5.S5-250951 6.S5-251120 7.S5-251121 |  |  |  | - pCR 28.561 Add skeleton - pCR 28.561 Add scope - pCR 28.561 Add the term and concepts of NDT - Rel-19 pCR TS 28.561 Add the Role of NDT in the Management Loop - pCR TS28.561 NDT support to Automation - pCR TS28.561 NDT LCM use case - pCR Add UC of using NDT to generate ML training data for TS 28.561 | 0.1.0 |
| 2025-04 | SA5#160 | 1.S5-251897 2.S5-251899 3.S5-251901 4.S5-251903 5.S5-252019 6.S5-252118 7.S5-252119 8.S5-252120 |  |  |  | - TS28.561 Add the NDT concept and scope - TS28.561 NDT support for network automation - Pseudo-CR on NDTs for verification - Pseudo-CR on TS 28.561 Add requirements for NDT supporting data generation - Rel-19 pCR TS 28.561 General improvements to use cases and requirements - Pseudo-CR on TS 28.561 NDT NRMs - Pseudo-CR on TS 28.561 Procedure for consuming NDT capability - Pseudo-CR on TS 28.561 Nested -NDT Definition | 0.2.0 |
| 2025-05 | SA5#161 | 1.S5-252610 2.S5-253004 3.S5-253005 4.S5-253063 5.S5-253065 6.S5-253068 7.S5-253102 |  |  |  | - Rel-19 pCR TS 28.561 Add authorization to all requirement - Pseudo-CR on TS 28.561 NDT Procedure update - Rel-19 pCR TS 28.561 Add procedures for delete and query an NDT job - Pseudo-CR on TS 28.561 NDT Use case Update - Rel-19 pCR TS 28.561 Deployment scenarios of NDT function - Pseudo-CR on NRM for Collaboration between NDTs - Pseudo-CR on General NRM for NDTs | 0.3.0 |
| 2025-05 | SA#108 | SP-250493 |  |  |  | Presentation to TSG SA for Information | 1.0.0 |
| 2025-08 | SA#109 | 1.S5-253397 2.S5-253436 3.S5-253640 4.S5-253868 5.S5-253869 6.S5-253870 7.S5-253871 8.S5-253874 9.S5-253876 10.S5-253877 11.S5-253878 12.S5-253879 13.S5-253880 14.S5-253881 15.S5-253882 16.S5-254022 17.S5-254023 18.S5-254024 19.S5-254025 |  |  |  | - Rel-19 pCR on TS 28.561 Rapporteur clean-up - Rel-19 pCR TS 28.561 Stage 3 of NDT NRM - Rel-19 pCR TS 28.561 Add RESTful HTTP-based solution set for NDT - Rel-19 pCR TS 28.561 Clarification on collaboratingNDT - Rel-19 pCR TS 28.561 Clarification on the usage of NDT in stage 2 - Rel-19 pCR TS 28.561 Update description for NDT supporting network automation - Rel-19 pCR TS 28.561 Update NDT NRM to solve misalignment - Rel-19 pCR 28.561 Updates regarding Network Issue Inducement - Rel-19 pCR TS 28.561 Update on the NDT Procedure - Rel-19 pCR on TS 28.561 Update NDT concept (relation to automation and LCM) - Rel-19 pCR TS 28.561 Update the overview of NDT - Clarify NDT management service - Clarify simulation and emulation in 4.1 - Rel-19 pCR TS 28.561 Update NDT data generation - Clarify support for data generation - Clarify evaluation of high-risk operation - Clarify NDT definition and collaborative NDT use case - Rel-19 pCR TS 28.561 Update the use case in 5.2.2.2, 5.3.2.2, 5.4.2.3 - Rel-19 pCR TS 28.561 Update reference and abbreviations | 1.1.0 |
| 2025-08 | SA#109 | S5-254089 |  |  |  | Presentation to TSG SA for Approval | 2.0.0 |