| 3GPP TR 28.886 V0.1.0 (2025-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Management Data Analytics (MDA) phase 4 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Concept and background	7

5	Use cases	7

5.1	Improvements to interoperability	7

5.1.X	Use case X: Use case title	7

5.2	Improvements to MDA framework	7

5.2.1	Use case 1: Integration of analytics with a network operator’s management system	7

5.3	Investigate new and enhanced analytics related capabilities	8

5.3.1	Use case 1: Enhance the Mobility performance analysis use case	8

5.3.2	Use case 2: MDA assisted failure resolution	8

6	Conclusions and recommendations	9

6.1	Improvements to interoperability	9

6.2	Improvements to MDA framework	9

6.3	New and enhanced analytics capabilities	9

# Foreword

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: " Management Data Analytics (MDA)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

# 4	Concept and background

# 5	Use cases

## 5.1	Improvements to interoperability

### 5.1.X	Use case X: Use case title

#### 5.1.X.1	Description

#### 5.1.X.2	Potential requirements

#### 5.1.X.3	Potential solutions

#### 5.1.X.4	Evaluation of solutions

## 5.2	Improvements to MDA framework

### 5.2.1	Use case 1: Integration of analytics with a network operator’s management system

#### 5.2.1.1	Description

A Network Operator (NOP) wishes to integrate Management Data Analytics Service with the NOP’s automated network management system.

To be able to successfully perform this integration, the NOP needs to know which management facilities are provided by MDAS. The NOP also needs to know if MDAS depends on other 3GPP management functions and/or management services, and if these dependencies are mandatory or optional.

The operator uses TS 28.104 [2] to learn the required information about MDAS. Clause 5 of TS 28.104 [2] describes the management facilities which are provided by MDAS. Clause 5 of TS 28.104 [x] describes how MDAS may be combined with other 3GPP management functions and/or management services to satisfy NOP requirements.

Currently, clause 5 of TS 28.104 [2] is lacking description on how MDAS relates to some 3GPP management functions and/or management services (e.g. CCL, NDT). The current description is very abstract and may benefit from adding examples.

#### 5.2.1.2	Potential requirements

#### 5.2.1.3	Potential solutions

#### 5.2.1.4	Evaluation of solutions

## 5.3	Investigate new and enhanced analytics related capabilities

### 5.3.1	Use case 1: Enhance the Mobility performance analysis use case

#### 5.3.1.1	Description

This use case is to enhance the existing MDA capability for mobility performance analysis. The enhancements to the existing mobility performance analysis are targeted towards to solve the mobility problems in NSA deployment architecture and to recommend optimization or repair actions.

The existing use case for mobility performance analysis is described in TS 28.104 [2]. This analysis output includes information such as mobility issue root cause, mobility issue location. The descriptions and requirements in the existing use cases also mention performance analysis under NSA deployment scenarios. However, the current solution does not provide additional information (e.g. mobility issue type such as NSA mobility issue and SA mobility issue, or the root cause of NSA mobility issue, or the recommended actions) which may enable the consumer to anticipate the impacts to the network more effectively.

The mobility performance related problems in NSA deployment scenarios may result from too-early/too-late PSCell change or too-early/too-late conditional PSCell addition or change due to inappropriate handover parameters. MDAS can be used to analyse network performance during handover period in different mobility scenarios. MDAS producer may also be capable to provide the recommendations of optimal handover parameters to MDAS consumer.

Correspondingly, the root cause of mobility issue in NSA deployment scenarios may be of interest to the consumer and can be provided in the analytics report. The root cause can be high interference in both uplink and downlink, wireless resource issues (such as high utilization of uplink and downlink PRBs), transport layer reasons (including insufficient transport resources leading to SN addition failure, such as S1 interface transport, X2 interface transport), and UE capability issues.

#### 5.3.1.2	Potential requirements

REQ-MRO_MDA-01: MDA capability for mobility performance issue analysis should provide the NSA mobility issue including too-early PSCell change, too-late PSCell change, too Late CPC Execution and too Early CPC/CPA Execution.

REQ-MRO_MDA-02: MDA capability for mobility performance issue analysis should include providing recommended actions to solve the mobility performance issue.

#### 5.3.1.3	Potential solutions

TBD

#### 5.3.1.4	Evaluation of solutions

TBD

### 5.3.2	Use case 2: MDA assisted failure resolution

#### 5.3.2.1	Description

As networks become more complex, it becomes more difficult to quickly resolve network failures, where network performance becomes degraded or lost completely. By looking only at the alarm list, it can be difficult to identify the most serious network failures. Network automation is essential to speed up the identification and resolution of network failures, thereby increasing the stability of the network. This use case enables an MDAS consumer (e.g. an automated trouble ticketing system) to automatically perform the steps needed to resolve a network failure.

The MDAS consumer requests the MDAS producer to identify alarms which may relate to network failures. The MDAS producer analyses alarm data and network data to correlate alarms with deteriorating performance in the network. The MDAS producer analyses the cause of deteriorating performance. The MDAS producer reports possible network failures and the possible cause to the MDAS consumer.

The MDAS consumer requests the MDAS producer to diagnose the cause of a network failure. The MDAS producer performs analysis to locate the likely cause of the network failure (e.g. network function, virtualized resource, physical resource). The MDAS producer reports the possible cause of the network failure to the MDAS consumer.

The MDAS consumer requests the MDAS producer to recommend how to resolve a network failure. The MDAS producer performs analysis to identify which domain is causing the network failure. The MDAS producer calculates possible methods (e.g. re-configurations) to resolve the network failure. The MDAS producer simulates or emulates the effect of the possible methods (e.g. by using NDT) to evaluate if the network failure is resolved. The MDAS producer reports the possible methods to the MDAS consumer.

#### 5.3.2.2	Potential requirements

#### 5.3.2.3	Potential solutions

#### 5.3.2.4	Evaluation of solutions

# 6	Conclusions and recommendations

## 6.1	Improvements to interoperability

## 6.2	Improvements to MDA framework

## 6.3	New and enhanced analytics capabilities

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 |  |  |  |  |  | Initial skeleton | 0.0.0 |
| 2025-08 | SA5#162 | S5‑253605 | pCR |  |  | Pseudo-CR on TR 28.886 Add clause structure | 0.1.0 |
| 2025-08 | SA5#162 | S5‑254044 | pCR |  |  | Pseudo-CR on TR 28.886 Enhance the mobility performance analysis use case | 0.1.0 |
| 2025-08 | SA5#162 | S5‑254045 | pCR |  |  | Pseudo-CR on TR 28.886 Add MDA integration use case | 0.1.0 |
| 2025-08 | SA5#162 | S5‑254046 | pCR |  |  | Pseudo-CR on TR 28.886 Add failure resolution use case | 0.1.0 |