3GPP TSG-SA5 Meeting #163	S5-254691

Wuhan, China, 13 - 17 October 2025

Source:	Nokia

Title:	Pseudo-CR on Clarification of Mechanisms to Request and Retrieve Management Data

Document for:	Approval

Agenda item:	6.20.8

Spec:	3GPP TR 28.887

Version:	0.1.0

Work Item:	FS_MADCOL_Ph3

Comments

The group is requested to discuss and approve the pCR below.

SA5 has specified several NRM fragments related to requesting and retrieving management data. In many cases, it is not obvious when and how combinations of these NRM fragments can be used when certain data is needed by an MnS consumer. This pCR is to motivate the need to enhance the documentation on the usage of these IOCs when an MnS consumer needs to request or retrieve data.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[XY]	3GPP TS 28.622: "Generic Network Resource Model (NRM)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

### 4.4.Y	Use Case#<Y>: Specified mechanisms to Request and Retrieve Management Data

#### 4.4.Y.1      Description

TS 28.622 [XY] specifies the following NRM fragments related to management data:

-	TraceJob IOC, as specified in clause 4.3.30, allows to request and retrieve trace metrics which include trace messages, MDT measurements (Immediate MDT, Logged MDT, Logged MBSFN MDT), RLF, RCEF and RRC reports.

-	PerfMetricJob IOC, as specified in clause 4.3.31, allows to request and retrieve performance metrics which include performance measurements and KPIs (end-to-end key performance indicators).

- 	QMCJob IOC, as specified in clause 4.3.54, allows to request and retrieve QoE information.

- 	ManagementDataCollection IOC, as specified in clause 4.3.47, allows to request and retrieve trace metrics and/or performance metrics.

-	ExternalDataType IOC, as specified in clause 4.3.73, allows to request external management data.

-	ThresholdMonitor IOC, as specified in clause 4.3.16, allows to check for threshold crossings of performance metric values related to specified managed objects.

-	Scheduler IOC, as specified in clause 4.3.62, allows to monitor whether a certain time criterion is fulfilled.

-	ConditionMonitor IOC, as specified in clause 4.3.65, allows to monitor whether one or several conditions are satisfied.

-	Files IOC, as specified in clause 4.3.44, represents a collection of files. It may be used together with a data collection job representing the files created in context of the collection.

-	File IOC, as specified in clause 4.3.45, represents a file. It may be used together with a data collection job and the file collection representing one file created in context of the collection.

-	FileDownloadJob IOC, as specified in clause 4.3.46, represents a job on a MnS producer that downloads a file to the MnS producer.

Furthermore, TS 28.622 [XY] specifies the following constraints-related IOCs:

-	ThresholdMonitor IOC, as specified in clause 4.3.16, allows to check for threshold crossings of performance metric values related to specified managed objects.

-	Scheduler IOC, as specified in clause 4.3.62, allows to monitor whether a certain time criterion is fulfilled.

-	ConditionMonitor IOC, as specified in clause 4.3.65, allows to monitor whether one or several conditions are satisfied.

Additionally, TS 28.622 [XY] specifies the following file-operations-related IOC:

-	Files IOC, as specified in clause 4.3.44, represents a collection of files. It may be used together with a data collection job representing the files created in context of the collection.

-	File IOC, as specified in clause 4.3.45, represents a file. It may be used together with a data collection job and the file collection representing one file created in context of the collection.

-	FileDownloadJob IOC, as specified in clause 4.3.46, represents a job on a MnS producer that downloads a file to the MnS producer.

It is expected that when an MnS consumer needs a particular type of data under certain circumstances, the MnS consumer needs to use one or more of the above listed IOCs. However, it is not always obvious when and how combinations of these NRM fragments can be used when certain data is needed by an MnS consumer.

#### 4.4.Y.2      Problem Statement

TS 28.622 [XY] specifies several NRM fragments related to management data. It is not always obvious which IOC can be used to request and retrieve management data as part of a given management service (MnS).

For example, one might not know the variation of specified NRM fragments related to management data. The other one might not understand whether to use ManagementDataCollection IOC or TraceJob IOC in case one is interested in MDT data only. Another one might not know which IOCs to use when looking for KPIs measured at weekend time where PerfMetricJob IOC in combination with Scheduler IOC will be useful. Another one might wonder whether there is a possibility to check the progress of its requested performance measurements where PerfMetricJob IOC in combination with FileDownloadJob IOC will be useful.

There is a need to provide an overview of the specified NRM fragments related to management data.

There is a need to provide clarification and documentation on the usage of the specified IOCs to request and retrieve management data.

It is desirable to also clarify how the management data related functionality apply the following constraints-related IOCs: Scheduler IOC, ConditionMonitor IOC and Threshold Monitor IOC.

Furthermore, it is desirable to clarify how the management data related functionality apply the following file-operations-related IOCs: Files IOC, File IOC, FileDownloadJob IOC.

#### 4.1.Y.3      Potential Requirements

No new requirements need to be addressed.

#### 4.1.Y.4      Potential Solution

TBA

* * * End of Changes * * * *