3GPP TSG-SA5 Meeting #164	S5-255147

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0541 | rev | <Rev#> | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.422 Corrections on Immediate MDT measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 has sent a reply LS S5-254621/R2-2506543 in response to S5-252842/R2-2505036 requesting to provide a solution to transfer specified measurements in the context of OAM-centric solution for NW-side data collection. This CR provides the corresponding corrections to align with agreed RAN2 Rel-19 enhancements in TS 37.320. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Logged L1 measurements as selectable option for management based Immediate MDT in NG-RAN - Add configuration parameters logging periodicity and event trigger configuration - Adapt the NG-RAN activation mechanism for logged L1 measurements for Immediate MDT in NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - RAN2 specifications are not aligned with SA5 specifications - RAN2 feature "network side data collection" is not supported by management system |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1.9.2, 5.10.3, 5.10.X (new), 5.10.11 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.622 CR0612 TS 28.623 CR0594 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 4.1.1.9.2	NG-RAN activation mechanisms for management based MDT data collections without IMSI/IMEI(SV)/SUPI selection in the case of non-split architecture

For management based MDT data collection with no IMSI/IMEI(SV)/SUPI criteria in the case of non-split architecture, the UE selection can be done in the radio network at gNB based on the input information received from management system and the user consent information stored in the gNB. This mechanism works for the following OAM input parameters:

-	Area information only

The following figure summarizes the flow as an example how the MDT configuration is done utilising the cell traffic trace functionality for this scenario:

Figure 4.1.1.9.2.1: Example for management based MDT activation in NG-RAN in the case of non-split architecture

Whenever the gNB receives the Management based MDT PLMN List IE in Initial Context Setup Request or in Handover Request message, it shall save it for possible later usage.

1)	The management system sends a Trace Session activation request to the gNB. This request includes the parameters for configuring UE measurements:

-	Job Type.

-	Area Scope where the UE measurements should be collected: list of NG-RAN cells. Tracking Area should be converted to NG-RAN cells, additionally a list of NPN IDs in NG-RAN.

-	List of Measurements.

-	Reporting Trigger.

-	Report Interval.

-	Report Amount.

-	Event Threshold.

-	Logging Interval.

-	Logging Duration.

-	Trace Reference.

-	TCE IP Address and Trace Reporting Consumer URI (if streaming based report is supported).

-	Anonymization of MDT Data.

-	Collection Period for RRM Measurements NR (present only if any of M4 or M5 measurements are requested).

-	Collection Period M6 in NR (present only if any of M6 measurements (DL or UL) is requested).

-	Collection Period M7 in NR (present only if any of M7 measurements (DL or UL) is requested).

-	Positioning Method.

-	MDT PLMN List.

-	Report Type for Logged MDT (periodical logged or event-triggered measurement) for logged MDT.

-	Event Threshold, Hysteresis and Time to Trigger (present only if L1 event is configured for logged MDT)

-	Event List for Event Triggered Measurement for logged MDT.

-	Area Configuration for Neighbouring Cells for logged MDT.

-	Sensor Information for logged MDT and immediate MDT.

-	Excess packet delay thresholds (present only if M6 measurements are requested).

-	Beam Level Measurement (present only if M1 measurement in NR is requested).

-	Logging Periodicity (present only if logged L1 measurements for immediated MDT in NR are requested).

-	Event Trigger Config for logged L1 measurements (present only if event-triggered logged L1 measurements for immediated MDT in NR are requested).

Note that at the same time not all the parameters can be present. The criteria for which parameters are present are described in clause 5 of the present document.

2)	When gNB receives the Trace Session activation request from its management system, it shall start a Trace Session and should save the parameters associated to the Trace Session.

3)	gNB shall select the suitable UEs for MDT data collection. The selection is based on the area received from the management system and the area where UE is located, user consent information received from the core network as part of the Management based MDT PLMN List IE (As described in section in 4.9.2 of this document). If the user is not in the specified area or if the Management based MDT PLMN List IE is not present in the UE context the UE shall not be selected by the gNB for MDT data collection. During UE selection, the gNB shall take into account also the UE capability (MDT capability) when it selects UE for logged MDT configuration. If the UE does not support logged MDT, the UE shall not be selected. Furthermore, during UE selection, the gNB shall take into account also the UE capability (see “AIML-Parameters” in TS 38.331 [43]), when it selects for logged L1 measurements for immediated MDT. If the UE does not support logging of UE radio measurements, the UE shall not be selected.
If M4 or M5 measurements are requested in the MDT configuration, gNB should start the measurement according to the received configuration. Details of the measurements are defined in TS 38.314 [50].

4)	gNB shall activate the MDT functionality to the selected UEs. When gNB selects a UE, it shall take into account the availability of Management based MDT PLMN List IE in the user context and the area scope parameter received in MDT configuration (Trace Session activation). Detailed description about user consent handling and how it is provided to the gNB is described in section 4.9.2. If there is no Management based MDT PLMN List IE in the user context or the user is outside the area scope defined in the MDT configuration, the UE shall not be selected for MDT data collection. The gNB shall assign Trace Recording Session Reference corresponding to the selected UE. The gNB shall send at least the following configuration information to the UE in case of Logged MDT:

-	Trace Reference

-	Trace Recording Session Reference

-	TCE Id

-	Logging Interval

-	Logging Duration

-	Absolute time reference

-	Area Scope where the UE measurements should be collected: list of NG-RAN cells/TA, additionally a list of NPN IDs in NR.

-	MDT PLMN List

-	Event List for Event Triggered Measurement

-	Event Threshold, Hysteresis and Time to Trigger (present only if L1 event is configured)

NOTE:	For UEs currently being in idle or inactive mode and camping in the cell the logged MDT configuration cannot be sent. These UEs may be configured when they initiate some activity (e.g., Service Request or  Mobility Registration) at next time.

In case of Immediate MDT, the following parameters shall be sent to the UE:

-	List of Measurements

-	Reporting Trigger

-	Report Interval

-	Report Amount

-	Event Threshold

-	Excess packet delay thresholds (present only if M6 UL measurements are requested)

-	Beam Level Measurement (present only if M1 measurement in NR is requested)

-	Logging Periodicity (present only if logged L1 measurements for immediated MDT in NR is requested).

-	Event Trigger Config for logged L1 measurements (present only if event-triggered logged L1 measurements for immediated MDT in NR are requested).

Note that at the same time not all the parameters can be present. Conditions of the parameters are described in clause 5 of the present document.

gNB performs necessary actions (e.g. activates GNSS module of the UE, enables and collects certain positioning measurements) specified in TS 38.305 [52] according to the value of Positioning Method (see clause 5.10.19) received in the Trace configuration. gNB captures location information and/or positioning measurements in the MDT trace record.

If Reporting Trigger parameter indicates that all configured RRM measurement trigger should be reported in MDT, then gNB should ask the UE to provide the "best effort" location information together with the measurement reporting by setting the includeCommonLocationInfo IE in all RRC measurement reporting configurations.

5)	When UE receives the MDT activation it shall start the MDT functionality based on the received configuration parameters.

6)	The gNB shall not retrieve MDT report from the UE if UE’s rPLMN does not match the PLMN where TCE used to collect MDT data resides (e.g. gNB’s primary PLMN). When the gNB receives the MDT report from UE, the gNB shall get the Trace Recording Session Reference, Trace Reference, and TCE Id  from the report, and compare the Trace PLMN (PLMN portion of Trace Reference) with the PLMN where TCE used to collect MDT data resides (e.g. its primary PLMN) and discard MDT report in case of a mismatch. Otherwise if the MDT anonymization requires the IMEI-TAC in the MDT record gNB shall send the Trace Recording Session Reference, Trace Reference, serving cell CGI, and TCE IP Address (or Trace Reporting Consumer URI) in the CELL TRAFFIC TRACE message to the AMF via the NG connection. When AMF receives this NG signalling message containing the Trace Recording Session Reference, Trace Reference, serving cell CGI, and the Privacy Indicator (that shall be set to Logged MDT or Immediate MDT depending on the configured Job Type) if so indicated in the privacy indicator, the AMF shall look up the subscriber identities (IMEI(SV)) of the given call from its database, and send the IMEI-TAC together with the Trace Recording Session Reference and Trace Reference and for immediate MDT also the serving cell CGI to the TCE, as described in section 4.7 of the present document. For logged MDT, AMF will send the IMEI-TAC together with the Trace Recording Session Reference, Trace Reference to the TCE.

NOTE:	For management based Immediate MDT, TRSR may be duplicated among different gNBs when multiple cells are selected as the area scope for the same MDT job. In this case, the combination of TRSR and the UE’s serving cell CGI in the MDT report can uniquely identify one trace recording session.

7)	For Immediate MDT when the gNB receives the MDT report from the UE in the RRC message the gNB shall capture it and put the UE’s serving cell CGI together with the MDT report from the UE to the trace record. A UE configured to perform Logged MDT measurements in IDLE or INACTIVE indicates the availability of MDT measurements, by means of a one-bit indicator, in RRC messages definitions in 3GPP TS 38.331 [43], e.g., RRCSetupComplete message and RRCReconfigurationComplete message. The gNB can decide to retrieve the logged measurements based on this indication by sending the UEInformationRequest message to the UE. The UE can answer with the collected MDT logs in UEInformationResponse message.

8)	The gNB shall forward the Trace Records to the Trace Collection Entity (TCE). In case of logged MDT, the TCE Id is indicated in the MDT report is translated to the actual IP address of the TCE or the URI of the Trace Reporting Consumer by the gNB before it forwards the measurement records. (The TCE identity translation is using configured mapping in the gNB.) In case of immediate MDT, the IP address of the TCE or the URI of the Trace Reporting Consumer is indicated for the gNB in the trace configuration.

The Immediate MDT measurement configuration is deleted in the UE together with the RRC context when entering idle or inactive mode.

The Logged MDT trace session is preserved in the UE until the duration time of the trace session expires, including also multiple idle or inactive periods interrupted by various state transitions such as idle-connected-idle state transitions.

The Logged MDT trace session context of the UE is stored in the network as long as the trace session is active, including also the periods when the UE is in connected state.

Management system shall validate that the MCC and MNC specified in the Trace Reference is the same as the PLMN supported by all the cells specified in the area scope. If the gNB receives a request with a PLMN in the TraceReference that does not match any PLMN in its list, it shall ignore the request.

Next change

### 5.10.3	List of Measurements

This parameter is mandatory if the Job Type parameter indicates Immediate MDT. This parameter defines the measurements that shall be collected. For further details see also TS 37.320 [30]. The parameter is 4 octet long bitmap.

The parameter can have the following values in UMTS:

-	M1: CPICH RSCP and CPICH Ec/No measurement by UE.

-	M2: For 1.28 Mcps TDD, P-CCPCH RSCP and Timeslot ISCP measurement by UE.

-	M3: SIR and SIR error (FDD) by NodeB

-	M4: UE power headroom (UPH) by the UE, applicable for E-DCH transport channels.

-	M5: Received total wideband power (RTWP) by Node B

-	M6: Data Volume measurement, separately for DL and UL, by RNC.

-	M7: Throughput measurement, separately for DL and UL, per RAB and per UE, by RNC.

-	Any combination of the above

Detailed information for M3, M5, is defined TS 25.215 [54], for M1, M2 in TS 25.331[31] and for M4 in TS 25.321[55].

The parameter can have the following values in LTE:

-	M1: RSRP, RSRQ and SINR measurement by UE

-	M2: Power Headroom (PH) measurement by UE
NOTE: Available from MAC layer

-	M3: Received Interference Power measurement by eNB

-	M4: Data Volume measurement separately for DL and UL, per QCI per UE,  by eNB

-	M5: Scheduled IP Throughput measurement separately for DL and UL, per RAB per UE and per UE for the DL, per UE for the UL, by eNB

-	M6: Packet Delay measurement, separately for DL and UL, per QCI per UE, see UL PDCP Delay, by the UE, and Packet Delay in the DL per QCI, by the eNB

-	M7: Packet Loss rate measurement, separately for DL and UL per QCI per UE, by the eNB

-	M8: RSSI measurement by UE for WLAN and Bluetooth®

-	M9: RTT measurement by UE only for WLAN

-	And any combination of above.

Detailed information for M4, M5, M6, M7 is defined TS 36.314 [56], for M1, M3, M8, M9 in TS 36.331[31], for M2 in TS 36.321[57].

The parameter can have the following values in NR:

-	M1: DL signal quantities measurement results for the serving cell and for intra-frequency/Inter-frequency/inter-RAT neighbour cells, including cell/beam level measurement by UE.

-	M2: Power headroom (PH) measurement by UE

-	M3 is not supported by this release

-	M4: PDCP SDU Data volume measurement separately for DL and UL, per DRB per UE by gNB

-	M5: Average UE throughput measurement separately for DL and UL, per DRB per UE and per UE for the DL, per DRB per UE and per UE for the UL, by gNB

-	M6: Packet delay measurement, separately for DL and UL, per DRB per UE by gNB

-	M7: Packet loss rate measurement, separately for DL and UL, per DRB per UE by gNB

-	M8: RSSI measurement by UE for WLAN and Bluetooth®

-	M9: RTT measurement by UE for WLAN

-	LoggedL1Meas: Logged L1 radio measurement results including the cell identity and the beam identifiers associated to CSI-RS resources or SSBs (CSI-RS IDs or SSB IDs) by UE; including the indicator of presence of a gap that is longer than the configured logging periodicity. Note that Logged L1 radio measurements can only be activated in management based Immediate MDT in NG-RAN. Furthermore, note that Logged L1 radio measurements are handled in the context of "Network-side data collection" in RAN specs (e.g. TS 37.320 clause 5.4.1.5) which means data which is collected for network side AI/ML models.

-	And any combination of above

Detailed information for M4, M5, M6, M7 is defined in TS 28.558 [62] and TS 38.314 [50], for M1, M8, M9 in TS 38.331[43], for M2 in TS 38.321[51].

| UMTS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |
| M7 for DL | M6 for UL | M6 for DL | M5 | M4 | M3 | M2 | M1 |
| spare |  |  |  |  |  |  | M7 for UL |
| spare |  |  |  |  |  |  |  |
| spare |  |  |  |  |  |  |  |

| LTE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |
| M7 for DL | M6 for DL | logging of M1 from event triggered measurement reports according to existing RRM configuration | M5 | M4 | M3 | M2 | M1 |
| spare |  |  |  | M7 for UL | M6 for UL | M9 | M8 |
| spare |  |  |  |  |  |  |  |
| spare |  |  |  |  |  |  |  |

| NR |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |
| M8 | logging of M1 from event triggered measurement reports according to existing RRM configuration. | M7 for DL | M6 for DL | M5 for DL | M4 for DL | M2 for DL | M1 for DL |
| spare |  | LoggedL1Meas | M7 for UL | M6 for UL | M5 for UL | M4 for UL | M9 |
| spare |  |  |  |  |  |  |  |
| spare |  |  |  |  |  |  |  |

NOTE: Value "1" indicates "activate" and value "0" indicates "do not activate".

Next change

### 5.10.43	TRSR Prefix Configuration

This parameter is mandatory if C-MDT is supported in NR-RAN. This parameter contains TRSR prefix configuration parameters which provides a concise way to represent a block of TRSRs, used by NR-RAN at TRSR assignment for a given C-MDT job. It defines both the base TRSR prefix and the size of the TRSR prefix. The base TRSR prefix is a 2 byte Octet String. The TRSR prefix length is an integer, representing the number of bits is used for the TRSR prefix.

### 5.10.X	Logging Periodicity

This NR parameter is optional if the parameter Job Type indicates Immediate MDT and the bit 14 (LoggedL1Meas) of the parameter List of Measurements (defined in Section 5.10.3) for NR is set to 1.

It defines the periodicity that the UE shall use for the logging of the CSI measurements. The Logging Periodicity is given as a multiple of the periodicity of the resources in which the UE performs channel measurement whose associated measurement results are logged by the UE.

If Logging Periodicity is not included, the UE performs the logging of CSI measurements according to the periodicity of the resources, i.e. for every occasion of the resources.

The parameter is an enumerated type with the following values (detailed definition is in 3GPP TS 38.331[43]):

- 	2 (0),

-	3 (1),

-	4 (2),

-	5 (3).

### 5.10.Y	Event Trigger Config for Logged L1 Measurements

This NR parameter is optional if the parameter Job Type indicates Immediate MDT and the bit 14 (NW-side DC) of the parameter List of Measurements (defined in Section 5.10.3) for NR is set to 1.

It configures the UE with either A1 event or A2 event triggered logged L1 measurements.

This configuration consists of the following five elements:

- Event Type: The parameter defines the type of event to configure. The parameter is an enumerate type with the following values:

-	eventA1 (0),

-	eventA2 (1).

- Measurement Quantity: The parameter defines the measurement quantity. It is either RSRP, RSRQ or SINR.

- Threshold: The parameter defines the threshold for logged L1 measurements. The parameter is an Integer number between 0 .. 127. For calculating the actual value see RSRP-Range, RSRQ-Range, SINR-Range in TS 38.331 [43]).

- Hysteresis: The parameter is used within the entry and leave condition of the event triggered logged L1 measurements. The parameter is an Integer number between 0 .. 30. For calculating the actual value see Hysteresis in TS 38.331 [43]).

- Time to Trigger: The parameter defines the time during which specific criteria for the event needs to be met in order to trigger logged L1 measurements.
The parameter is an is an enumerated type with the following values:

-	0 ms (0),

-	40 ms (1),

-	64 ms (2),

-	80 ms (3),

-	100 ms (4),

-	128 ms (5),

-	160 ms (6),

-	256 ms (7),

-	320 ms (8),

-	480 ms (9),

-	512 ms (10),

-	640 ms (11),

-	1024 ms (12),

-	1280 ms (13),

-	2560 ms (14),

-	5120 ms (15).

For detailed definitions of the parameters see TS 38.331 [43]:

End of changes