3GPP TSG-SA5 Meeting #164	S5- 255159

Dallas, USA, 17 - 21 November 2025

Source:	NTT Docomo

Title:	Pseudo-CR on TR28.883 Use case about NDT for NTN

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	0.1.0

Work Item:	FS_NDT_Ph2

Comments

Use case about NDT in NTN in Release 20

Proposed Changes

* * * First Change * * * *

## 5.X  Use Case #8: NDT for Non-Terrestrial Network (NTN) Performance and Optimization Evaluation

### 5.X.1  Description

Non-Terrestrial Networks (NTN) include satellite, HAPS (High Altitude Platform Stations), and UAV-based communication systems and are expected to be a key component in providing 6G connectivity. Compared with terrestrial networks, NTNs exhibit unique characteristics such as high mobility of nodes, dynamic topologies, long propagation delays, and variable link conditions. These aspects make planning, optimization, and management challenging for mobile network operators.

This use case proposes to use Network Digital Twin (NDT) technology to simulate, analyze, and optimize the end-to-end behavior of NTN segments and their interaction with terrestrial networks. For example, the NDT can create a near real-time virtual representation of the satellite constellation, gateway stations, and user terminals to predict network conditions and support decision-making for service continuity and QoS/QoE management.

The NDT framework for NTN can be used to:

- Predict and mitigate handover failures during satellite transitions for NTN entities' constellations.

- Optimize beam management and power allocation based on user distribution and traffic demand.

- Evaluate end-to-end latency and throughput under different orbit configurations or payload loads.

* * * Next Change * * * *

### 5.X.2  Potential requirements

REQ-NDTNTN-01: The 3GPP management system should support a capability enabling an authorized MnS consumer to instantiate and manage NTN-specific NDT instances, including satellite and HAPS network elements as simulation objects.

REQ-NDTNTN-02: The 3GPP management system should support synchronization of orbital and link state information between NTN and terrestrial NDTs to enable end-to-end performance simulation.

* * * End of Changes * * * *