3GPP TSG-SA5 Meeting #164	S5-255160

Dallas, USA 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.567 | CR | 0007 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | R19 CR TS 28.567 clarification on CCL scopes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CCL scopes are described in many places of the current specification. The contents are not aligned. For example, in clause 4.3.4, CCL scopes contain measurement scope, targeted scope, desired impact scope, control scope, Monitored scope and impact scope; in clause 5.1.2, CCL scopes contain a desired outcomes scope, measurement scope, a control scope and an impact scope; scopeType as defined in clause 6.4.1 has CCL_MEASUREMENT_SCOPE, CCL__TARGETED__SCOPE, CCL_CONTROL_SCOPE, CCL_IMPACT_SCOPE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is suggested to align the description of CCL scope based on the scopeType as defined in clause 6.4.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May cause implementation issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4 and 5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 4.3.4	CCL scopes

The scope is the set of managed objects, their properties and network outcomes that are associated with the CCL for measurement, configuration and impact. The scopes for the different CCLs can be managed by the MnS consumer, i.e. they can be defined on to the CCL or revised by the MnS consumer. A CCL may have four scopes: the measurement scope, target  scope, control scope and impact scope, defined as follows:

-	measurement scope: the measurement scope is where related measurements are collected

-	control scope: control scope is the scope to which the CCL's actions are desired to be applied, e.g., the set of network functions and attributes that are the planned candidates to be modified by the CCL. The control scope is also called the action-space as it describes the set of candidate actions that the CCL can (is configured to be able to) execute.

-	target scope: which relates to purpose of the CCL

-

-	impact scope: impact scope is the scope to which the CCL's actions have influence, e.g., it is both the network functions and attributes as well network outcomes like coverage areas that are influenced by the configuration actions of the CCL. This is different from the measurement scope, i.e. the scope where the CCLs measure and control scope, i.e. the scope where they act.

The impact scope may be known and bounded or unbounded and thus unknown - see figure 4.3.4-1. The bounded scope indicates that the area known by the CCL is the scope where its actions will impact. The unbounded impact-scope is the full network scope where the CCL’s action will have impact, but the CCL does have information that its action will have that impact to that scope.

Figure 4.3.4-1: Exemplification of known/bounded vs. unknown/unbounded impact scope: CCL A takes action in cell A expecting impact in cells A, B, C and D. if the impact is strictly in cells A, B, C and D, then the impact scope is known and bounded. However, if the impact scope includes cells E and F, then for the CCL, the true impact scope is unknown and thus unbounded.

2nd Change

### 5.1.2	Use cases

#### 5.1.2.1	General CCL Control – DynCCL_01

A CCL contains a set of logic functionalities or steps, each providing a specific functionality and where the steps work together to achieve the stated desired outcomes over a given network scope. The MnS consumer should be able to configure and receive information about the desired outcomes of the CCL.

Generally, the four CCL steps of Monitoring, Analysis, Decision and Execution are expected with the expectation that each step is accomplished by a single management function or service. However, one management function or service may also accomplish the functionality of more than 1 step. The MnS consumer should be able to receive information about the management functions or services that form the CCL.

A CCL may have four scopes including a measurement scope, a control scope, a target scope and an impact scope. The scopes for the different CCLs can be managed by the MnS consumer. The MnS consumer should also be able to receive reports about these different aspects of the CCL, e.g., about the status of the CCLs execution as well as to configure the reporting.

End of Change