3GPP TSG-SA5 Meeting #164	S5-255168

Dallas, USA, 17 - 21 November 2025

Source:	China Unicom

Title:	Pseudo-CR on Rel-20 TR 28.885 Add new solution for EC and EE per PLMNID in Network sharing scenario

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	V0.1.0

Work Item:	FS_Energy_Ph4_OAM

Comments

This contribution proposes to add a new  solution for  energy consumption and energy efficiency per PLMNID in network sharing scenario.

Proposed Changes

* * * First Change * * * *

5.4.3	Use case #3: Support estimation of EC and EE KPIs at per PLMN-ID granularity in Network sharing scenario

5.4.3.1	Description

When a NG-RAN is shared between multiple operators in the network sharing (see TS 32.130 [11]), the participating operators are interested in the operator-specific energy consumption (EC) and energy efficiency (EE) of the NG-RAN.

The NG-RAN can support and serve multiple operators (i.e., PLMN-IDs) in Network sharing scenarios. The existing EE and EC KPIs do not consider network sharing scenarios, and do not provide KPIs at operator-specific (i.e., PLMN-ID) granularity.

Following are the currently supported measurements and KPIs related to energy consumption and energy efficiency:

- Power, Energy and Environmental (PEE) measurements are defined in clause 5.1.1.19 of TS 28.552 [10], and PNF Energy consumption for ManagedElement is defined in clause 5.1.1.19.3 of TS 28.552 [10].

- Energy consumption and energy efficiency related KPIs are defined in clause 6.7 of TS 28.554 [4].

This use case is to study the enhancements to enable monitoring and/or estimation of energy consumption and energy efficiency KPIs and reporting of a NG-RAN at operator-specific (i.e., at a per PLMN-ID) granularity in network sharing scenarios.

5.4.3.2	Potential requirements

PREQ-Energy_Consumption_Network_Sharing-1: The 3GPP management system should be able to report the energy consumption and energy efficiency of a NG-RAN at operator-specific, i.e., at a per PLMN-ID granularity.

5.4.3.3	Potential solutions

5.4.3.3.i	Potential solution #<i>

5.4.3.3.i.1	Introduction

This solution proposes to calculate the PLMN level EC and EE in network sharing scenario based on the existing measurements defined in TS 28.552[10]. The detailed mechanism is described in the following part.

5.4.3.3.i.2	Description

This solution proposes to define  two new KPIs based on the existing measurements to represent the EC and EE of a shared gNB  in PLMN granularity.

- PLMN-level Energy Consumption KPI

The new EC KPI describes the PLMN-level Energy Consumption (EC) of the gNB when the gNB is  shared among several operators. It is obtained by summing up the per PLMN energy consumption of all network functions (NFs) that constitute the shared gNB. The unit of this KPI is kWh.

Below is the equation for gNB energy consumption per PLMN

ECgNBperPLMN=PNFii∈Group1PEE.EnergyPNFiN+DRB.PdcpSduVolumeDL_PLMN+DRB.PdcpSduVolumeUL_PLMNDRB.PdcpSduVolumeDL+DRB.PdcpSduVolumeUL×PNFjj∈Group2PEE.EnergyPNFj

Where  PNFi and PNFj are all PNFs that constitute the shared gNB. PEE.EnergyPNFi and PEE.EnergyPNFj are energy consumption of these NFs, which can be measured as defined in clause 5.1.1.19.3 in TS 28.552. DRB.PdcpSduVolumeDL_PLMN and DRB.PdcpSduVolumeUL_PLMN and DRB.PdcpSduVolumeDLand DRB.PdcpSduVolumeUL are defined in clause 5.1.2.1 in TS 28.552. N is the number of PLMN sharing the same gNB.

Energy consumption of PNFs in group1 may be independent from data volume while energy consumption of PNFs in group2 can be affected by data volume. So the per PLMN energy consumption of PNFs in group1 is obtained by dividing the sum of energy consumption of PNFs in gourp1 with the number of PLMN sharing the same gNB. The per PLMN energy consumption of PNFs in group 2 is obtained by multiplied the sum of group2-PNF energy consumption with the ratio of PDCP data volume of the corresponding PLMN out of all PDCP data volume.

- PLMN-level Energy Efficiency KPI

The new EC KPI is defined to reflect per PLMN energy efficiency with data volume as performance indicator. It can help each participating operators be aware of its individual energy efficiency in network sharing scenario. The unit of this KPI is kbit/kWh.

Below is the equation for gNB energy efficiency per PLMN on data volume

EEgNBperPLMN=DataVolumeperPLMNECgNBperPLMN

The performance indicator DataVolumeperPLMN is the sum of the data volume delivered over air interface both in uplink and down link per PLMN. DataVolumeperPLMN is the sum of DRB.PdcpSduVolumeDL_PLMN and DRB.PdcpSduVolumeUL_PLMN, defined in clause 5.1.2.1 in TS 28.552. ECgNBperplmn is the energy consumption per PLMN as defined in the above paragraph.

5.4.3.4	Evaluation of potential solutions

* * * End of Changes * * * *