3GPP TSG-SA5 Meeting #164	S5-255172

Dallas, USA, 17 - 21 November 2025

# Source:	Huawei

# Title:	Rel-20 pCR 32.872 Introduce the solution#2 on LBO Inter CHFs scenario Key issue #1

# Document for:	Approval

Agenda Item:	7.5.2

1	Decision/action requested

This is a pCR to TR 32.872 Introduce a solution on Local Breakout Inter CHFs scenario key issue1.

2	References

[1]	3GPP TR 32.872: "Study on 5GA roaming charging reliability enhancement".

3	Rationale

This pCR proposes to introduce a new solution on Local Breakout Inter CHFs scenario key issue1.

4	Detailed proposal

Propose to incorporate the following change into the TR 32.872 [1]

First change

5.1.4.a	Solution #1.y: CTF detected failure

A possible solution for key issue #1 covering requirements REQ-3GPPCH-LBIC-1, enhanced failure handling for scenarios where a failure is detected between the CTF and the V-CHF.

When the CHF is determined not reachable by NF consumer (CTF), NF consumer (CTF) uses application level failure handling (Terminate, Continue, Retry_and_terminate) as specified in clause 5.5.1.1 of 3GPP TR 32.290 [3].

When failure handling is set to 'Continue', the NF consumer (CTF) re-sends the request with the ‘Remote CHF resource’ to an alternative V-CHF, then the alternative V-CHF replaces the original V-CHF to interact with the H-CHF.

End of change