3GPP TSG-SA5 Meeting #164	S5-255173

Dallas, USA, 17 - 21 November 2025

# Source:	Huawei

# Title:	Rel-20 pCR 32.872 Introduce the solution#1 on LBO Inter CHFs scenario Key issue #2

# Document for:	Approval

Agenda Item:	7.5.2

1	Decision/action requested

This is a pCR to TR 32.872 Introduce a solution on Local Breakout Inter CHFs scenario key issue2.

2	References

[1]	3GPP TR 32.872: "Study on 5GA roaming charging reliability enhancement".

3	Rationale

This pCR proposes to introduce a new solution on Local Breakout Inter CHFs scenario key issue2.

4	Detailed proposal

Propose to incorporate the following change into the TR 32.872 [1]

First change

5.1.4.b	Solution #2.x: V-CHF detected failure of charging session with H-CHF

A possible solution for key issue #2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When the V-CHF, acting as an NF consumer, detects a failure of the H-CHF, it determines how to handle the charging session with the NF (CTF) based on the operator's agreement, e.g. consumer CHF may continue the session with NF (CTF) and store the charging information for settlement, as specified in clause 5.5.1.3 of 3GPP TS 32.290[3].

In the case of V-CHF decides to release the charging session between the V-CHF and CTF, the V-CHF sends a Charging Data Response or Charging Notify Request to CTF to indicate that the charging session should be released.

In the case of V-CHF decides to continue the charging session between the V-CHF and CTF:

-	without quota management: the V-CHF stores Charging Data Request(s) or charging information from the NF consumer (CTF).

-	with quota management: the V-CHF instructs the NF consumer (CTF) to suspend quota management as specified in clause 5.3.2.5 of 3GPP TS 32.290[3], and stores the Charging Data Request(s) or charging information marked with ‘QUOTA_MANAGEMENT_SUSPENDED’ from the NF consumer (CTF).

End of change