3GPP TSG-SA5 Meeting #164	S5-255180

Dallas, USA, 17 - 21 November 2025

# Source:	Huawei

# Title:	Rel-20 pCR 32.872 Introduce the solution#2 on HR scenario Key issue #1

# Document for:	Approval

Agenda Item:	7.5.2

1	Decision/action requested

This is a pCR to TR 32.872 Introduce a solution on Home Routed scenario key issue1.

2	References

[1]	3GPP TR 32.872: "Study on 5GA roaming charging reliability enhancement".

3	Rationale

This pCR proposes to introduce a new solution on Home Routed scenario key issue1.

4	Detailed proposal

Propose to incorporate the following change into the TR 32.872 [1]

First change

5.2.4.b	Solution #1.y: Failures detected between the V-CHF and the V-CTF

A possible solution for key issue #1 covering requirements REQ-3GPPCH-LBHR-1, failure is detected in one of the charging sessions.

When the V-CHF detects that an expected charging data request for a particular session has not been received within a period of time, the V-CHF handles as specified in clause 5.5.1.2 of 3GPP TS 32.290[3].

When the V-CHF is determined not reachable by V-SMF (CTF), the V-SMF (CTF) uses application level failure handling (Terminate, Continue, Retry_and_terminate) and may store Charging Data Request(s) or charging information as specified in clause 5.5.1.1 of 3GPP TS 32.290[3]. if the failure handling indicates 'Terminate', the service will be terminated, and the associated charging sessions will be terminated accordingly.

End of change