3GPP TSG-SA5 Meeting #164	S5-255211

Dallas, USA, 17 - 21 November 2025

Source:	Huawei

Title:	pCR TR 28.884 Update use case on software management

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	V0.1.0

Work Item:	FS_SBMA_Ph4

Comments

This contribution proposes requirements related to software management in 5G which are aligned with the ones specified in TS 32.531 in LTE:

- TS 32.531 clause 4.2.3 defines the requirements on software download. See REQ_NASWM_FUN_1 and REQ_NASWM_FUN_2.

- TS 32.531 clause 4.2.3 defines the requirements on software activation. See REQ_NASWM_FUN_5 and REQ_NASWM_FUN_8.

- TS 32.531 clause 4.2.1 and 4.2.3 defines the requirements on software fallback/rollback. See REQ_SWM_FUN_1, and REQ_SWM_FUN_6.

- TS 32.531 clause 4.2.1 defines the requirements on software information retrieval. See REQ_SWM_FUN_2.

- TS 32.531 clause 4.2.1 defines the requirements on software monitoring on software download, installation, activation and fallback/rollback. See REQ_SWM_FUN_3.

It is proposed to add software management capability mentioned above in 5G.

Proposed Changes

* * * First Change * * * *

## 5.4	Use case#4 on software management for 5G

### 5.4.1	Description

3GPP TS 32.531 [11], TS 32.532 [12], and TS 32.533 [13] define the concepts, requirements, Information Service, and CORBA solution set for the software management of NEs for 4G, encompassing both automated and non-automated software management approaches.

Software management for 5G enhances 5G network operational efficiency. The benefits of software management retain for 5G network management:

-	One benefit of software management is interoperability: standardized procedures make it possible for operators to manage software on heterogeneous network elements in a multi-vendor environment and reduces operational complexity.

-	Another benefit is network service continuity. By supporting mechanisms such as staged upgrades, version control, and fallback strategies, software management capability minimizes downtime and protects user experience.

Currently, TS 28.533 [2] does not include support for software management. As 5G networks evolve in scale and complexity, it is essential to introduce software management capabilities into SBMA. These capabilities will enable operators to maintain software of NEs and NFs.

Editor's note:	To enable software management within SBMA, the associated potential requirements and potential solutions are FFS, building upon the legacy of software management of NEs for LTE while adapting to the principles of SBMA.

### 5.4.2	Potential requirements

#### 5.4.2.1	Potential general requirements

REQ-SBMA-SWM-1: The 3GPP management system should support software management.

#### 5.4.2.2	Potential requirements for PNF

REQ-SBMA-PNFSWM-1: The 3GPP management system should support the capability to monitor software download, installation and activation.

REQ-SBMA-PNFSWM-x: The 3GPP management system should support software download.

REQ-SBMA-PNFSWM-y: The 3GPP management system should support software activation.

REQ-SBMA-PNFSWM-z: The 3GPP management system should support software rollback.

REQ-SBMA-PNFSWM-w: The 3GPP management system should support the capability to retrieve the information of a software.

#### 5.4.2.3	Potential requirements for VNF

* * * End of Changes * * * *